Contents

a. Calculate the resistance of E and H for the circuit’s configuration. 	[3]	1

b. B and C are identical and the current passing through A is 0.5A, calculate the current passing through B and C. [2]	1

c (i) The Resistance of A. [2]	1

c (ii) The Resistance of B and C [3]	1

c (iii) The potential difference across A, B and C [1]	1

c (iv) Since the total potential difference across L to M is 6V, calculate the potential difference across D and find its Resistance. [2]	1

d. The ‘lost volts across one of the cells in the parallel arrangement is 2 V. Calculate r and hence the total resistance of the parallel combination of cells. [3]	1

e. Given that the resistance F is 6Ω, calculate the voltage drop across it. [2]	1

f. Calculate the total resistance of the circuit, this includes total internal and the resistance cause by the reverse polarity battery and the open diodes in given scenario (ignore any “non-open diodes”). [2]	1

g. Calculate the emf of each of the cells in the parallel combination. [3]	1

## a. Calculate the resistance of E and H for the circuit’s configuration. 	[3]

RE = VEIE

RE = 0.6V0.5A

RE = 1.2Ω

RH = VHIH

RH = 0.7V0.5A

RH = 1.4Ω

## b. B and C are identical and the current passing through A is 0.5A, calculate the current passing through B and C. [2]

Current = 1A

1-0.5 = 0.5A

0.52 = 0.25A = Resistor B = Resistor C

## c (i) The Resistance of A. [2]

R = PI2

Ra= 1.250.52=5Ω

## c (ii) The Resistance of B and C [3]

R = PI2

Rb= 1.250.252=20Ω

Rc= 1.250.252=20Ω

## c (iii) The potential difference across A, B and C [1]

V = I . R

A = 0.5 x 5 = 2.5V

B = 0.25 x 20 = 5V

C = 0.25 x 20 = 5V

## c (iv) Since the total potential difference across L to M is 6V, calculate the potential difference across D and find its Resistance. [2]

RD = VDID

RD = 6V1A

RD = 6Ω

## d. The ‘lost volts across one of the cells in the parallel arrangement is 2 V. Calculate r and hence the total resistance of the parallel combination of cells. [3]

Ir = 1A4 = 0.25A

Rr = VrIr

Rr= 2V0.25=8Ω

Rtotal = 11R1+1R2+1R3+1R4

Rtotal = 118+18+18+18

Rtotal=2Ω

## e. Given that the resistance F is 6Ω, calculate the voltage drop across it. [2]

V = I . R

V = 1A x 6Ω

V = 6V

## f. Calculate the total resistance of the circuit, this includes total internal and the resistance cause by the reverse polarity battery and the open diodes in given scenario (ignore any “non-open diodes”). [2]

Rabctotal = 11Ra+1Rb+1Rc

Rabctotal = 1120+120+15

Rabctotal = 3.33

Rcircuit = Rabc+RD+RF+Rr

Rcircuit = 3.33+6+6+2

Rcircuit=17.33Ω

## g. Calculate the emf of each of the cells in the parallel combination. [3]

emf = V + Ir

emf = 2 + 8