| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 4 | LEFEL 4 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| may 2020 | mai 2020 |

MODULE CODE: ACDB4002 & ACDA4006

Electrical and Electronic Principles

TIME:  2 HOURS + 30 mins upload time

answer all questions

Equipment or materials required – Calculator, Graph paper

Adnoddau neu ddefnyddiau sydd eu hangen – cyfrifiannell, papur graff

Formula sheet and graph paper are after questions

Detach graph paper and place in your Answer book write your student number and question number on any graph paper used

you will have 30 mins following the exam to photograph and upload your answers. your answers can be handwritten and do not have to be typed.

ANSWER ALL QUESTIONS

- An oscilloscope display shows the above voltage waveform.  The oscilloscope settings are shown in the box next to the waveform.

Oscilloscope settings:

5V/div

25ms/div

- Estimate the period of the waveform

[1 Mark]

4 div x 25 ms/div = 100 ms

- From the estimated period, determine its frequency

[1 Mark]

f = 1/t = 1/0.1 = 10 Hz

- Determine its peak to peak amplitude

[1 Mark]

Vpk-pk = 4 div x 5 V = 20 V

- Determine the RMS value of the waveform

[2 Marks]

VRMS = Vpk x 0.7071 = 20/2 x 0.7071 = 7.071 V

- The discharging equation for a series resistor/capacitor network is given by:

VC=V0e-tτ

Where Vc is the voltage across the capacitor and V0 is the initial capacitor voltage.

Assuming the following: R = 80k, C = 10µF and V0 = 10V:

- Calculate Vc at time t = 1.6 seconds.

[3 Marks]

τ=RC

τ=80×103×10×10-6=0.8 Seconds

VC=V0e-tτ

VC=10e-1.60.8=1.353 V

- Sketch a labelled graph that would represent the discharging curve.

[5 Marks]

| t (seconds) | Vc Volts |
| --- | --- |
| 0 | 10 |
| 0.8 | 3.679 |
| 1.6 | 1.353 |
| 2.4 | 0.498 |
| 3.2 | 0.183 |
| 4.0 | 0.067 |

- What is the stored energy within the capacitor before it is discharged?

[3 Marks]

E=12CV2

E=12×10×10-6×102=500×10-6J

- Calculate the time taken for the stored energy within the capacitor to reach 0.1mJ

[4 Marks]

E=12CV2

V=E12C=0.1×10-31210×10-6=4.4721 V

VC=V0e-tτ

-lnVCVO×τ=t

-ln4.472110×0.8=0.64378 Seconds

- Design a logic circuit to fulfil the behaviour shown in the truth table in Fig Q3.

[7 Marks]

| A | B | C | X |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 1 |
| 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 1 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 1 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 |

Fig Q3

- For the circuit shown in Fig Q4. Determine the following:

  - Total circuit resistance

[4 Marks]

1RT=1R1+1R2+…+1Rn

RT=114000+2000+12000+1000=2 kΩ

  - Total circuit current

[2 Marks]

I=VR=122000=6 mA

  - Current flowing through R4

[2 Marks]

I=6×10-3×4000+20004000+2000+2000+1000=4 mA

  - Total power delivered by the battery

[2 Marks]

P=IV=6×10-3×12=72 mW

  - Power dissipated by R1

[4 Marks]

IR1=6 mA-4 mA=2 mA

P=I2R=2×10-32×4×10-3=16 mW

  - R4 is replaced by a wire link. What is the new circuit current?

[4 Marks]

1RT=1R1+1R2+…+1Rn

RT=114000+2000+12000=1500 Ω

I=VR=121500=8 mA

Fig. Q4

- For the circuit shown in Fig Q5, calculate the current flowing through the all resistors given V1=5V, V2=10V, R1=10Ω, R2=15Ω, R3=5Ω.

[5 Marks]

Fig. Q5

By nodal analysis:

I1+I2=I3

I1=5-VX10

I2=10-VX15

I3=VX5

VX5=5-VX10+10-VX15

Multiply by 15

3VX=1.55-VX+10-VX

3VX=7.5-1.5VX+10-VX

5.5VX=17.5

VX=17.55.5=3.1818 V

I1=5-3.181810=0.1818 A

I3=10-3.181815=0.4545 A

I3=3.18185=0.6363 A

Diwedd/End

# Formulae Sheet

|  |  | RT=R1+R2+…+Rn 1RT=1R1+1R2+…+1Rn 1CT=1C1+1C2+…+1Cn CT=C1+C2+…+Cn LT=L1+L2+…+Ln 1LT=1L1+1L2+…+1Ln | VOut=VinRBRA+RB I=IinRBRA+RB |
| --- | --- | --- | --- |
|  |  |  | XC=12πfC XC=-j1ωC XL=2πfL XL=jωL |
| Z=R+jXL-XC Z=R2+XL-XC2 | ω=2πf XT=XL-XC | φ=tan-1XTR pf=cosφ | fres=12πLC |
| VC=Vs1-e-tτ IC=VRe-tRC VC=V0e-tτ IC=-VRe-tRC τ=RC | τ=LR VL=Ve-RtL VL=-Ve-RtL IL=VR1-e-RtL IL=VRe-RtL | f=12πRC AV=20logVoutVin AP=10logPoutPin | Q=XLR Q=XCR Q=fRfU-fL Q=1RLC |
|  |  | E=12CV2 E=12LI2 |  |
| VB =RB2(RB1 + RB2)× VCC VE=VB-0.7 IC=IE VC=VCC-IC×RC Av=vcvb VE=10%VCC RIN=11RB1+1RB2+1hFE×RE ROUT=RC AV≈-RCRE |  | Vout=-R2R1×Vin Vout=1+R2R1×Vin Vout=R2R1×V2-V1 Vout=-R2R1×V1+V2 Vout=-RR1×V1+RR2×V2+…+RRn×Vn |  |

Detach this page and place in answer book

Student Number ………………………….	Question Number ……………………….