Vehicle Technology

Engine Performance Characteristics

Jonathan Pugh

1808090

01/01/2020

This assignment is to test an engine and evaluate the results in the questions featured above. The engine is from a Honda CRX 1.6i 16v (ED9) 130 PS (1987-›1991).  Here are some of the technical specifications and data regarding said engine:

Transverse front engine with a compression ratio of 9.5:1

4 cylinders with a cylinder volume of 398cm^3 in line 1590 cm^3 (bore 75mm x 90mm) & (stroke 1590cm^3)

16 valve single injection with 4 valves per cylinder

Power 130PS (96kW / 128 bhp) at 6800 rpm (Max RPM 7300 rpm)

Weight (EU): 1015 kg / 2238 lbs (kerb weight)

Weight/power ratio: 7,8 kg/PS (17.5 lbs/hp / 94 kW/T)

Torque weight ratio: 142 Nm/T

Fuel Consumption economical 6.9 L/100 (34.1 mpg) / Sport 12.9 L/100 (18.2 mpg)

Production range from 1983 with the end of production in 1991 and was produced in Suzuka Plant, Suzuka, Mie, in japan

The ED9 engine was produced in 1983 for the Honda civic CRX. The CRX was a front wheel drive sport compact. The CRX was originally launched as a Honda ballade but eventually changed its name to the CRX.

The CRX has a place in motorsport especially in the UK. Due to the parts being so widely available. The CRX is most commonly used in autocross, drag and road racing events

| Model | 1988 | 1989 | 1990 | 1991 | Total |
| --- | --- | --- | --- | --- | --- |
| Honda CRX HF | 17,516 | 23,846 | 22,947 | 2,617 | 66,926 |
| Honda CRX | 29,000 | 23,000 | 24,000 | 25,000 | 101,000 |
| Honda CRX Si | 18,054 | 18,735 | 14,009 | 14,447 | 65,245 |
| Total | 64,570 | 65,581 | 60,956 | 42,064 | 233,171 |

Below is a table showing production of the CRX by model year:

Table 1 : Production rate (Crxcommunity.com)

now on to the actual assignment.

The engine dynamometer test is used to obtain raw data of a load from the ED9 engine from a late 80s early 90s Honda CRX. The test will show load, speed and fuel consumption rate with the throttle opening to 100%. Then you must enter the collected data on your own into a suitable spread sheet. Then calculate the correct power, torque BMEP, SFC and overall thermal efficiency.

The equipment used is a apicomDSG dynamometer. The apicomDSG is connected to several safety features including an extractor fan, sprinkler system and safety mechanisms on all the doors. Below is the rig setup itself showing the dynamometer, engine and wiring loom.

Figure 3: apicom DSG dynamometer

Figure 2: apicom DSG dynamometer

Figure 1: apicom DSG dynamometer

the dynamometer is manufactured by a company called apicomDSG. Apicom DSG Ltd is a UK based privately owned company that supplies systems for petrol and diesel engines, transmission components and end of line testing worldwide. The apicom DSG dynamometer is a hydraulic dyno, the benefits of a hydraulic dyno is that they are relatively small compared to some other dyno types. They have a simple structure, easy operation and installation and are very accurate compared to other less efficient dynos.

Figure 4: apicom DSG dynamometer

Here is the software that was used to record the data and is again produced by apicom DSG, the software is called DaTAQ and has several versions ranging from DaTAQ Prolog with basic processes and functions to DaTAQ pro that has everything you will need with the engine dynamometer.

Figure 5: apicom DSG DaTAQ software

After having a safety tour of the facilities and equipment we then turned on the equipment and vacated the room to the observatory room. In pairs we then took it in turns to control the engine speed and dyna speed to get a feel of the dials and how much pressure was needed to control the engine.

It is very similar to driving a manual car, in that you need to keep the engine speed up to match the revs to prevent stalling. After the initial learning to handle the engine speed and dyno speed simultaneously we then learned how to control the apicom systems and how to record data with said system. The apicom DSG system is fairly intuitive and can be picked up very quickly even for a novice.

With all the systems ready and in place we then started the engine and set the limit, engine speed and time to get to said speed. The software then takes over and records data at the intervals set by the programmer. Once the system has completed all of the intervals it then slows the engine down t a speed also set by the user and comes to a complete stop. The data is then uploaded into a spreadsheet with all of the data that the user has asked for.

We were then given a copy of the spreadsheet with most of the data removed and had to use equations to calculate the missing data.

- Torque.

Torque is calculated with the equation

T=WR

W = force and R = Radius

Therefore, torque is  T = 110.79  x 0.815775

T = 90.38 Nm

- Graph of Torque against engine speed

To calculate peak torque, find the max HP which is 128.222, then find the max rpm which is 7300.

Then calculate peak torque use equation (5252 x 128.222) / 7300

Which gives an answer of 92.24958137

Then you must convert it to lbs/ft by multiplying the answer by 1.3558179483314

Giving you 125.073638148 lbs/ft

Chart 1: torque VS Engine speed

- BP or Break Power is calculated by using

BP=2πNT60

So 2pi x revolutions per sec x torque all over 60

At 1000 rpm the revolutions per second is 16.6rev/s

2πx16.6x90.381160

= 157.1137985W

- The correct power for temperature and pressure is calculated by using the equation

PsBP=1.01325paTa293

- Graph of power against engine speed

Chart 2: Power vs Engine Speed

- The specific fuel consumption (S.F.C)

S.F.C=V×0.75×3600t×1000×BP

The find V = (r^2)v

The volumetric flow rate is l/min x the specific gravity of fuel (0.739).

V = 0.074 x 0.739 = 0.0520256

BP = 157.7447776

t = ?

S.F.C=0.0520256×0.75×3600t×1000×157.7447776

= 8.905x10^-4

- Graph of S.F.C against engine speed

Chart 3: S.F.C vs Engine speed

- The piston area of the engine is calculated by this equation

A=πd24

A Honda D16A6 piston also known as a D16Z2 is 75mm (0.075m) in diameter

A=π0.07524

= 4.414x10-3 m2

- The B.M.E.P (Break Mean Effected Pressure) is calculated with the formula

BP=B.M.E.P ×L×A×N×n60×2

First the formula must be transposed to make

120 x BPL×A×N×n = B.M.E.P

L = Engine stroke = 1590cm3

A = Piston area = 4.414x10-3 m2

N = Load =110.7917011

n = number of cylinders = 4

120 x 157.74477761590 x 4.414x10-3 x 110.7917011 x 4 = B.M.E.P

B.M.E.P  = 6.086106693.

- The break thermal efficiency is calculated with this equation

ηt=(BP/1000)S.F.C (kgsec)×C.V

The calorific value of fuel is 45.8 MJ/kg (megajoules per kilogram) petrol has a slightly higher value than diesel

ηt=(157.7447776/1000)0.000890483(kgsec)×45.8

ηt = 3.867798844

| Load | Dyno Speed | Fuel Flow Rate | Lambda |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Newtons | rpm | l/min | Lambda | radius (m) | Torque | Brake Power | Volumetric flow rate | S.F.C | B.M.E.P |
| 110.792 | 1000 | 0.070 | 1.348 | 0.816 | 90.381 | 157.745 | 0.052 | 0.001 | 6.086 |
| 111.063 | 1050 | 0.072 | 1.342 | 0.816 | 90.602 | 166.037 | 0.054 | 0.001 | 6.390 |
| 111.539 | 1100 | 0.074 | 1.333 | 0.816 | 90.991 | 174.690 | 0.054 | 0.001 | 6.695 |
| 112.170 | 1150 | 0.073 | 1.323 | 0.816 | 91.506 | 183.664 | 0.054 | 0.001 | 6.999 |
| 112.938 | 1200 | 0.074 | 1.309 | 0.816 | 92.132 | 192.961 | 0.055 | 0.001 | 7.303 |
| 113.884 | 1250 | 0.079 | 1.291 | 0.816 | 92.904 | 202.684 | 0.058 | 0.001 | 7.608 |
| 114.983 | 1300 | 0.079 | 1.269 | 0.816 | 93.801 | 212.827 | 0.058 | 0.001 | 7.912 |
| 116.186 | 1350 | 0.080 | 1.241 | 0.816 | 94.782 | 223.324 | 0.059 | 0.001 | 8.216 |
| 117.338 | 1400 | 0.084 | 1.222 | 0.816 | 95.721 | 233.891 | 0.062 | 0.001 | 8.521 |
| 118.577 | 1450 | 0.085 | 1.208 | 0.816 | 96.732 | 244.802 | 0.063 | 0.001 | 8.825 |
| 119.670 | 1500 | 0.089 | 1.201 | 0.816 | 97.624 | 255.579 | 0.066 | 0.001 | 9.129 |
| 120.585 | 1550 | 0.093 | 1.201 | 0.816 | 98.370 | 266.118 | 0.069 | 0.001 | 9.433 |
| 121.357 | 1600 | 0.096 | 1.206 | 0.816 | 99.000 | 276.459 | 0.071 | 0.001 | 9.738 |
| 121.978 | 1650 | 0.099 | 1.212 | 0.816 | 99.507 | 286.559 | 0.073 | 0.001 | 10.042 |
| 122.465 | 1700 | 0.100 | 1.225 | 0.816 | 99.904 | 296.421 | 0.074 | 0.001 | 10.346 |
| 122.903 | 1750 | 0.103 | 1.236 | 0.816 | 100.261 | 306.230 | 0.076 | 0.001 | 10.651 |
| 123.391 | 1800 | 0.109 | 1.243 | 0.816 | 100.659 | 316.230 | 0.081 | 0.001 | 10.955 |
| 123.977 | 1850 | 0.114 | 1.245 | 0.816 | 101.137 | 326.557 | 0.084 | 0.001 | 11.259 |
| 124.681 | 1900 | 0.117 | 1.244 | 0.816 | 101.712 | 337.290 | 0.087 | 0.001 | 11.564 |
| 125.468 | 1950 | 0.121 | 1.244 | 0.816 | 102.354 | 348.351 | 0.089 | 0.001 | 11.868 |
| 126.314 | 2000 | 0.126 | 1.242 | 0.816 | 103.044 | 359.691 | 0.093 | 0.001 | 12.172 |
| 127.224 | 2050 | 0.128 | 1.237 | 0.816 | 103.786 | 371.339 | 0.094 | 0.001 | 12.477 |
| 128.165 | 2100 | 0.131 | 1.232 | 0.816 | 104.554 | 383.210 | 0.097 | 0.001 | 12.781 |
| 129.060 | 2150 | 0.134 | 1.225 | 0.816 | 105.284 | 395.074 | 0.099 | 0.001 | 13.085 |
| 129.835 | 2200 | 0.138 | 1.214 | 0.816 | 105.916 | 406.688 | 0.102 | 0.001 | 13.389 |
| 130.413 | 2250 | 0.141 | 1.204 | 0.816 | 106.388 | 417.785 | 0.104 | 0.001 | 13.694 |
| 130.775 | 2300 | 0.145 | 1.201 | 0.816 | 106.683 | 428.253 | 0.107 | 0.001 | 13.998 |
| 130.955 | 2350 | 0.150 | 1.200 | 0.816 | 106.830 | 438.166 | 0.111 | 0.001 | 14.302 |
| 131.048 | 2400 | 0.154 | 1.197 | 0.816 | 106.906 | 447.807 | 0.114 | 0.001 | 14.607 |
| 131.112 | 2450 | 0.156 | 1.192 | 0.816 | 106.958 | 457.358 | 0.115 | 0.001 | 14.911 |
| 131.131 | 2500 | 0.160 | 1.186 | 0.816 | 106.973 | 466.758 | 0.118 | 0.001 | 15.215 |
| 131.081 | 2550 | 0.163 | 1.181 | 0.816 | 106.933 | 475.915 | 0.120 | 0.001 | 15.520 |
| 130.936 | 2600 | 0.168 | 1.180 | 0.816 | 106.814 | 484.707 | 0.124 | 0.001 | 15.824 |
| 130.686 | 2650 | 0.171 | 1.176 | 0.816 | 106.610 | 493.084 | 0.127 | 0.001 | 16.128 |
| 130.396 | 2700 | 0.175 | 1.173 | 0.816 | 106.374 | 501.276 | 0.129 | 0.001 | 16.432 |
| 130.051 | 2750 | 0.178 | 1.171 | 0.816 | 106.092 | 509.205 | 0.132 | 0.001 | 16.737 |
| 129.711 | 2800 | 0.180 | 1.172 | 0.816 | 105.815 | 517.110 | 0.133 | 0.001 | 17.041 |
| 129.394 | 2850 | 0.183 | 1.170 | 0.816 | 105.556 | 525.055 | 0.135 | 0.001 | 17.345 |
| 129.080 | 2900 | 0.188 | 1.167 | 0.816 | 105.300 | 532.971 | 0.139 | 0.001 | 17.650 |
| 128.771 | 2950 | 0.193 | 1.162 | 0.816 | 105.048 | 540.863 | 0.142 | 0.001 | 17.954 |
| 128.520 | 3000 | 0.199 | 1.155 | 0.816 | 104.843 | 548.957 | 0.147 | 0.001 | 18.258 |
| 128.467 | 3050 | 0.206 | 1.147 | 0.816 | 104.800 | 557.877 | 0.152 | 0.001 | 18.563 |
| 128.680 | 3100 | 0.210 | 1.138 | 0.816 | 104.974 | 567.964 | 0.155 | 0.001 | 18.867 |
| 129.074 | 3150 | 0.216 | 1.126 | 0.816 | 105.295 | 578.889 | 0.160 | 0.001 | 19.171 |
| 129.473 | 3200 | 0.218 | 1.112 | 0.816 | 105.621 | 589.899 | 0.161 | 0.001 | 19.476 |
| 129.896 | 3250 | 0.223 | 1.102 | 0.816 | 105.966 | 601.073 | 0.165 | 0.001 | 19.780 |
| 130.363 | 3300 | 0.226 | 1.089 | 0.816 | 106.347 | 612.515 | 0.167 | 0.001 | 20.084 |
| 130.798 | 3350 | 0.229 | 1.079 | 0.816 | 106.702 | 623.871 | 0.169 | 0.001 | 20.388 |
| 131.215 | 3400 | 0.234 | 1.069 | 0.816 | 107.042 | 635.200 | 0.173 | 0.001 | 20.693 |
| 131.575 | 3450 | 0.236 | 1.062 | 0.816 | 107.336 | 646.311 | 0.174 | 0.001 | 20.997 |
| 131.860 | 3500 | 0.242 | 1.054 | 0.816 | 107.568 | 657.096 | 0.179 | 0.001 | 21.301 |
| 132.020 | 3550 | 0.245 | 1.051 | 0.816 | 107.699 | 667.294 | 0.181 | 0.001 | 21.606 |
| 132.027 | 3600 | 0.255 | 1.048 | 0.816 | 107.704 | 676.724 | 0.188 | 0.001 | 21.910 |
| 131.902 | 3650 | 0.258 | 1.044 | 0.816 | 107.602 | 685.473 | 0.190 | 0.001 | 22.214 |
| 131.653 | 3700 | 0.264 | 1.042 | 0.816 | 107.399 | 693.552 | 0.195 | 0.001 | 22.519 |
| 131.332 | 3750 | 0.265 | 1.040 | 0.816 | 107.137 | 701.210 | 0.196 | 0.001 | 22.823 |
| 130.907 | 3800 | 0.269 | 1.038 | 0.816 | 106.791 | 708.265 | 0.199 | 0.001 | 23.127 |
| 130.350 | 3850 | 0.274 | 1.039 | 0.816 | 106.336 | 714.527 | 0.202 | 0.001 | 23.432 |
| 129.694 | 3900 | 0.276 | 1.042 | 0.816 | 105.801 | 720.165 | 0.204 | 0.001 | 23.736 |
| 129.037 | 3950 | 0.283 | 1.045 | 0.816 | 105.265 | 725.702 | 0.209 | 0.001 | 24.040 |
| 128.423 | 4000 | 0.285 | 1.047 | 0.816 | 104.764 | 731.391 | 0.210 | 0.001 | 24.344 |
| 127.832 | 4050 | 0.287 | 1.048 | 0.816 | 104.282 | 737.126 | 0.212 | 0.001 | 24.649 |
| 127.299 | 4100 | 0.289 | 1.048 | 0.816 | 103.847 | 743.114 | 0.214 | 0.001 | 24.953 |
| 126.844 | 4150 | 0.293 | 1.048 | 0.816 | 103.476 | 749.489 | 0.217 | 0.001 | 25.257 |
| 126.404 | 4200 | 0.295 | 1.047 | 0.816 | 103.117 | 755.887 | 0.218 | 0.001 | 25.562 |
| 126.034 | 4250 | 0.297 | 1.045 | 0.816 | 102.815 | 762.646 | 0.220 | 0.001 | 25.866 |
| 125.705 | 4300 | 0.302 | 1.042 | 0.816 | 102.547 | 769.607 | 0.223 | 0.001 | 26.170 |
| 125.313 | 4350 | 0.306 | 1.039 | 0.816 | 102.227 | 776.126 | 0.226 | 0.001 | 26.475 |
| 124.869 | 4400 | 0.309 | 1.036 | 0.816 | 101.865 | 782.267 | 0.228 | 0.001 | 26.779 |
| 124.359 | 4450 | 0.310 | 1.033 | 0.816 | 101.449 | 787.925 | 0.229 | 0.001 | 27.083 |
| 123.728 | 4500 | 0.314 | 1.029 | 0.816 | 100.934 | 792.734 | 0.232 | 0.001 | 27.387 |
| 122.960 | 4550 | 0.317 | 1.026 | 0.816 | 100.308 | 796.571 | 0.234 | 0.001 | 27.692 |
| 122.056 | 4600 | 0.319 | 1.024 | 0.816 | 99.571 | 799.404 | 0.236 | 0.001 | 27.996 |
| 121.062 | 4650 | 0.321 | 1.021 | 0.816 | 98.759 | 801.508 | 0.237 | 0.001 | 28.300 |
| 120.047 | 4700 | 0.321 | 1.018 | 0.816 | 97.931 | 803.333 | 0.237 | 0.001 | 28.605 |
| 119.031 | 4750 | 0.325 | 1.018 | 0.816 | 97.103 | 805.011 | 0.240 | 0.001 | 28.909 |
| 117.941 | 4800 | 0.325 | 1.017 | 0.816 | 96.213 | 806.033 | 0.240 | 0.001 | 29.213 |
| 116.732 | 4850 | 0.326 | 1.015 | 0.816 | 95.227 | 806.079 | 0.241 | 0.001 | 29.518 |
| 115.420 | 4900 | 0.328 | 1.014 | 0.816 | 94.157 | 805.242 | 0.242 | 0.001 | 29.822 |
| 114.094 | 4950 | 0.328 | 1.014 | 0.816 | 93.075 | 804.113 | 0.243 | 0.001 | 30.126 |
| 112.766 | 5000 | 0.329 | 1.016 | 0.816 | 91.992 | 802.782 | 0.243 | 0.001 | 30.431 |
| 111.413 | 5050 | 0.330 | 1.016 | 0.816 | 90.888 | 801.076 | 0.244 | 0.001 | 30.735 |
| 110.011 | 5100 | 0.330 | 1.019 | 0.816 | 89.744 | 798.830 | 0.244 | 0.001 | 31.039 |
| 108.555 | 5150 | 0.331 | 1.020 | 0.816 | 88.556 | 795.982 | 0.245 | 0.001 | 31.343 |
| 107.061 | 5200 | 0.334 | 1.019 | 0.816 | 87.338 | 792.652 | 0.247 | 0.001 | 31.648 |
| 105.581 | 5250 | 0.331 | 1.018 | 0.816 | 86.131 | 789.213 | 0.245 | 0.001 | 31.952 |
| 104.182 | 5300 | 0.332 | 1.017 | 0.816 | 84.989 | 786.172 | 0.246 | 0.001 | 32.256 |
| 102.860 | 5350 | 0.331 | 1.017 | 0.816 | 83.911 | 783.516 | 0.244 | 0.001 | 32.561 |
| 101.672 | 5400 | 0.339 | 1.017 | 0.816 | 82.941 | 781.702 | 0.250 | 0.001 | 32.865 |
| 100.661 | 5450 | 0.344 | 1.016 | 0.816 | 82.117 | 781.102 | 0.254 | 0.001 | 33.169 |
| 99.880 | 5500 | 0.349 | 1.013 | 0.816 | 81.480 | 782.150 | 0.258 | 0.001 | 33.474 |

| Baro | Humidity | Ambient temp |
| --- | --- | --- |
| mBar | % | Deg C |
| 1024.55 | 39.79 | 24 |

Table 2: excel dynamometer results

References

Table 1: Crxcommunity.com. (2020). CRX Community Forum • View topic - CRX Suspension Tech, READ if you're a suspension newbie!. [online] Available at: https://crxcommunity.com/viewtopic.php?t=4900&postdays=0&postorder=asc&start=0 [Accessed 1 Jan. 2020].