#include <main.h>

#define LCD_ENABLE_PIN PIN_C0

#define LCD_RS_PIN PIN_C2

#define LCD_RW_PIN PIN_C1

#define LCD_DATA4 PIN_C4

#define LCD_DATA5 PIN_C5

#define LCD_DATA6 PIN_C6

#define LCD_DATA7 PIN_C7

#include <lcd.c>

unsigned int A=0, B=0, C=0;

int but1_prev_state = 0;

int but2_prev_state = 0;

int buttoncounter = 1;

void readabc		(void);

void displayabc		(void);

void buttoninput	(void);

void CWmotor		(void);

void ACWmotor		(void);

void stopmotor		(void);

void displayLED		(void);

void main()

{

setup_adc_ports(sAN0 | sAN1 | sAN2, VSS_VDD); //this sets up the ADC ports for the potentiometers

setup_adc(ADC_CLOCK_INTERNAL | ADC_TAD_MUL_0); //this function sets up the operation of the ADC

lcd_init(); 					 //get the lcd ready for information

while(TRUE)					//calling all the needed function when the circuit is "true"

{

readabc();

displayabc();

buttoninput();

displayLED();

}

}

void readabc()

{

set_adc_channel(0);				 //select ADC channel 0

delay_us(30); 					 //delay

A = read_adc(); 				 // read ADC value and save to A

set_adc_channel(1);				 //select ADC channel 1

delay_us(30);  					//delay

B = read_adc();  				// read ADC value and save to B

set_adc_channel(2);				//select ADC channel 2

delay_us(30);  					//delay

C = read_adc(); 				// read the ADC value and save to C

}

void displayabc()

{

lcd_gotoxy(7,1);			//goto center of line 1

printf(lcd_putc,"A: %u ", A);		//show the value of A on the lcd screen

lcd_gotoxy(7,2);			//goto center of line 2

printf(lcd_putc,"B: %u ", B);		//show the value of B on the screen

lcd_gotoxy(27,1);			//goto center of line 3

printf(lcd_putc,"C: %u ", C);		//show the value of C on the screen

}

void buttoninput()

{

if(input(Pin_A5) && !but2_prev_state)		//checks button input and views if the previos state of the button is 0

{

switch(buttoncounter)			//switch statement

{

case 1: CWmotor();

lcd_gotoxy(27,2);			//display on the centre of line 4

printf(lcd_putc,"I    "); 		//display the task number of section 3(i)

buttoncounter=buttoncounter+1;		//add one to button counter so it knows it has added one in order to execute the next tasks when button is pressed

break;

case 2: stopmotor();

lcd_gotoxy(27,2);			//display on the centere of line 4

printf(lcd_putc,"II");			//print the task number of section 3 (ii)

buttoncounter=buttoncounter+1;		//add one to button counter so it knows it has added one in order to execute the next tasks when button is pressed

break;

case 3: ACWmotor();

lcd_gotoxy(27,2);			//display on the centere of line 4

printf(lcd_putc,"III");			//print the task number of section 3 (iii)

buttoncounter=buttoncounter+1;		//add one to button counter so it knows it has added one in order to execute the next tasks when button is pressed

break;

case 4: stopmotor();

lcd_gotoxy(27,2);			//display on the centere of line 4

printf(lcd_putc,"IV   ");		//print the task number of section 3 (iv)

buttoncounter=1;			//add one to button counter so it knows it has added one in order to execute the next tasks when button is pressed

break;

default:	break;

}

}

but2_prev_state = input(Pin_A5);

}

void CWmotor()					//call CWmotor function

{

output_low(PIN_A7);				//low output on pin A7

output_high(PIN_A6);				//high output on pin A6

}

void ACWmotor()

{

output_low(PIN_A6);				//low output on pin A6

output_high(PIN_A7);				//high output on pin A7

}

void stopmotor()

{

output_low(PIN_A6);				//low output on pin A6

output_low(PIN_A7);				//high output on pin A7

}

void displayLED()

{

if(input(Pin_A4) && !but1_prev_state) 	//if there is an input on button 1 and previous state is 0

{

output_b(A & 0xFF);

delay_ms(3000);

output_b(0x00);

output_b(B & 0xFF);

delay_ms(3000);

output_b(0x00);

output_b(C & 0xFF);

delay_ms(3000);

output_b(0x00);

}

but1_prev_state = input(Pin_A4);

}

Comments

- You show conversion values for each potentiometer nicely centred on each line. You have nicely written functions for capturing the values from the potentiometers. 10/10

- The value of each ADC is shown correctly on the LEDs for 3 seconds each. You have nicely written functions for doing this. 10/10

- You have correctly created a state machine to control the rotation of the motor, correctly testing for the button to be released stopping the state machine from changing state unintentionally. 30/30