Figure 1- BEAST Zetec Track (BZT) (Blitz World, 2019)

Series Hybrid Vehicle Pt.1a

Alternative Powertrain Technology

M6X02209

Toby Marshall (1702695)

Contents

Introduction	3

Off-road Buggy	3

Design of Off-Road Track	4

Methodology	5

Acceleration	5

Velocity	5

Rolling Circumference	5

Frontal Aero Area	5

Gradient	5

Aero Resistance	5

Rolling Resistance Drag	6

Gradient Resistance	6

Analysis	6

Conclusion	7

References	8

Appendices	9

Table 1 Buggy Specification (Blitz World, 2019)	8

Table 2 Buggy Calculated Specification	9

Table 3 Calculations one	9

Table 4 Calculation two	9

Table 5 Calculation three	10

Table 6 Calculations four	10

Table 7 Calculations Five	10

Figure 1- BEAST Zetec Track (BZT) (Blitz World, 2019)	1

Figure 2-BEAST Zetec Track (BZT) (Blitz World, 2019)	3

Figure 3- Off-road Track Created for report.	4

# Introduction

The report will look into the duty cycle for an off-road buggy used for corporate events. The report will evaluate the vehicles performance, top speed, acceleration and grade-ability. A drive cycle/duty cycle is a fixed schedule of vehicle operation which allows for a vehicles performance and operation to be monitored for comparison and validation. (Barlow, Latham, McCrae, & Boulter, 2009) The Track used in this report was created to produce a duty cycle to evaluate power requirement, tractive force, acceleration and grade-ability of an off-road buggy.

# Off-road Buggy

The report has been developed around the ‘BEAST Zetec Track (BZT)’ manufactured by BlitzWorld. The buggy is manufactured for off-road use and climbing steep inclines, the buggy has a NASCAR style roll cage essential for corporate use whilst remaining entertaining but safe. The specs for the buggy can be seen in Table 1 in the appendices. (Blitz World, 2019)

Figure 2-BEAST Zetec Track (BZT) (Blitz World, 2019)

An electric off road buggy which can perform the same duty cycle and performance as a petrol engine equivalent boasts benefits which could allow it to have precedence over petrol engine buggies for corporate use. An electric buggy promotes quite engines allowing for running within noise restrictions, zero local emissions and potential for renewable energy sources for recharge. Corporate buggies have key possible for electrification due to their short range use and availability for recharge close to their tack use which is normally an issue for off-road buggies which require a long range before recharge. (Scrosati, Garche, & Tillmetz, 2015)

# Design of Off-Road Track

The track is a recreated of Anglesey National Circuit, however the track has be created with a gradient of 22m within the circuit and the tracks surface is gravel with a small tarmac section, an air density of 1.2 due the circuit be at sea level with a track length of 1.2 miles long. (Anglesey Circuit, 2019) The specs of the circuit can be found in table 2 in the appendices.

Start / Finish

Figure 3- Off-road Track Created for report.

# Methodology

Listed are the equation used to calculate values created in the excel spread.

## Acceleration

mph2.2374=m/s

(Engineering ToolBox, 2011)

## Velocity

DistanceTime

(Engineering ToolBox, 2011)

## Rolling Circumference

CπD

Wheel Size2+Tyre Sidewall2π

(x engineer, 2019)

## Frontal Aero Area

H - Height

Gc – Ground Clearance

W - Weight

H-GcW

## Gradient

G=asinElavationDistance

G 180π=G degrees

(x engineer, 2019)

## Aero Resistance

Fu= 12 PAfCd(V+Vu)2

V – Velocity

Cd – Coefficient of Drag

P – Air Density

Af –Frontal Aero Area

(Ehsani, Gao, Gay, & Emadi, 2005)

## Rolling Resistance Drag

RR= gMvFr

Fr – Rolling Coefficient

Mv – Vehicle Weight

g – Gravity

(Engineering ToolBox, 2008)

## Gradient Resistance

Fg=Mvgsina

Mv – Vehicle Weight

g – Gravity

a – Gradient (degrees)

(Ehsani, Gao, Gay, & Emadi, 2005)

# Analysis

The duty cycle in the excel spread sheet evaluates the tack and the capabilities of the buggy and whether it holds realistic values.

Each section shows power required to complete each section with realistic acceleration and grip. The buggy’s maximum acceleration is 6.7 m/s2, during the duty cycle the acceleration doesn’t exceed 3.7 m/s2 and doesn’t exceed 3.1 m/s2 when decelerating, the acceleration being of these values means the buggy’s is realistic for the duty cycle of the track. To calculate an accurate power required requires accurate acceleration through each section which has been validated above. The total power required is realistic for the buggy as throughout the duty cycle the buggy does not exceed its maximum power output available from the buggy’s engine.

The total resistance forces (Aero, Gradient, and Rolling Drag) of the track are realistic as the buggy does not exceed its power available when overcoming these forces. The vehicle reaches a top speed of 46 mph during its duty cycle, this is well below the buggy’s top speed of 130 mph. However, the duty cycle is predominately on a gravel surface where this speed is realistic and adequate to provide entertaining yet safe experience.

# Conclusion

The duty cycle of the track is realistic and validated through calculation contained in the excel spread sheet.

The duty cycle has realistic acceleration of the buggy whilst traveling on the track due to the buggy not exceeding its maximum acceleration.  The maximum acceleration of the buggy is also used to validate the deceleration of the buggy, where if the buggy’s deceleration exceed its maximum acceleration the buggy would not be able to maintain traction. However, in the duty cycle the buggy does not exceed maximum acceleration value during its deceleration, showing the buggy maintains traction throughout the duty cycle.

The force resistance which the buggy experiences are realistic during the duty cycle due to the buggy not exceeding its maximum available power overcoming the forces whilst still maintaining  a relatively fast speed during the duty cycle.

The power available is calculated by the buggy’s engine power output. During the duty cycle the buggy does not exceed the power available when traveling through each sector. Helping Validate the duty cycle and values produced from calculations. The excel spread sheet’s validation allows for accurate modelling for a specification for alternatively powered buggies. This generated specification would allow for an electric power system to be evaluated for use with the buggy round the track’s duty cycle.

# References

(n.d.).

Anglesey Circuit. (2019, 11 29). NATIONAL. Retrieved from Anglesey Circuit: https://www.angleseycircuit.com/venue/circuit-configurations/national/

Barlow, T., Latham, S., McCrae, I., & Boulter, P. (2009). A Reference Book of Driving Cycles for use in the Measurment of Road Vehicle Emissions. Crowthorne: TRL Limited.

Blitz World. (2019, 11 29). BEAST Zetec Track (BZT). Retrieved from Blitzworld: https://www.blitzworld.co.uk/high-performance/BEAST-Off-Road

Ehsani, M., Gao, Y., Gay, S. E., & Emadi, A. (2005). Modern Electric, Hybrid Electric, and Fuel Cell Vehicles. New York: CRC Press.

Engineering ToolBox. (2008). Rolling Resistance. Retrieved 11 29, 2019, from Engineering ToolBox: https://www.engineeringtoolbox.com/rolling-friction-resistance-d_1303.html

Engineering ToolBox. (2011). Acceleration and Velocity Equations. Retrieved 11 29, 2019, from Engineering ToolBox: https://www.engineeringtoolbox.com/acceleration-velocity-d_1769.html

Engineering ToolBox. (2011). Traction Force. Retrieved 11 29, 2019, from Engineering ToolBox: https://www.engineeringtoolbox.com/tractive-effort-d_1783.html

Scrosati, B., Garche, J., & Tillmetz, W. (2015). Advances in Battery Technologies for Electric Vehicles. Cambridge: Woodhead Publishing.

x engineer. (2019, 11 29). How to calculate wheel radius. Retrieved from x engineer: https://x-engineer.org/automotive-engineering/chassis/longitudinal-dynamics/calculate-wheel-radius/

x engineer. (2019, 11 29). Introduction to trigonometry. Retrieved from x engineer: https://x-engineer.org/undergraduate-engineering/mathematics/trigonometry/introduction-to-trigonometry/

# Appendices

| Engine CC | 2000 CC Zetec |
| --- | --- |
| Power (BHP) | 150 |
| Torque (Nm) | 174 |
| 0-60 MPH (s) | 4 |
| Top Speed MPH | 130 |
| Length mm | 3000 |
| Width mm | 1600 |
| Height mm | 1500 |
| Weight KG | 500 |
| C.O.G | Low |
| Ground Clearance mm | 290 |
| Wheel Centres mm | 2230 |
| Wheel Size m | 0.3 |
| Tyre Sidewall m | 0.17 |
|  | (Blitz World, 2019) |

Table 1 Buggy Specification (Blitz World, 2019)

| Buggy Specification |  |
| --- | --- |
| Acceleration (m/s2) | 6.70540903 |
| Power (Kw) | 111.8568233 |
| Top Speed (m/s) | 58.11354493 |
| Frontal Aero Area (m2)(Assumption) | 1.936 |
| Coefficient of Drag (Assumption) | 0.35 |
| Rolling Radius (m) | 2.010619298 |
| Rolling Resistance Coefficient (Gravel) | 0.02 |
| Rolling Resistance Coefficient(Tarmac) | 0.015 |
| Gravity (m/s2) | 9.8 |
|  | (Engineering ToolBox, 2008) |
| Circuit |  |
| Air Density | 1.2 |
| Length | 1.21 Miles |
| Max Elevation (m) | 22 |

Table 2 Buggy Calculated Specification

| Sectors / (Surface) | Gradient | Elevation Above Sea Level (m) | Change in Elavation (m) | Gradient (degrees) |
| --- | --- | --- | --- | --- |
| Start / (Gravel) |  | 5 | 0 | 0 |
| Striaght / (Gravel) | Downhill | 0 | -5 | -1.234918445 |
| Turn 1 / (Gravel) | Uphill | 5 | 5 | 1.675554005 |
| Stright / (Tarmac) | Uphill | 8 | 3 | 1.420702096 |
| Turn 2 / (Tarmac) | Uphill | 22 | 14 | 14.47751219 |
| Turn 3 / (Tarmac) | Flat | 22 | 0 | 0 |
| Straight / (Gravel) | Flat | 22 | 0 | 0 |
| Turn 4 / (Gravel) | Downhill | 15 | -7 | -3.289269227 |
| Straight / (Gravel) | Downhill | 10 | -5 | -1.80205116 |
| Turn 5 / (Gravel) | Downhill | 5 | -5 | -3.539026459 |
| Straight / (Gravel) | Flat | 5 | 0 | 0 |
| Turn 6 / (Gravel) | Flat | 5 | 0 | 0 |
| Staright / (Gravel) | Flat | 5 | 0 | 0 |
| Finiish / (Gravel) |  |  |  |  |

Table 3 Calculations one

| Distance (m) | Distance Covered (m) | Velocity (m/s) | Acceleration (m/s2) |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 232 | 232 | 15.46666667 | 1.031111111 |
| 171 | 403 | 21.375 | 0.738541667 |
| 121 | 524 | 20.16666667 | -0.201388889 |
| 56 | 580 | 18.66666667 | -0.5 |
| 111 | 691 | 13.875 | -0.598958333 |
| 156 | 847 | 39 | 6.28125 |
| 122 | 969 | 18.76923077 | -3.112426036 |
| 159 | 1128 | 21.2 | 0.324102564 |
| 81 | 1209 | 16.2 | -1 |
| 388 | 1597 | 48.5 | 4.0375 |
| 81 | 1678 | 54 | 3.666666667 |
| 253 | 1931 | 63.25 | 2.3125 |
|  |  |  | Total- |

Table 4 Calculation two

| Time Taken (s) | Accumilative Time (s) | Rolling Drag (N) | Aero Resistance (N) |
| --- | --- | --- | --- |
| 0 | 0 | 98 | 0 |
| 15 | 15 | 98 | 97.25637973 |
| 8 | 23 | 98 | 185.7534525 |
| 6 | 29 | 73.5 | 165.3456933 |
| 3 | 32 | 73.5 | 141.6635733 |
| 8 | 40 | 73.5 | 78.2691525 |
| 4 | 44 | 98 | 618.37776 |
| 6.5 | 50.5 | 98 | 143.2245927 |
| 7.5 | 58 | 98 | 182.7243264 |
| 5 | 63 | 98 | 106.6976064 |
| 8 | 71 | 98 | 956.33076 |
| 1.5 | 72.5 | 98 | 1185.52896 |
| 4 | 76.5 | 98 | 1626.46869 |
| 76.5 |  |  |  |

Table 5 Calculation three

| Gradient Resistance (N) | Total Resistance Forces (N) | Total Resistive Power (W) |
| --- | --- | --- |
| 0 | 98 | 0 |
| -4626.194495 | -4430.938115 | -68531.84284 |
| 4873.13786 | 5156.891313 | 110228.5518 |
| 4844.90926 | 5083.754954 | 102522.3916 |
| 4618.933426 | 4834.096999 | 90236.47732 |
| 0 | 151.7691525 | 2105.796991 |
| 0 | 716.37776 | 27938.73264 |
| 720.9879294 | 962.2125221 | 18059.98888 |
| -4769.559819 | -4488.835492 | -95163.31244 |
| 1896.561861 | 2101.259467 | 34040.40337 |
| 0 | 1054.33076 | 51135.04186 |
| 0 | 1283.52896 | 69310.56384 |
| 0 | 1724.46869 | 109072.6446 |
|  | Total- | 450955.4376 |

Table 6 Calculations four

| Power Remaining (W) | Average Power / Second / Section | Power Required (W) |
| --- | --- | --- |
| 111855 | 0 | 0 |
| 180386.8428 | -4568.789523 | 68531.84284 |
| 1626.448192 | 13778.56898 | 110228.5518 |
| 9332.608437 | 17087.06526 | 102522.3916 |
| 21618.52268 | 30078.82577 | 90236.47732 |
| 109749.203 | 263.2246239 | 2105.796991 |
| 83916.26736 | 6984.68316 | 27938.73264 |
| 93795.01112 | 2778.459827 | 18059.98888 |
| 207018.3124 | -12688.44166 | 95163.31244 |
| 77814.59663 | 6808.080673 | 34040.40337 |
| 60719.95814 | 6391.880233 | 51135.04186 |
| 42544.43616 | 46207.04256 | 69310.56384 |
| 2782.355358 | 27268.16116 | 109072.6446 |

Table 7 Calculations Five