| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 6 | LEFEL 6 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| January 2016 | IONAWR 2016 |

MODULE CODE: M3X8858

Energy and Environmental Technology

TIME:  2 HOURS

answer Three quESTIONs out of four

Equipment or materials required – Calculators may be used

Adnoddau neu ddefnyddiau sydd eu hangen - Dim

ANSWER Three QUESTIONS

Question 1

With demand for increased energy performance it is essential that heat loss from buildings is minimised.

- An office has an external wall with a U-value of 1.25 W/mK and contains air at 20°C when the outside temperature is 1°C.  The internal surface resistance is 0.15m K/W.

Calculate the temperature on the internal surface of the wall using the following formula.

∆θθT = RRT

(6 marks)

- Explain what is meant by internal heat gains and their importance in designing heating systems.

(5 marks)

- A building has a calculated fabric heat loss of 140kW when the outside temperature is 4°C and the inside temperature is 21°C.  The volume of the building is 92m3 and there are four air changes per hour.  The constant is equivalent to 1300 J/mK.

QV = Constant × n × V × ∆T3600

Calculate the total heat loss from the structure.

(6 marks)

- A building is occupied for 5 days per week for 16 hours a day.  It is of a heavyweight construction and is heated by an intermittent responsive plant with a seasonal efficiency of 80%.  Natural gas costs 14.0p/kWh and is located in a region which experiences 2825 degree days per year.  Using the heat loss value calculated in part (c) and the attached values in Tables 1, 2, and 3 calculate the annual energy demand and the annual fuel costs using the equation below.  The inside temperature is 21°C and the outside temperature 4°C.

E = QTTai-  Tao ×SDD ×24 × 1η

(8 marks)

Question 2

Geothermal energy is heat from within the earth and it can be used to generate electricity or provide heat for buildings.

- Explain two geothermal technologies that can be used to generate electricity and the issues associated with their use.

(10 marks)

- Discuss the environmental implications of using geothermal energy to provide heat and generate electricity.

(10 marks)

- Explain the principle behind how ground source heat pumps are used to provide heating in buildings.

(5 marks)

Question 3

Evaluate the main stages of the nuclear fuel cycle, clearly indicating any areas where environmental pollution could occur.

(25 marks)

Question 4

Increasingly, work areas and offices utilise air conditioning systems to provide an all year round comfortable working environment.

AED = Q x SDD/dt

AEC = 0.5 x AED

- Evaluate what factors need to be taken into account when designing air conditioning systems.

(10 marks)

- A company occupies a three storey office building which has a floor area of 600m2 per floor and has occupancy of one person per 6m2 of floor.  The heat gains are 110W per person; lighting 12W/m2 and desktop computer which are evenly spread throughout the building are 23W/m2.  The air conditioning plant is centralised.  External heat gains are 80kW and summer design indoor and outdoor air temperatures are 20°C and 31°C respectively.  Standard degree hours are 6,250.

- Estimate the annual energy consumption.

(9 marks)

- Determine the performance indicator for the building.

(2 marks)

- Calculate the annual carbon dioxide emissions using a factor of 0.27.

(4 marks)

Diwedd/End

Table 1:	Correction Factor for Length of Working Week

| Occupied Days per Week | Lightweight building | Heavyweight building |
| --- | --- | --- |
| 7 days | 1.0 | 1.0 |
| 5 days | 0.75 | 0.85 |

Table 2:	Correction Factor Length of Working Day

| Occupied Period | Lightweight building | Heavyweight building |
| --- | --- | --- |
| 4 hours | 0.68 | 0.96 |
| 8 hours | 1.00 | 1.00 |
| 12 hours | 1.25 | 1.02 |
| 16 hours | 1.40 | 1.03 |

Table 3:	Correction Factor for the Response of Building and Plant

| Type of heating | Lightweight building | Medium weight building | Heavyweight building |
| --- | --- | --- | --- |
| Continuous | 1.00 | 1.00 | 1.00 |
| Intermittent (responsive plant) | 0.55 | 0.70 | 0.85 |
| Intermittent (with long time lag) | 0.70 | 0.85 | 0.95 |