Applied Computing

Wales Institute of Science and Art (WISA)

ACCA6023

Design, Configure and Troubleshoot Multiarea OSFP

Objectives:

Part 1

- Design network as per the topology stated

- Configure Basic security and connectivity Tasks

Part 2

- Configure Multiarea OSPFv2

- Verify and Examine Multiarea OSPFv2

Part 1

- Create a topology and assign IP Address as stated in the below design.

- Configure Hostnames, Line, passwords, banner, security and activate the network.

Part 2

- Configure OSPFv2

Use the following requirements to configure OSPF routing on all routers:

- Process ID (as required)

- Router ID for each router (as required)

- Network address for each interface

- LAN interface set to passive (do not use the default keyword)

- Advertise each directly connected network in OSPFv2 (as required).

- Verify connectivity to each of the OSPFv2 areas.

- Record the command used to configure and verify the connectivity.

- Repeat the step from 4 to 7 and configure OSPF v3.

OSPFv2 Configuration

router ospf 1

router-id 1.1.1.1 (Must be unique – 32 Bit)

network Network Address Wildcard Mask area Area Number

default-information originate (For a Default Route – External Network ASBR)

passive-interface Gi0/0/0

interface GigabitEthernet0/0

ip ospf hello-interval 5

ip ospf dead-interval 20

ip ospf priority 150

OSPFv3 Configuration

ipv6 unicast-routing

ipv6 router ospf 1

router-id 2.2.2.2

interface GigabitEthernet0/0

ipv6 ospf 1 area 0

interface Serial0/0/0

ipv6 ospf 1 area 0

Troubleshooting Commands

OSPF v2

show ip ospf neighbor

show ip protocols

show ip ospf

show ip ospf interface

OSPF v3

show ipv6 interface brief

show ipv6 protocols

show ipv6 ospf interfaces brief

show ipv6 ospf neighbor

clear ipv6 ospf process

show ipv6 route

show ipv6 route ospf

show ipv6 ospf database