APPLICATIONS OF

BERNOULLI’S EQUATION

Problem 1

Oil of density 800 kg/m3 enters a 500 mm diameter pipeline with a velocity of 3 m/s and pressure 400 kPa, and is discharged through an orifice 150 mm in diameter and 30 m below the entry point. If the frictional losses are equivalent to a head of 3.5 m of the oil, determine the velocity and pressure of the oil at the point of discharge.

SOLUTION

PA = 400 kPa

ΦA = 0.5 m

vA = 3 m/s

A

ρ =  800 kg/m3

frictional losses:   hf = 3.5 m

oil

flow

30 m

ΦB = 0.15 m

vB = ?

B

From continuity equation:   Q=AreaA.vA=AreaB.vB   ⇒   AA.vA=AB.vB

Therefore:

vB=AA.vAAB=π0.524×3π0.1524=0.52×30.152=33.3 m/s

Applying Bernoulli's equation:

PAρ.g+vA22.g+hA=PBρ.g+vB22.g+hB+hf

400×103800×9.81+322×9.81+30=PB800×9.81+33.322×9.81+3.5

PB800×9.81=400×103800×9.81+322×9.81+30-33.322×9.81-3.5

PB=400×103800×9.81+32-33.322×9.81+26.5×800×9.81

PB=167.13 kN/m2

PB=1.67 bar

Problem 2

An open water tank discharges water to the atmosphere through a pipe at its base. The height of the water in the tank above the discharge pipe is 5 m.

- With what velocity will the water emerge through the discharge pipe?

- What will be the volume flow rate from the outlet pipe if the discharge pipe has a diameter of 100 mm?

SOLUTION

A

5 m

B

The pressures at the open surface in the tank and at the outlet are both the same as they are open to the atmosphere, PA = PB = Patm. Assuming the height drops very slowly due to the leak, we may set the velocity at the upper surface vA = 0.

Bernoulli's equation becomes:

Patmρ.g+02.g+hA=Patmρ.g+vB22.g+hB

vB2=2.ghA-hB

vB=2×9.81×5=9.9 m/s

And:

Q=AB.vB=π0.124×9.9=77.8×10-3 m3/s

Problem 3

A 50 mm diameter pipe-line falls a vertical distance of 30 m from an open oil reservoir and discharges into an open tank. The head of oil above the pipe entrance is 6 m and the loss of head due to pipe friction is 3.6 m. Calculate the discharge in the atmosphere in litres per second and in tonnes per hour. Relative density is 0.8. What is the loss of power due to friction, in kilowatts?

SOLUTION

A

6 m

ρ =  800 kg/m3

frictional losses:   hf = 3.6 m

30 m

B

ΦB = 0.05 m

Applying Bernoulli's equation:

PAρ.g+vA22.g+hA=PBρ.g+vB22.g+hB+hf

Patmρ.g+02.g+36=Patmρ.g+vB22.g+3.6

vB=36-3.6×2×9.81=25.2 m/s

Therefore:

Q=AB.vB=π0.0524×25.2=0.0495 m3/s

And:

m=Q.ρ=0.0495×800=39.6 kg/s

=39.6×36001000 tonne/h

=142.58 tonne/h

The energy lost in friction, i.e. the loss of power, is given by:

loss of power = mass flow rate  g  hf

kg/s	      m

J/s  or  Watt	     m/s2

Therefore:

loss of power=39.6 ×9.81×3.6

∴    loss of power=1,398.7 W

Problem 4

The diameter of a pipe tapers gradually in the direction of oil flow as the level drops 9 m from point A to point B. The relative density of oil is 0.9. At A the gauge pressure is 210 kN/m2 and the pipe diameter is 200 mm; at B the diameter is 100 mm. The frictional losses are equivalent to a head of 2.5 m of oil. What is the pressure at B when the flow rate is 226.8 tonnes per hour?

SOLUTION

ΦA = 0.2 m

PA = 210 kN/m2

A

ρ =  900 kg/m3

frictional losses:   hf = 2.5 m

oil

flow

9 m

ΦB = 0.1 m

B

The mass flow rate m is given:

m=Q×ρ

=226.8 tonnes/hr

=226.8×10003600 kg/s

=63 kg/s

The density of oil is 900 kg/m3, therefore the volumetric flow rate Q is:

Q=mρ

=63900 m3/s

=0.07 m3/s

From continuity equation:   Q=AreaA.vA=AreaB.vB   ⇒   0.07=AA.vA=AB.vB

Therefore:

vA=QAA=0.07π4×0.22=2.2 m/s

vB=QAB=0.07π4×0.12=8.9 m/s

Applying Bernoulli's equation:

PAρ.g+vA22.g+hA=PBρ.g+vB22.g+hB+hf

210×103900×9.81+2.222×9.81+9=PB900×9.81+8.922×9.81+2.5

PB900×9.81=210×103900×9.81+2.222×9.81+9-8.922×9.81-2.5

PB=210×103900×9.81+2.22-8.922×9.81+6.5×900×9.81

PB=233.9 kN/m2

Problem 5

Water flows down a pipe from a level A to a level B. The pipe is 250 m long and is inclined at 52° to the horizontal. The pipe is tapered from a diameter of 35 cm at A to a diameter of 25 cm at B. Knowing:

- the pressure head at A is 17 m,

- the pressure head at B is 26 m,

- the velocity of water at B is 24×103 cm/hr.

Find the loss of power due to friction in Watts.

Pressure HeadB = 26m

A = 0.35m

Pressure HeadA = 17m

B = 0.25m

height

=

250.sin52°

=

197 m

52°

SOLUTION

vB = 24×103 cm/hr

=240 m3600 s

vB=0.067 m/s

- Applying the continuity equation:

AA∙vA    =    AB∙vB

⇒   π∅A24vA = π∅B24vB

⇒   vA=∅B∅A2vB

⇒   vA=0.250.352×0.067

⇒   vA=0.034 m/s

- Applying Bernoulli's equation:

PAρ.g+vA22.g+hA=PBρ.g+vB22.g+hB+hf

17+0.03422×9.81+197=26+0.06722×9.81+0+hf

hf=17+0.03422×9.81+197-26-0.06722×9.81

hf=17+0.03422×9.81+197-26-0.06722×9.81

hf=188 m

- The mass flow rate m is given by:

m=Q×ρ

=AA.vA×ρ

=π∅A24vA×ρ

=π×0.3524×0.034×1000

=3.27 kg/s

- The energy lost in friction, i.e. the loss of power, is given by:

loss of power = mass flow rate  g  hf

kg/s	      m

J/s  or  Watt	     m/s2

Therefore:

loss of power=3.27 ×9.81×188

loss of power=6,035.4 W

Problem 6

An open tank contains oil of relative density 0.85 to a depth of 2.4 m. It discharges the oil to the atmosphere through a 25 mm diameter straight pipe at a point 6 m below the bottom of the tank. Ignoring losses, determine the discharge in litres per second and tonnes per hour.

SOLUTION

v1 = 0

P1 = P

2.4 m

6 m

v2 = ?

P2 = P

Φ2 = 25mm

Applying Bernoulli's equation gives:

P1ρ+v122+g∙h1=P2ρ+v222+g∙h2

The pressure at the open surface in the tank and at the outlet are both the same as they are open to the atmosphere. The velocity at the upper water surface in the tank is assumed to be zero. Hence:

Pρ+0+g.6+2.4=Pρ+v222

v222=g×8.4

v22=2×10×8.4

v2=2×10×8.4

v2=12.96 m/s

The volumetric flow rate Q is given by:

Q =A2.v2

=π4×0.0252×12.96

=6.4×10-3 m3/s

Q=6.4 litres/s      as  1 m3=1,000 litres

The mass flow rate m is given by:

m=Q×ρ

=6.4×0.85  kg/s

=5.4 kg/s

=5.4×36001000  tonne/h

m=19.4  tonne/h

Problem 7

A tank of water is filled to a height h1 of 2m. Water sprays out from a small opening at a height h2 on the tank’s side, and lands in a glass located at a horizontal distance of 1.5m from the tank. Determine the height h2 of the opening if the glass stands at a height h3 of 0.5 m as shown in the following figure. Neglect frictions and glass’ dimensions.

h1 = 2 m

h2

h3 = 0.5 m

d = 1.5 m

SOLUTION:

Applying Bernoulli's equation gives:

P1ρg+v122g+h1=P2ρg+v222g+h2

The pressure at the open surface in the tank and at the outlet are both the same as they are open to the atmosphere. The velocity at the upper water surface in the tank is assumed to be zero. Hence:

Pρg+0+2=Pρg+v222g+h2

∴    v222g=2-h2

∴    v22=2g2-h2

∴    v22=39.24-19.62h2

If the jet of water travels for a time t before hitting the glass, then from linear motion law,

s=v0t+12at2

- Motion along x axis:

d=v2t      ∴       t=dv2

- Motion along y-axis:

h2-h3=12gt2

∴ h2-h3=gd22v22=gd2239.24-19.62h2

∴ h2-0.5=9.81×1.5278.48-39.24h2

∴ h2-0.578.48-39.24h2=9.81×1.52

∴ 78.48h2-39.24h22-39.24+19.62h2=9.81×1.52

∴ 39.24h22-98.1h2+61.3125=0

∴ h2=98.1∓98.12-4×39.24×61.31252×39.24=98.178.48=1.25m