| LOCATION Birmingham | LLEOLIAD BIRMINGHAM |
| --- | --- |
| LEVEL 5 | LEFEL 5 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| May 2023 | medi 2022 |

MODULE CODE: ACCA5024

Advance Networking

TIME:  3 Hours                                                          TOTAL: 100 MArks

answer all questions

Name :

Student ID :

Date & Time :

This is an individual practical test, held under examination conditions. You will be using the software “Packet tracer” for the given network configuration; you should configure the devices based on the given specifications.

ANSWER ALL Tasks

# Topology

# Scenario

You are required to build and configure the devices in a small network as shown in the above topology. You must configure a router, switch and PCs to support both IPv4 and IPv6 connectivity. In addition, you will test and document the network using common CLI commands.

# Task 1: Develop an IP Addressing Scheme (50 Marks)

Your allocated IPv4 address is 192.168.202.0/24 and IPv6 address is 2001:db8:acad:0000::/64. Host requirements for subnet A is  45 and subnet B is 100. You will subnet it to provide IP addresses to the two subnets that will support the required number of hosts. No subnet calculators may be used. All workout must be shown. Results should be recorded in the IP Addressing table below.

- I- Develop the IP addressing scheme and fill the tables (20 Marks)

| Specification | Subnet A | Subnet B |
| --- | --- | --- |
| Subnet IP address |  |  |
| Subnet mask (slash notation) |  |  |
| Subnet mask (dotted decimal) |  |  |
| 1st Host IP address |  |  |
| Last Host IP address |  |  |
| Number of usable hosts per subnet |  |  |
| Broadcast address |  |  |

- II-Record subnet assignment (10 Marks)

Assign the first IPv4 address of the relevant subnet to each router interface

Assign the last IPv4 address of the relevant subnet to each PC NIC

Assign the second IPv4 address of the relevant subnet to S1

| Device | IP address | Subnet Mask | Gateway |
| --- | --- | --- | --- |
| PC-A |  |  |  |
| R1-G0/0/0 |  |  |  |
| R1-G0/0/1 |  |  |  |
| S1 |  |  |  |
| PC-B |  |  |  |

- III- Use the allocated address to create two subnets for use in this network. Record the IPv6 addresses in the table (10 marks)

| Assigned to Interface | IPv6 Subnet Address | Prefix Length |
| --- | --- | --- |
| R1-G0/0/1 |  |  |
| R1-G0/0/0 |  |  |

- IV-Record the IPv6 address information for each device (10 Marks)

Note: Use FE80::1 as the link-local address on both router interfaces.

| Device | IPv6 address | Prefix Length | Gateway |
| --- | --- | --- | --- |
| R1-G0/0/0 |  |  |  |
| R1-G0/0/1 |  |  |  |
| S1 |  |  |  |

# Task 2: Device Configuration (35 marks)

Note: Please provide screenshots with all the following subtasks.

Configure intermediary devices

- Set the router hostname as R1 (2 mark)

- Set switch hostname as S1 (2 mark)

- Set an encrypted privileges EXEC password for all intermediary devices: ccnaenpass (2 marks)

- Secure the console access to all intermediary devices with the password: ccnaconpass (2 marks)

- Set the minimum length for passwords to be 10 characters (2 marks)

- Create an administrative user in the local database and set the username and password as: (2 marks)

  - Username: admin

  - Password: admin1pass

- Configure an MOTD banner for all intermediary devices with the message: No Unauthorised Access! (2 mark)

- Encrypt all the used passwords on all intermediary devices. (2 marks)

- Disable IP domain lookup on all intermediary devices. (1 marks)

- Enable synchronous logging on all intermediary devices to avoid interruptions during configuration tasks. (2 marks)

- Configure router’s interfaces (8 marks)

  - Set description for the interface

  - Assign IPv4 and IPv6 unicast and link-local addresses

- Configure switch’s management (SVI) interface (8 marks)

  - Set description for the interface

  - Assign IPv4 and IPv6 unicast and link-local addresses

# Task 3: Network connectivity (15 marks)

Using the ping command, verify end-to-end connectivity for both IPv4 and IPv6 networks. Please provide screenshots of the results.

| From | To | Protocol | IP Address | Ping Results |
| --- | --- | --- | --- | --- |
| PC-A | R1 G0/0/0 | IPv4 |  |  |
| tftttttttPC-A | R1 G0/0/0 | IPv6 |  |  |
| PC-A | R1 G0/0/1 | IPv4 |  |  |
| PC-A | R1 G0/0/1 | IPv6 |  |  |
| PC-A | S1 VLAN 1 | IPv4 |  |  |
| PC-A | S1 VLAN 1 | IPv6 |  |  |
| PC-A | PC-B | IPv4 |  |  |
| PC-A | PC-B | IPv6 |  |  |
| PC-B | R1 G0/0/0 | IPv4 |  |  |
| PC-B | R1 G0/0/0 | IPv6 |  |  |
| PC-B | R1 G0/0/1 | IPv4 |  |  |
| PC-B | R1 G0/0/1 | IPv6 |  |  |
| PC-B | S1 VLAN1 | IPv4 |  |  |
| PC-B | S1 VLAN1 | IPv6 |  |  |

Submission Requirement

The packet tracer file should be saved as “studentname_id” and uploaded onto Moodle. In

addition to the packet tracer file, you must also submit

• the output of show running-configuration command in a text file for all devices

configured.

• This document with Addressing Scheme and Task 3 completed

All documentation for your solution should be submitted electronically on Moodle.

It is your responsibility that you submit all the outputs and files correctly.