/*****************************************************************************************

PREPROCESSOR DIRECTIVES, USED TO INITIALISED ANY HEADER FILES NEEDED AND DEFINITIONS THAT MAY BE USEFUL.

*****************************************************************************************/

#include <main.h>

#case

#define LCD_ENABLE_PIN PIN_C0

#define LCD_RS_PIN PIN_C2

#define LCD_RW_PIN PIN_C1

#define LCD_DATA4 PIN_C4

#define LCD_DATA5 PIN_C5

#define LCD_DATA6 PIN_C6

#define LCD_DATA7 PIN_C7

#include <lcd.c>

#define Ma PIN_A6

#define Mb PIN_A7

/*****************************************************************************************

* Declares and initialises checkBit, which is used to store the previous function that has been ran, used to clear the screen if needed.

* motorState is the state that the motor is in, it is used to control the motor.

* change is used to indicate whether the function that is running has changed to another, is used in conjunction with checkBit.

* change is used by the motor controller to prevent long button presses causing several incrementations to motorState.

*****************************************************************************************/

unsigned checkBit = 0, motorState = 0, change = 0;

/*****************************************************************************************

* Read channel

* Used to read an ADC channel and return it to caller.

* Inputs: channel, an integer which is used to select which ADC channel will be read.

* Outputs: read_adc(), which returns the value read within the ADC (0-255).

*****************************************************************************************/

int read_channel (int channel){						// Decleration for read_channel function, returns an int and requires an int

set_adc_channel(channel);						// Sets channel ADC will be read from using parsed argument

delay_us(30);									// Delays by 30 microseconds, allows for time to set

return read_adc();								// Returns value from read_adc function, from 0-255

}													// Ends function, the rest will be omitted for my sanity.

/*****************************************************************************************

* Analogues

* Handler for reading all analogue channels then outputting them to the LCD.

* Inputs: None.

* Outputs: None.

*****************************************************************************************/

void analogues (void){								// Declares analogues function, takes and returns no values.

unsigned input;									// Initialises unsigned integer input, which is used to store analogue values

for (unsigned i = 0; i < 3; i++){				// Creates for loop from 0 -> 2, using i

input = read_channel(i);					// Uses read_channel function to retrieve value of ADC port at value i

if (i > 1){									// In this for loop, if i == 2,

lcd_gotoxy(26, 1);						// Sets LCD position where text will be written, used to centre text

printf(lcd_putc, "Read: %3u", input);	// Prints value read at analogue port i

} else{										// Everything else, in this for loop 0 or 1

lcd_gotoxy(6, i+1);						// Go to centre of line 1 or 2

printf(lcd_putc, "Read: %3u", input);	// Print analogue value read in.

}

}

}

/*****************************************************************************************

* LED out

* Outputs all analogue channels to LEDs in form of binary.

* Inputs: None.

* Outputs: None.

*****************************************************************************************/

void led_out (void){									// Declares function ledOut, no inputs, no outputs

unsigned input;									// Declares variable used to store read_channel return value

for (unsigned i = 0; i < 3; i++){				// Inits for loop between 0 -> 2, incrementing

input = read_channel(i);					// Gets output from read_channel, stores in input

output_b(input);							// Outputs to LEDs (this works due to LEDs being able to represent 0-255 and value being 0-255)

delay_ms(3000);								// Delays by 3 seconds, as per specification.

}

}

/*****************************************************************************************

* Motor Driver

* Outputs motor controls depending on value of motorState.

* Inputs: None.

* Outputs: None.

*****************************************************************************************/

void motorDriver (void){							// Declares motorDriver function, no ins, no outs

if (change && input(PIN_A5)){					// If functions not ran consecutively and button is pressed

motorState++;								// Increment motor state global variable

switch (motorState){						// Create switch statement, which will change output depending on state

case 1:									// If 1

output_high(Ma);					// Spin clockwise, turning PIN_A6 (Ma) on

output_low(Mb);						// Turn PIN_A7 (Mb) off

break;								// Ends case, preventing rolling over to next case.

case 2:									// If 2

output_low(Ma);						// Turn motor off, turning PIN_A6 (Ma) off

output_low(Mb);						// Turn PIN_A7 (Mb) off

break;								// Ends case, preventing rolling over to next case.

case 3:									// If 3

output_low(Ma);						// Spin anticlockwise, turning PIN_A6 (Ma) on

output_high(Mb);					// Turn PIN_A7 (Mb) on

break;								// Ends case, preventing rolling over to next case.

case 4:									// If 4

output_low(Ma);						// Turn motor off, turning PIN_A6 (Ma) off

output_low(Mb);						// Turn PIN_A7 (Mb) off

motorState = 0;						// Resets motorState to 0, preventing overflow

break;								// Ends case, preventing rolling over to next case.

}

}

}

/*****************************************************************************************

* Check flag

* Checks whether function has been called concurrently or not then clears screen and sets change.

* Inputs: num, integer which corresponds to function called.

* Outputs: Sets change depending on whether same function has been called concurrently or not.

*****************************************************************************************/

void check_flag (unsigned num){				// Declares check_flag, takes unsigned int num used for function id, returns nothing

if(checkBit != num){					// If switch has changed state (bit in corresponding to state is not set)

//printf(lcd_putc, "\f");				// Clear LCD

output_b(0);						// Turn off all LEDs

checkBit = num;						// Sets new checkbit corresponding to current state

change = 1;							// Sets change if function isn't consecutive

} else {								// Everything else

change = 0;							// Function hasn't changed

}

}

/*****************************************************************************************

* Main

* main function where all others spin off of, contains while to loop forever.

* Inputs: None.

* Outputs: None.

****************************************************************************************/

void main(void){										// Declare main, takes nothing, returns nothing

unsigned input;										// Declares variable input, used to represent input from buttons

setup_adc_ports(sAN0 | sAN1 | sAN2, VSS_VDD);		// Sets ports that will be used for ADC

setup_adc(ADC_CLOCK_INTERNAL | ADC_TAD_MUL_0);		// More ADC setup, uses internal clock and no multiplier

lcd_init();											// Initialises LCD using function

while(TRUE) {										// Infinite loop

input = input_a();								// Input from digital inputs, used for buttons

input &= 0b00110000;							// Masks off bits that aren't used for functions

switch(input){									// Switches which function will run depending on input

case 32:									// If 32 (button 2)

check_flag(32);							// Check whether function call is consecutive // clear if needed

motorDriver();							// Call motorDriver function

break;									// Ends case, prevents rolling over

case 16:									// If case is 16 (button 1)

check_flag(16); 						// Check_flag function to reset if needed

led_out();								// Calls LED_Outs

break;									// Ends case, prevents rolling over

default:									// Catch all case

check_flag(0); 							// Check if concurrent

analogues();							// Call analogues function

break;									// Ends case

}

}

}

Comments

1.	You show conversion values for each potentiometer nicely centred on each line. You have nicely written functions for capturing the values from the potentiometers. 10/10

2.	The value of each ADC is shown correctly on the LEDs for 3 seconds each. You have nicely written functions for doing this. 10/10

3.	You have correctly created a state machine to control the rotation of the motor, although the motor does not rotate because the line goes to a high impedance state instead of going low. I suspect this is due to the way you are reading the buttons - this call sets the port to an input to read it. You correctly test for the button to be released stopping the state machine from changing state unintentionally. 25/30