# Formulae Sheet

| Ohms Law |  | Series Resistors RT=R1+R2+…+Rn | Potential Divider VOut=VinRBRA+RB |
| --- | --- | --- | --- |
|  |  | Parallel Resistors 1RT=1R1+1R2+…+1Rn | Current Divider I=IinRBRA+RB |
| Capacitive Reactance XC=12πfC XC=-j1ωC | Inductive Reactance XL=2πfL XL=jωL | Series Capacitors 1CT=1C1+1C2+…+1Cn | Series Inductors LT=L1+L2+…+Ln |
| Impedance Z=R+jXL-XC Z=R2+XL-XC2 | Total Reactance XT=XL-XC | Parallel Capacitors CT=C1+C2+…+Cn | Parallel Inductors 1LT=1L1+1L2+…+1Ln |
| Angular Frequency ω=2πf | Phase Angle φ=tan-1XTR | Power Factor pf=cosφ | Resonant Frequency fres=12πLC |

| Capacitive Transients Time Constant τ=RC Charging VC=Vs1-e-tτ IC=VRe-tRC Discharging VC=V0e-tτ IC=-VRe-tRC | Inductive Transients Time Constant τ=LR Charging VL=Ve-RtL VL=-Ve-RtL Discharging IL=VR1-e-RtL IL=VRe-RtL | Cut Off Frequency f=12πRC High Pass Circuit Low Pass Circuit Voltage Gain (dB) AV=20logVoutVin Power Gain (dB) AP=10logPoutPin | Q Factor Q=XLR Q=XCR Q=fRfU-fL Q=1RLC |
| --- | --- | --- | --- |
| Class A Common Emitter Transistor Amplifier Design Equations VB =RB2(RB1 + RB2)× VCC VE=VB-0.7 IC=IE VC=VCC-IC×RC Av=vcvb VE=10%VCC RIN=11RB1+1RB2+1hFE×RE ROUT=RC AV≈-RCRE |  |  |  |

Operational Amplifier Design Equations

| Inverting Amplifier Vout=-R2R1×Vin |  |
| --- | --- |
| Non-Inverting Amplifier Vout=1+R2R1×Vin |  |
| Difference (Differential) Amplifier Vout=R2R1×V2-V1 |  |
| Summing Amplifier (Equal) Vout=-R2R1×V1+V2 |  |
| Summing Amplifier (not Equal) Vout=-RR1×V1+RR2×V2+…+RRn×Vn |  |