University of Wales Trinity St David

School of Engineering

Combustion Chemistry Calculations

Powertrain and Sustainability

Malcolm Mcdonald

Phillip Bristow P144394

09/11/2017

Contents

Figures	iii

Tables	iii

1	Mass Percentages	iv

1.1	Molecular Formulas	iv

1.2	Atomic Mass	iv

1.3	Molecular Mass	iv

1.4	Mass Percentage	iv

2	Air:Fuel Mass Ratio	v

2.1	Air Mass	v

2.2	Fuel Mass	v

2.3	Air/Fuel Ratio	v

3	Enthalpy of Formation	vi

4	Calculating Lower Heating Value using Enthalpy of Formation	vii

5	Higher and Lower Heating Values	viii

6	CO2 Mass Produced	ix

7	Ideal Fuel	x

7.1	High Octane Rating	x

7.2	High Laminar Flame Speed	x

7.3	Low Mass	x

7.4	Low Pollution	x

7.5	High Ratio of Specific Heats (ɤ)	x

7.6	Conclusion	xi

8	References	xii

9	Bibliography	xiii

10	Appendix – Periodic Table	xiv

# Figures

Figure 1: Enthalpy of Formation Diagram (Goodger, 2000)	vi

Figure 2: Periodic Table (Science Notes, 2017)	xiv

# Tables

Table 1: Atomic Masses	iii

Table 2:Stoichiometric Fuel Molecular Quantity Calculations	iv

# Mass Percentages

## Molecular Formulas

Ethanol Molecular Formula – C2H5OH

Iso-Octane Molecular Formula – C8H18

(Heywood, 1988)

## Atomic Mass

| Element | Symbol | Atomic Mass |
| --- | --- | --- |
| Hydrogen | H | 1.008 |
| Carbon | C | 12.011 |
| Nitrogen | N | 14.007 |
| Oxygen | O | 15.999 |

Table 1: Atomic Masses

## Molecular Mass

Mass of Ethanol=12.011×2+1.008×6+15.999×1 = 46.069

Mass of Isooctane=12.011×87+1.008×18=114.232

Using a 1:1 molar composition, the total mass is:

Total Mass=160.301

## Mass Percentage

Mass Percentage Ethanol= Ethanol MassTotal Mass=46.069160.301

Mass Percentage Ethanol=0.287=28.7%

Mass Percentage Isooctane= Isooctane MassTotal Mass=114.232160.301

Mass Percentage Isooctane=0.713=71.3%

# Air:Fuel Mass Ratio

Assuming stoichiometric combustion, the quantity of molecules within the fuel blend can be calculated in table 2.

| Reactants | Products |
| --- | --- |
| C8H18+C2H5OH+X(O2+3.76*N2) | a*CO2 + b*H2O + c*N2 |
| Number of Carbons [C] | a = Number C = 10 |
| = 8+2 = 10 |  |
| Number of Hydrogen [H] | 2b = Number H = 24 |
| = 18+5+1 = 24 | b= 12 |
| Number of Oxygens [O] /(X) | Number of O = 2a+b = 2*(10)+12 |
| = 2(X)+1 = 32 | = 32 |
| = 2(X) = 31 |  |
| = X = 31/2 = 15.5 |  |

Table 2:Stoichiometric Fuel Molecular Quantity Calculations

Therefore, the formula for stoichiometric combustion is:

C8H18+C2H5OH+15.5 ×(O2+3.76×N2)→10×CO2+12×H2O+(58.28×N2)

## Air Mass

Air= 15.5 ×(O2+3.76×N2)

Mass of Air=15.5 ×((15.999×2)+3.76×2×14.007

Mass of Air=2128.65

## Fuel Mass

Fuel= C8H18+C2H5OH

Mass of Fuel=12.011×8+1.008×18+12.011×2+1.008×6+15.999×1

Mass of Fuel=160.292

## Air/Fuel Ratio

Air:Fuel Ratio=Mass of AirMass of Fuel

Air:Fuel Ratio= 2128.65160.292

Air:Fuel Ratio=13.3

# Enthalpy of Formation

The Enthalpy of Formation of a compound is the energy, or the increase in enthalpy, needed to form 1 mole of the compound from its base elements at a constant pressure of 1 bar and a constant temperature of 298.15K (Heywood, 1988). This means that a material is in a stable state or energy trough until there is enough energy to raise them out of the trough and to the next level, this is shown in figure 1 (Goodger, 2000). The base elements Oxygen, Carbon, Nitrogen and Hydrogen are assigned an Enthalpy of Formation value of 0.

Figure 1: Enthalpy of Formation Diagram (Goodger, 2000)

# Calculating Lower Heating Value using Enthalpy of Formation

| Reactants | Products |
| --- | --- |
| C8H18+C2H5OH+X(O2+3.76*N2) | 10*CO2 + 12*H2O + 58.28*N2 |
| Enthalpy needed to reach 0 level | Enthalpy released from 0 level |
| C8H18 (Gas) = -224.1 MJ/mol C8H18 (Liquid) = -259.28 MJ/mol | CO2 = -393.5 MJ/mol |
| C2H5OH (Gas) = -234.6 MJ/mol C2H5OH (Liquid) = -277.0 MJ/mol | H2O (Gas) = -241.8 H2O (Liquid) = -285.8 |
| O = 0 | N = 0 |
| N = 0 |  |
| Gas = (-224.1) + (-234.6) Gas = -458.7 MJ/mol | Gas = (10*(-393.5))+(12*(-241.8)) Gas = -6836.6 MJ/mol |
| Liquid = (-259.28) + (-277.0) Liquid = -536.28 MJ/mol | Liquid = (10*(-393.5))+(12*(-285.8)) Liquid = -7364.6 MJ/mol |
| Net Enthalpy Released (Gas) = (-458.7)+(6837.16) Net Enthalpy Released (Gas) = 6377.9 MJ/mol |  |
| Net Enthalpy Released (Liquid) = (-536.28)+(7365.28) Net Enthalpy Released (Liquid) = 6828.32 MJ/mol |  |
| 1kmol Fuel (Gas) releases 6377.9 MJ/mol Heat Enthalpy 1kmol Fuel (Gas) releases = (12.011*8)+(1.008*18)+(12.011*2)+(1.008*6)+(15.999*1) 1kmol Fuel (Gas) = 160.292 Kg 1kg Fuel (Gas) releases = 6377.9/160.292 Mj Lower Heating Value = 39.789 Mj |  |

# Higher and lower heating values

Heating values are used to quantify the amount of heat produced by a fuel during combustion and what happens to the water vapour that is produced. The Higher Heating Value (HHV) also known as the Gross Heating Value, considers the energy produced by the condensation of the water vapour, while the Lower Heating Value (LHV) also known as the Net Heating Value does not consider this. (Green, 2016)

# CO2 Mass produced

1 molecule of C8H18+C2H5OH makes 10 molecules of CO2

10×C+24×HKg Fuel →10 ×C×2×O

∴10×12.011+24×1.008Kg Fuel →10 ×12.011×2×15.999

144.302 kg Fuel →440.09 Kg CO2

∴1Kg Fuel → 440.09144.302 kg →3.049kg CO2

# Ideal Fuel

There are many characteristics of an ideal fuel. A few are listed below;

## High octane rating

The octane rating of a fuel is its resistance to ‘knock’ or autoignition (Exxon Mobil, 2017). The higher the value, the higher the resistance. An ideal fuel would have a high octane rating to allow for an increase in the compression ratio of the engine which would increase the power and efficiency of the engine.

## High laminar flame speed

An ideal fuel would have a high laminar flame speed. The laminar flame speed is the speed that the flame spreads into the region of unburned gas under adiabatic conditions (Colin R Ferguson, 2001). If the fuel has a faster rate of combustion, then there is less time there is for heat transfer/loss and therefore a higher engine efficiency. A faster rate of combustion also lowers the chance of knock.

## Low mass

An ideal fuel should have a low mass. The lighter the particles are, the faster it moves, the faster it moves the more collisions it will have with other particles and therefore promotes a faster combustion.

## Low pollution

Real fuels need to pass the real driving emissions tests such as the Portable Emissions Measuring System (PEMS) tests. These systems are attached to the exhaust and accurately measure the cars emissions. In 2016 the fleet CO2 average was 130 g/Km and by 2020 it is expected to reduce to 95 g/Km (Mahle Powertrain, 2017). An ideal fuel would have no CO2 emissions or other pollutants caused by incomplete combustion. It is possible to largely reduce these emissions by running the engine lean (Heywood, 1988).

## High ratio of specific heat (ɤ)

An ideal fuel would have a high ratio of specific heat, which would improve its efficiency. The ratio of specific heat of a fuel is calculated using the following formula;

ɤ=CpCv

Where;

ɤ = Ratio of Specific Heat

Cp = Constant Pressure

Cv= Constant Volume

Therefore, the ratio of specific heat is higher if the fuel has a low Cv value. By having a higher specific heat ratio, the efficiency of the engine would increase as can be seen in the formula below;

Ƞ=1-1CRɤ-1

Where;

CR = Compression Ratio

ɤ = Ratio of Specific Heats

To prove this, an engine with a compression ratio of 10 would give the following results with 2 different specific heat ratios;

- Using a ratio of specific heat of 1.2

Ƞ=1-1101.2-1 = 0.37 =37% Efficiency

- Using a higher ratio of specific heat of 1.4

- Ƞ=1-1101.4-1 = 0.6 =60% Efficiency

## Conclusion

In conclusion, the ideal fuel would have a high-octane rating allowing higher pressures and an increased compression ratio within the engine. This would allow a faster combustion and therefore less time for heat loss, making the engine more efficient and more powerful. The ideal fuel would be able to have a high/fast combustion rate while the engine runs lean. This would mean that the density within the cylinder would be lower than when running rich, so a light, high energy particle with a high laminar flame speed would promote fast combustion. The fast rates of lean combustion would also mean that less waste products would be produced, making the fuel environmentally friendly.

# References

Class Zone, 2017. ES0501 Reading the Periodic Table. [Online] 
Available at: https://www.classzone.com/books/earth_science/terc/content/investigations/es0501/es0501page06.cfm

Colin R Ferguson, A. T. K., 2001. Internal Combustion Engines. In: Internal Combustion Engines. New York: John Wiley & Sons, Inc, p. 256.

Exxon Mobil, 2017. Octane ratings. [Online] 
Available at: https://www.exxon.com/en/octane-rating

Goodger, E. M., 2000. Enthalpy of Formation. In: Transport Technology: Mobility for the Future. Norwich: Landfall Press, p. 15.

Green, A., 2016. HHV vs LHV - Energy Basics. [Online] 
Available at: http://adgefficiency.com/energy-basics-hhv-versus-lhv/

Heywood, J. B., 1988. Internal Combustion Engine Fundametals. In: Internal Combustion Engine Fundametals. New York: McGraw-Hill Inc, p. 9113.

Mahle Powertrain, 2017. PEMS testing service now available. [Online] 
Available at: http://www.mahle-powertrain.com/en/news-and-press/press-releases/pems-testing-service-now-available-23104

Science Notes, 2017. Printable Periodic Tables. [Online] 
Available at: https://sciencenotes.org/printable-periodic-table/

# Bibliography

Class Zone, 2017. ES0501 Reading the Periodic Table. [Online] 
Available at: https://www.classzone.com/books/earth_science/terc/content/investigations/es0501/es0501page06.cfm

Colin R Ferguson, A. T. K., 2001. Internal Combustion Engines. In: Internal Combustion Engines. New York: John Wiley & Sons, Inc, p. 256.

Exxon Mobil, 2017. Octane ratings. [Online] 
Available at: https://www.exxon.com/en/octane-rating

Goodger, E. M., 2000. Enthalpy of Formation. In: Transport Technology: Mobility for the Future. Norwich: Landfall Press, p. 15.

Green, A., 2016. HHV vs LHV - Energy Basics. [Online] 
Available at: http://adgefficiency.com/energy-basics-hhv-versus-lhv/

Heywood, J. B., 1988. Internal Combustion Engine Fundametals. In: Internal Combustion Engine Fundametals. New York: McGraw-Hill Inc, p. 9113.

Mahle Powertrain, 2017. PEMS testing service now available. [Online] 
Available at: http://www.mahle-powertrain.com/en/news-and-press/press-releases/pems-testing-service-now-available-23104

Science Notes, 2017. Printable Periodic Tables. [Online] 
Available at: https://sciencenotes.org/printable-periodic-table/

# Appendix – Periodic Table

Figure 2: Periodic Table (Science Notes, 2017)