University of Wales Trinity Saint David

School of Engineering

Engine Performance Characteristics                                           Module code:	M4X02196

Bradley Burton

1803066

HNC Motorsport Engineering

18th January 2020

Contents

List of figures	3

1.	    Introduction	4

2.	    Research and Literature Review	5

3.	    Experimental Procedure	6

4.	    Results	8

5.	    Analysis	11

6.	    Discussion	15

7.	    Conclusion	17

8.	    Evaluation	18

9.	    Further work	19

10.	Bibliography / References	20

11.	Appendices	22

# List of figures

Figure 1 	– 	Dynamometer calibration.

Figure 2 	– 	Torque arm specifications.

Figure 3		– 	Torque vs Engine Speed graph.

Figure 4 	– 	Power vs Engine Speed graph.

Figure 5		– 	S.F.C vs Engine Speed graph.

Figure 6		– 	Example of Power and Torque curve, Honda Engine (Team-BHP).

Figure 7 	– 	Brake Thermal Efficiency vs Engine Speed graph.

Table 1 		– 	Dynamometer raw data.

Table 2 		– 	Dynamometer complete data.

# Introduction

This report is the investigation into the performance and characteristics of an internal combustion engine through testing output using an engine dynamometer.

The aims are to describe dynamometers and the procedure used to enable the collection of raw data from a D16 Honda engine.  This data is used to calculate the engines torque, power, specific fuel consumption (S.F.C) and thermal efficiency values which can be presented graphically throughout the engines rev range in order to assess its overall performance.

These results will be analysed and compared to give an understanding of how power outputs differ depending on the test conditions and how the results can be improved through further testing or engine maintenance.

# Research and Literature Review

A dynamometer is a device used to measure the torque required to operate an engine at a driven load.  (T. Martin, 2014, p. 34)  The first system invented by Gaspard de Prony in 1821 and referred to as the ‘Prony Brake’, was used to test the performance of engines and other machinery, however, it wasn’t until the late 1800’s when dynamometers became widely used to measure the torque produced by steam engines.

The oldest type of dynamometer, still in use today, is the ‘water brake system’, which consists simply of two half couplings, a rotor and a stator.  Water brake dynamometers can test engines of all applications from two horsepower lawnmower motors with minimal output to marine engines that produce thousands of horsepower.  (T. Martin, 2014, p. 34)

Power output can be tested by either using an engine or a chassis dynamometer.  Engine units require the powertrain to be removed from the vehicle and mounted to the test rig, whereas, chassis units test the power output at the driving wheels using either rollers or direct hub mounts which reduce the risk of tyre slip producing unreliable data.

Although engine dynamometers are more effective at testing engine output, vehicles loose between ten to twenty five percent of their power through their drivetrain.  This is due to frictional losses in the gearbox, differentials, driveshafts and bearings, therefore, to gain a true depiction of a vehicles overall output a hub mounted chassis dynamometer will give the most accurate result. (X-engineer, 2016)

The two main types of dynamometer currently used are ‘inertia dynos’ and ‘steady state dynos’.  (Dtec, 2012) Inertia dynos use the engine to accelerate an inertial mass where torque is calculated using the mass’s inertial value and the rate of its acceleration.  Steady state, often referred to as ‘brake dynos’ apply either a mechanical or electrical load to the engine whilst running.  Torque is then applied to the brakes housing which is restricted from rotating using an electronic load cell and the force value read by a sensor. (Dtec, 2012)

(B. Bergeron, 2015) Steady state dynamometers can incorporate a rotary transducer that constantly measures the torque of a rotating arm, however, dynamometers without, measure force through a torque arm.  In this case the outer housing is free floating, whereby rotation is prevented by radially mounting the torque arm which incorporates a load cell transducer in its linkage and converts the applied force into a torque signal. (B Bergeron, 2015)

# Experimental Procedure

This reports methodology describes the process of testing a Honda D16 engine using a DSG eddy current dynamometer.  An eddy current dyno is a steady state water cooled electromagnetic break.  Load is controlled by increasing the amp input which in turn increases the electromagnetic field between the dynos coils and rota therefore creating resistance.

The DSG dynamometer can carry out four types of test in order to fully understand an engines output and characteristics.  ‘Step tests’ are used to test from a set engine speed to another whereas ‘manual tests’ enable the controller to apply any load at any RPM using dials for both speed and resistance.  ‘Test sequence’ is utilised to simulate a driving route using a range of engine loads at different speeds.  For the purpose of the experimental procedure the ‘sweep test’ is utilised, which tests the force emitted through the engines whole rev range.

Equipment required:

- Honda D16 engine (pre-installed)

- DSG dynamometer

- DATA Q Pro V2.6 XX computer software

- Secondary torque arm

- 20.45 kg calibration weight and hanger assembly

- Weighing scales (kg)

Firstly, to check calibration, fit an additional torque arm to the dynamometers outer housing and hang the calibration weight from its external mounting point as per Figure 1.

Figure 1. Dynamometer Calibration

Next calculate the load (W) emitted by the hanging mass:

W=Mass× Gravity

20.45 kg ×9.81 m/s²=200.61 N

Figure 2. Torque arm specifications

Using the calculated load value and dynamometers radius (R) (stated on the torque arm as ‘Length’ in Figure 2.) calculate the torque (T) produced by the calibration weight:

T=W R

200.61 N × .815776 m=163.65 Nm

This value is compared with the software readout for load to check the dynamometers calibration.  If incorrect, to minimise errors, use the software’s torque readout to re check the value of the hung mass using the following equation:

Mass= TR × G

166.03 Nm.815776 m × 9.81 m/s²= 20.75 kg

Using the weighing scales measure the calibration weight and hanging assembly to ascertain whether the weights value has been marked incorrectly or if the dynamometer requires calibration.

To begin test, turn on the engine by pressing the ‘start’ button on the control panel and allow the engine to idol until reaching a running temperature of 90°c.  Set the dynamometer software to ‘sweep test’ and select test duration (greater test duration increases quality as more data is collected throughout test period).  To prevent initial bogging at the tests start, set dynamometer to run for a short period above idol before the initiating the ‘sweep test’ and press begin to initiate.

The engine speed will increase to a maximum set engine speed and will decrease slowly until it reaches idol.  The dynamometer supplies the data for ‘load’ (N), ‘engine speed’ (RPM), ‘fuel flow rate’ (l/min) and ‘lamda’ in RPM increments (dependant on test duration) from 1000 RPM at idol to 5500 RPM.  Once complete allow to run at idol momentarily in order to lower the running temperature, after which the engine can be cut off to conclude the test.

Ensure to record the ambient temperature (°c), barometric pressure (mbar) and humidity (%) to enable further calculations.  Also note the fuel type and supplier to enable research of the fuels calorific value.

Lastly, create a spreadsheet and import the raw data values supplied by the dynamometer’s software in order to carry out further calculations and graphically analyse the derived data.

# Results

| Table 1. Dynamometer raw data. |  |  |  |
| --- | --- | --- | --- |
| Load; Dyno Speed; Fuel Flow Rate; Lambda. (Newtons); (RPM); (l/min); (Lambda). 83.49; 1000; 0.0704; 1.269. 85.13; 1050; 0.0724; 1.2682. 87.82; 1100; 0.0737; 1.2674. 90.01; 1150; 0.0728; 1.2667. 92.04; 1200; 0.0744; 1.266. 93.90; 1250; 0.07895; 1.265. 97.03; 1300; 0.079; 1.2633. 100.10; 1350; 0.0796; 1.2614. 102.87; 1400; 0.0835; 1.26. 105.44; 1450; 0.0847; 1.2587. 107.92; 1500; 0.08865; 1.2568. 109.77; 1550; 0.0932; 1.2541. 110.59; 1600; 0.0959; 1.2521. 111.32; 1650; 0.09875; 1.25. 111.89; 1700; 0.10005; 1.2474. 112.41; 1750; 0.10305; 1.2456. 112.81; 1800; 0.1093; 1.2435. 113.42; 1850; 0.11415; 1.2416. 113.95; 1900; 0.11745; 1.2397. 114.65; 1950; 0.12055; 1.238. 115.36; 2000; 0.12555; 1.2361. 116.41; 2050; 0.12785; 1.2339. 117.41; 2100; 0.1309; 1.2288. 118.37; 2150; 0.13435; 1.2227. 118.86; 2200; 0.1377; 1.2161. 119.33; 2250; 0.14105; 1.2095. 119.97; 2300; 0.14495; 1.2032. 120.37; 2350; 0.14985; 1.1995. 120.85; 2400; 0.1537; 1.1961. 121.77; 2450; 0.15565; 1.1929. 122.51; 2500; 0.1596; 1.191. 122.29; 2550; 0.1626; 1.189. 122.06; 2600; 0.16805; 1.1863. 122.02; 2650; 0.17145; 1.1824. 122.13; 2700; 0.17465; 1.1783. 121.84; 2750; 0.1784; 1.1739. 121.81; 2800; 0.18005; 1.1688. 120.84; 2850; 0.1831; 1.1641. 120.51; 2900; 0.1879; 1.159. 118.94; 2950; 0.19255; 1.1541. 117.67; 3000; 0.199; 1.1493. 117.51; 3050; 0.20605; 1.1449. 117.15; 3100; 0.21005; 1.1402. 116.67; 3150; 0.21605; 1.1356. 116.79; 3200; 0.21795; 1.1312. 116.76; 3250; 0.22305; 1.1263. 116.42; 3300; 0.2255; 1.122. 117.35; 3350; 0.2285; 1.118. 117.59; 3400; 0.2342; 1.1141. 118.53; 3450; 0.23605; 1.11. 119.20; 3500; 0.24195; 1.1056. 119.66; 3550; 0.24545; 1.102. 120.26; 3600; 0.25505; 1.0988. 119.91; 3650; 0.25755; 1.0951. 120.13; 3700; 0.26445; 1.0917. 120.18; 3750; 0.2651; 1.0888. 120.11; 3800; 0.2693; 1.0854. 119.78; 3850; 0.27375; 1.0824. 119.18; 3900; 0.2759; 1.0795. 118.53; 3950; 0.28285; 1.0768. 117.87; 4000; 0.28455; 1.0739. 117.14; 4050; 0.2871; 1.0715. 116.54; 4100; 0.28895; 1.0685. 116.64; 4150; 0.29305; 1.0663. 115.85; 4200; 0.2949; 1.0644. 115.54; 4250; 0.29715; 1.0624. 115.03; 4300; 0.3022; 1.0607. 114.38; 4350; 0.30605; 1.0593. 113.81; 4400; 0.3092; 1.0583. 113.46; 4450; 0.3099; 1.0573. 113.20; 4500; 0.31445; 1.0563. 112.37; 4550; 0.3171; 1.0555. 111.14; 4600; 0.3194; 1.0546. 110.13; 4650; 0.32055; 1.0542. 108.90; 4700; 0.3207; 1.0538. 107.67; 4750; 0.32485; 1.0533. 106.29; 4800; 0.32455; 1.0529. 104.86; 4850; 0.3257; 1.0529. 103.66; 4900; 0.3277; 1.0527. 101.90; 4950; 0.32845; 1.0525. 99.84; 5000; 0.3287; 1.0523. 98.22; 5050; 0.32985; 1.0523. 96.56; 5100; 0.3297; 1.0521. 94.47; 5150; 0.33095; 1.0518. 92.52; 5200; 0.3338; 1.0515. 90.36; 5250; 0.3313; 1.0508. 88.54; 5300; 0.3324; 1.0508. 87.60; 5350; 0.3308; 1.0504. 86.40; 5400; 0.3388; 1.05. 85.38; 5450; 0.3438; 1.05. 84.61; 5500; 0.3488; 1.05. |  |  |  |

# Analysis

Using the raw data obtained from engine testing, calculations are made to enable analysis.  As calculations are required for every increment of data, Microsoft Excel is utilised for inputting equations due to the programs ability to quickly calculate rows of data and produce graphs for comparison.

The values required to carry out calculations as explained in the ‘Experimental Procedure’ are as follows:

Radius of the torque arm (R):		.815776 m

Ambient temperature (Ta): 		21 °c

Barometric pressure (Pa): 		1.041 Bar

Humidity:				33 %

Engines bore (d):			0.075 m

Engines stroke (L): 			0.0774 m

Fuel calorific value (C.V):		45800LkJ/kg                                                                                                                           (The fuel calorific value can be researched if the supplier and fuel quality is known, however, can differ slightly batch to batch)

Using the tests raw data and values, calculations are made as follows:

Torque (T) – “The moment of force about a given point is the product of the force and the perpendicular distance of the line of action of the force to that point.” (S.J. Zammit, 1987, p. 270)       (W = Load (N) R = Radius at which loads acts (m))

T=W R

Brake Power (BP) – “The rate of transfer of energy” (S.J. Zammit, 1987, p. 271)                                                               (N = Engine Speed (RPM))

BP= 2πNT60

Correct Power (Ps) – Precise calculation of power, taking ambient pressure and temperature into consideration. (1.01325 = Standard atmospheric pressure)

Ps= 1.01325PaTa293 ×BP

Specific Fuel Consumption (S.F.C) – “Amount of fuel consumed for each unit of power output.” (Collins, 2018) (V = fuel flow velocity (kg/hour), .75 = specific gravity of fuel, convert BP in W to kW by dividing by 1000)

S.F.C= V × .75 × 60(BP1000)

Piston Area - 			A= πd²4

Brake Mean Effective Pressure (B.M.E.P) – Pressure available throughout a stroke to carry out work with frictional losses taken into consideration. (A. Summerfield, 2019) (n = number of cylinders)

B.M.E.P = BP × 60 × 2L × A × N × n

Brake thermal efficiency (ⴄt) – An engines ability to convert fuel into mechanical energy.

ⴄt= (BP1000)S.F.C60 × C.V  × 100

Once all values are calculated and organised using Microsoft Excel (see appendices for full Excel spread sheet), graphs are produced to visually analyse the data.  This enables comparison when carrying out further tests in different conditions or after any changes are made to the engine to help increase its output.

Figure 3. Torque vs Engine Speed graph.

Figure 4. Power vs Engine Speed graph.

Figure 5. S.F.C vs Engine Speed graph.

# Discussion

Figure 3 is a visual representation of the calculated torque against engine speed.  The torque curve rises to its peak of 122 Nm at 2550 RPM, it then dips and rises again to 120 Nm at 3600 RPM before dropping further in a linear manner as engine speed increases until the tests conclusion.

The torque dip occurs, due to the intake valve opening and inlet pressure wave becoming inconsistent with one another between 2550 and 3600 RPM.  The traveling pressure wave should return as the intake valves are opening and be encouraged into the cylinder by the vacuum created by the falling piston.  This torque dip could be resolved with a ‘variable intake manifold’ (VIM) which shortens the runner length at high load and increases its length at low loads to keep the intake valve opening in correlation with the return of the pressure wave.  (Mohiuddin, Rahman, Arfah, Farraen, 2008, P. 185)

The researched torque specifications for the Honda D16 state that peak torque of 152 Nm should occur at 4300 RPM, which shows a difference of 30 Nm.

Figure 4 visually represents power against engine speed.  The linear power curve increases until it reaches a peak of 43.75 kW at 4650 RPM before dropping until the tests end.

Also, the graph compares brake power (BP) and corrected power (Ps) through the rev range.  Ps is a value derived from BP with temperature and pressure considered.  The graph clearly shows the difference between the two values becomes greater as engine speed and therefore heat caused by friction incrementally increases.

The researched Honda D16 specifications state that its peak power should be 81 Kw at 5600 RPM and therefore the test engines power has been decreased by half, however, it is believed the engine originated from Japan whereby it had been restricted to adhere to the countries stringent emission laws.  This could have been achieved by reducing the size of the inlets or adjusting the fuel and air mixture controlled by the ecu as the engine is fuel injected.

Figure 5 illustrates how S.F.C varies with engine speed.  The data shows that the engine is most fuel efficient at 2500 RPM, after which the curve fluctuates until 3000 RPM before efficiency continues to decrease until the end of the test.

Specific fuel consumption starts high due to a loss of heat energy at low speed then decreases as the test engine enters its midrange.  This is due to its good volumetric efficiency between 2300 and 3100 RPM and therefore directly corresponds with the rise in torque curve.  S.F.C then begins to rise due to greater frictional losses and the engines inability to completely fill the cylinder with fresh charge at greater engine speeds.

Another critical variable in S.F.C is the fuels calorific value.  Calorific value is the amount of heat energy released per unit of fuel burnt in combustion, therefore, the lower the calorific value the greater the S.F.C as the engine will require more fuel to output the power required. (S.J Zammit, 1986, P. 285)

Figure 6. Example of Power and Torque curve, Honda Engine (Team-BHP)

Figure 6 shows comparative data from a passed test, using a similar engine showing both torque and power curves laid over one another.  The two curves express many similarities to the test described in this report.  The Power curve rises in a linear motion before falling off at high RPM.  The Torque curve increases until mid-range where a similar dip occurs then it rises to its peak value, which differs from the test engine, before falling toward the end of the test due to reduced efficiency at greater engine speed.

This comparative data proves that internal combustion engine values can differ due to history, capacity or design, however, all are subject to the same limitations and loss of efficacy at high engine speeds.

# Conclusion

To conclude, power increases in a linear manner through the rev range until reaching its maximum point before falling toward the end of the test as the RPM increases. This is mainly as a result of the fall in brake mean effective pressure due to increased frictional losses.

The increase of torque directly correlates with the reduction of S.F.C as the engine reaches its mid-range whereby it is most efficient.  However, when engine speed is increased, the torque value drops and S.F.C rises due to an incremental reduction in brake thermal efficiency as per figure 7.

Figure 7. Brake Thermal Efficiency vs Engine Speed graph.

# Evaluation

This report successfully describes the operation of several dynamometers and the operation to gain data from a Honda D16 test engine, using the DSG eddy current dyno to enable analysis of engine performance throughout its rev range.

Using the raw data derived from the sweep test, formulas have been utilised and explained in order to calculate values for Torque (T), Brake power (BP), Corrected ower (CP), Specific fuel consumption (S.F.C), Brake mean effective pressure (BMEP), and Thermal efficiency (ⴄt) in order to fully understand the engines characteristics.

Then, using the calculated data, graphs have been generated to enable visual analysis of the test engine’s performance.  Using the graphs, investigation into the rise and fall of torque, power and S.F.C has been carried out and an understanding of the running characteristics of the internal combustion engine have been improved.

Finally comparing the test graphs to researched graphs (figure 6) for a similar engine, it is evident that there have been no large anomalies or issues with the test.  This is clear due to the comparative similarities of the torque and power curves for both tests.

# Further work

In order to improve the data, further tests can be carried out on different days to gauge how ambient temperature and pressure will affect the engines performance.  Also, general maintenance such as servicing is likely to have a positive effect on the engines output as there is no clear record of when the oil, filters and spark plugs had last been renewed.

# Bibliography / References

Bergeron, B. (2015) Dynamometer Tech: How They Work.                                                                 Available at: https://www.dynomitedynamometer.com/dyno-dynamometer-article.htm                   (Accessed: 11th December 2019)

Collins Dictionary (2018) Specific fuel consumption. Available at:  https://www.collinsdictionary.com/dictionary/english/specific-fuel-consumption                    (Accessed: 14th December 2019)

Dtec (2012) Inertia & Brake Dyno (Dynamometer) System. Available at: http://dtec.net.au/Inertia%20&%20Brake%20Dyno%20(Dynamometer)%20System%20-%20Kart,%20RC,%20Bike%20&%20Car-%20Engine%20&%20Chassis.htm                                           (Accessed: 11th December 2019)

Martin, T. (2014) ‘Dynamometers Explained’, Dealernews, 50(7), pp. 34-36. Available at: https://whel-primo.hosted.exlibrisgroup.com/permalink/f/mqg008/TN_proquest1545049098

Mohiuddin, A.K.M. Rahman, A. Arfah, A. Farraen, M.A. (2008) ‘Investigation and probable solution of low end torque dip problem of Campro 1.6L Engine’, International Journal of Mechanical and Materials Engineering, 3(2) 176-186. Accessed on: 3rd January 2020, Available at: https://www.researchgate.net/publication/281114610_Investigation_And_Probable_Solution_Of_Low_End_Torque_Dip_Problem_Of_Campro_16_L_Engine

Summerfield, A. (2019) Powertrain lecture handout. Received by: Bradley Burton – HNC Powertrain year 1.  9th September 2019.

X-engineer (2016) Drivetrain losses (efficiency).                                                                                              Available at: https://x-engineer.org/automotive-engineering/drivetrain/transmissions/drivetrain-losses-efficiency/ (Accessed: 10th December 2019)

Zammit, S. J. (1987) Motor Vehicle Engineering Science for Technicians. Essex: Longman Group UK Limited

Table 2. Dynamometer complete data

Appendices