University of Wales Trinity Saint David

### Wales Institute of Science & Art

Low Pass & High Pass Filter Experiment & Analysis

Electrical & Electronic Principles (ACDB4002)

James Levi

2416761

June 2025

# Abstract

The experiment investigates and compares simulations and practical circuits designed to represent the functionality of high pass and low pass filters. This was completed using a breadboard and simulation software called Proteus. The main objective of the experiment was to validate the results by comparing the simulation to the practical experiment, from these results values such at the attenuation and cut off frequencies were to be calculated.

For the high pass filter, attenuation was high at low frequencies and decreased as the frequency increased, with the -3dB point at approximately 1000Hz.

As expected, the low pass filter showed low attenuation at low frequencies, with attenuation increasing to -3dB at approximately 28,000Hz.

The experiment was successful, both filters behaved as expected, and the data supports the validity of the circuits design. However if it were to be conducted again some minor improvements could be made to eliminate and variance in results.

Table of Contents

1	Abstract	1

2	Introduction	3

2.1	Intentions	3

3	Circuit Descriptions	4

3.1	Low Pass Filter	4

3.2	High Pass filter	5

3.3	Circuit Simulations	5

4	Results	7

4.1	Low Pass Filter Results	7

4.2	High Pass Filter Results	9

5	Discussion	12

6	Conclusion	12

7	References	13

Table 1 - Low Pass Filter Results	6

Table 2 - High Pass Filter Results	8

Figure 1 - Low Pass Filter Circuit Diagram	4

Figure 2 - Low Pass Fiter Breadboard Circuit	4

Figure 3 - High Pass Filter Circuit Diagram	5

Figure 4 - High Pass Filter Breadboard Circuit	5

Figure 5 - Proteus High Pass Filter Circuit Diagram	5

Figure 6 - Proteus Low Pass Filter Circuit Diagram	6

Figure 7 - Low Pass Filter Graph	8

Figure 8 - Low Pass Filter Graph (to -3dB)	8

Figure 9 - High Pass Filter Graph	10

Figure 10 - High Pass Filter Graph (to -3dB)	10

# Introduction

High pass and low pass filters are electronic filters that only allow signals with a frequency higher or lower than a certain cutoff frequency to pass through. This cutoff frequency can be calculated using the following equation:

fc= 12πRC

Equation 1 - Cutoff frequency equation

At this frequency, fc, the attenuation (measured in dB) will be -3dB, which, at this point the output voltage is less than 70.7% of the input voltage. Anything outside of this frequency will be attenuated. An everyday example where low pass and high pass filters are used is a stereo system, such as one in your car. A tweeter (treble) speaker blocks low frequency sound the same way a high pass filter works, “a treble speaker connected in series with a capacitor will serve as a high pass filter”. Using the same example, a woofer (bass) speaker is used to produce low frequency sound, blocking out high frequencies, “an inductor connected in series with a bass speaker will serve as a low-pass filter for low frequencies.”

A band-pass filter is another use of high and low pass filters. A band-pass filter consists of both a low and high-pass filter, which work together to allow a desired ‘band’ of frequencies to pass through. They’re seen in everyday life within telecommunications for example, since they work to allow receivers to only hear a selected range of frequencies, also known as having a bandwidth. Since a band-pass filter works to only allow a range of frequencies, rather than any frequency above or below a cutoff frequency, it has two cutoff frequencies, a fcL (lower cutoff) and a fcH (higher cutoff). The bandwidth can be calculated by subtracting the two values, seen below:

BW= fcH-fcL

Equation 2 - Bandwidth Formula

## Intentions

The aim of this report is to analyze the findings from a laboratory exercise, in which both a low pass and high pass filter will be created using a breadboard and in a simulation software, proteus. These filters will be given an input voltage with varying frequency at a constant of 5V peak-peak from a signal generator. Then, using an oscilloscope the output voltage (Vout) will be measured and recorded across the range of frequencies, 100Hz to a maximum of 300kHz. Using the Vout results attenuation can be calculated using the following formula:

A=20×log10VoutVin

Equation 3 - Attenuation equation

The output voltage results are recorded in an excel spreadsheet for analysis, where they are used to calculate attenuation and then plot scatter graphs, which can be used determine the -3dB cut off frequencies. From the graphs it’s easy to visualize the effect frequencies have on output voltage and therefore attenuation.

# Circuit Descriptions

## Low Pass Filter

Figure 1 - Low Pass Filter Circuit Diagram

Using the above circuit diagram (figure 1), a low pass filter circuit was created by using a breadboard in conjunction with a signal generator and then an oscilloscope all in series. The signal generator is used to generate a constant 5V pk-pk input voltage with variable frequency, between 100Hz and 300kHz. The measurements are taken using an oscilloscope by connecting it to the breadboard between the 10kΩ resistor and 560pF capacitor (see figure 2), this measurement will be the output voltage, Vout, which will then be used to calculate attenuation using the attenuation equation (Equation 2) in the excel spreadsheet. The Vout will be measured at specific intervals of frequency, starting from 100Hz up to 300kHz.

Figure 2 - Low Pass Fiter Breadboard Circuit

## High Pass filter

Figure 3 - High Pass Filter Circuit Diagram

To imitate a high pass filter a similar circuit to the low pass filter seen in section 3.1 is set up, using a breadboard, signal generator and oscilloscope. As seen in figure 3 however, the position of the resistor and capacitor are swapped, and the capacitors capacitance is increased to 15nF. This means that the. The Vout is still measured at specific intervals of frequency, starting from 100Hz up to 300kHz, and the Vin is still 5V pk-pk.

Figure 4 - High Pass Filter Breadboard Circuit

## Circuit Simulations

Figure 5 - Proteus High Pass Filter Circuit Diagram

Figure 6 - Proteus Low Pass Filter Circuit Diagram

A simulation of each circuit was created and carried out using proteus, which is a software tool used for electronic design. Using the simulation function within the software, results could be recorded and analyzed using the same method as the laboratory exercise. The circuits contain all the same elements, consisting of a signal generator, resistor, capacitor and oscilloscope (see figures 5,6).

# Results

## Low Pass Filter Results

Table 1 - Low Pass Filter Results

| Low pass Filter |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Simulation |  | Measured |  |
| F (Hz) | Vin(V) | Vout(V) | Simulation Attenuation(dB) | Vout(V) | Measured Attenuation(dB) |
| 100 | 5 | 5 | 0 | 5 | 0 |
| 500 | 5 | 5 | 0 | 5 | 0 |
| 1000 | 5 | 5 | 0 | 4.96 | -0.069766557 |
| 5000 | 5 | 5 | 0 | 4.4 | -1.110346557 |
| 10000 | 5 | 4.9 | -0.175478486 | 4.1 | -1.723722952 |
| 20000 | 5 | 4.7 | -0.537442928 | 3.94 | -2.06947565 |
| 25000 | 5 | 4.1 | -1.723722952 | 3.8 | -2.383728154 |
| 26000 | 5 | 3.7 | -2.615365605 | 3.6 | -2.853350071 |
| 27000 | 5 | 3.6 | -2.853350071 | 3.6 | -2.853350071 |
| 28000 | 5 | 3.54 | -2.999334846 | 3.54 | -2.999334846 |
| 29000 | 5 | 3.5 | -3.0980392 | 3.4 | -3.349821746 |
| 30000 | 5 | 3.46 | -3.197878111 | 3.2 | -3.87640052 |
| 50000 | 5 | 2.46 | -6.160697945 | 2.4 | -6.375175252 |
| 100000 | 5 | 1.35 | -11.37272472 | 1.4 | -11.05683937 |
| 200000 | 5 | 0.7 | -17.07743929 | 0.8 | -15.91760035 |
| 300000 | 5 | 0.45 | -20.91514981 | 0.6 | -18.41637508 |

Figure 7 - Low Pass Filter Graph

Figure 8 - Low Pass Filter Graph (to -3dB)

Table 1 above shows the results from both the practical and simulation investigations. The frequencies highlighted in yellow are of close interest, since at these frequencies the attenuation is closing into its -3dB point, its cutoff frequency. Looking at the results on the table, both the simulation and practical reached its cutoff frequency by 2900Hz. Therefore, it can be observed that fc must lie between 28000Hz and 29000Hz, and the Vout value at this frequency is approximately 3.5V.

The graphs in figures 7 and 8 help to visualize the recorded results more clearly. We can deduct from these that the attenuation generally follows the same trend, however quite a disparity between the frequency values 1000Hz  to 20000Hz becomes quite noticeable. This could be due to the intervals at which readings were taken, since over this 19000Hz range there was only four readings. Regardless, the graph does support the analysis made from Table 1, since the disparity lessens when more frequent readings are taken, and towards -3dB the results are very close, suggesting that fC is at approximately 28000Hz.

## High Pass Filter Results

Table 2 - High Pass Filter Results

| High Pass Filter |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| F (Hz) | Vin | Vout | Simulation Attenuation | Vout | Practical Attenuation |
| 100 | 5 | 0.45 | -20.91514981 | 0.6 | -18.41637508 |
| 200 | 5 | 0.95 | -14.42492798 | 1 | -13.97940009 |
| 300 | 5 | 1.35 | -11.37272472 | 1.6 | -9.897000434 |
| 400 | 5 | 1.75 | -9.118639113 | 2 | -7.958800173 |
| 500 | 5 | 2.16 | -7.290325064 | 2.4 | -6.375175252 |
| 600 | 5 | 2.46 | -6.160697945 | 2.6 | -5.679933127 |
| 700 | 5 | 2.76 | -5.161218445 | 2.8 | -5.03623946 |
| 800 | 5 | 3 | -4.436974992 | 3.2 | -3.87640052 |
| 850 | 5 | 3.16 | -3.985658434 | 3.2 | -3.87640052 |
| 900 | 5 | 3.26 | -3.715048085 | 3.4 | -3.349821746 |
| 950 | 5 | 3.36 | -3.452614539 | 3.6 | -2.853350071 |
| 1000 | 5 | 3.46 | -3.197878111 | 3.6 | -2.853350071 |
| 1050 | 5 | 3.5 | -3.0980392 | 3.6 | -2.853350071 |
| 1100 | 5 | 3.6 | -2.853350071 | 3.8 | -2.383728154 |
| 1150 | 5 | 3.7 | -2.615365605 | 3.8 | -2.383728154 |
| 1200 | 5 | 3.76 | -2.475643188 | 4 | -1.93820026 |
| 1500 | 5 | 4.1 | -1.723722952 | 4.2 | -1.514414279 |
| 2000 | 5 | 4.4 | -1.110346557 | 4.6 | -0.724243453 |
| 3000 | 5 | 4.7 | -0.537442928 | 4.7 | -0.537442928 |
| 5000 | 5 | 4.9 | -0.175478486 | 4.8 | -0.354575339 |
| 10000 | 5 | 4.96 | -0.069766557 | 5 | 0 |
| 50000 | 5 | 5 | 0 | 5 | 0 |
| 100000 | 5 | 5 | 0 | 5 | 0 |
| 200000 | 5 | 5 | 0 | 5 | 0 |
| 300000 | 5 | 5 | 0 | 5 | 0 |

Figure 9 - High Pass Filter Graph

Figure 10 - High Pass Filter Graph (to -3dB)

Table 2 above shows the results from the investigations into the low pass filter. Again, the frequencies of interest that are near the -3dB point, the cutoff frequency, are highlighted yellow. Looking at the results recorded in table 2, the simulation appears to reach the -3dB point at 1050Hz, where the practical reached its -3dB point at 900Hz. This means that the cutoff frequency lies somewhere between those two frequencies, and by taking the median fC=1000Hz it can be observed that the Vout value at -3dB must be lay between 3.46V (Simulation fC=1000Hz) and 3.6V (Practical fC=1000Hz).

The graphs in figures 9 and 10 help to visualize the recorded results more clearly. From these graphs it’s clear that the recorded results are comparable and are both accurate since there’s no notable disparity. The graphs also support the analysis made from Table 2, since the -3dB points lie between 900Hz and 1050Hz.

# Discussion

From the experiments results it’s clear that both the filters operated as expected, and the disparities observed earlier on can be determined to be caused imperfections in the real-world components used combined with variations in measurements collected. In the high pass filter the attenuation is significant at low frequencies, and as the frequency increases the attenuation decreases until it reaches 0, meaning that the signal is passed through without any attenuation. From the median taken in section 4.2 for fC=1000Hz it can be observed that the Vout value at -3dB must be lay between 3.46V (Simulation fC=1000Hz) and 3.6V (Practical fC=1000Hz), this is clear that the data is consistent with the simulation. There are a few small variations in the results, such as at 100Hz there’s a difference between attenuations of around 2.5dB. Since the overall trend of the results are consistent, seen in the graphs (figures 9 and 10), this variation can be disregarded and as previously mentioned it is likely due to factors in the real circuit such as noise (disturbances that interfere with signals) and resistor tolerances (resistors allowable deviation from its actual resistance values).

The low pass filters result also demonstrates the expected behavior of a low pass filter. At low frequencies (100Hz up to 20000Hz) the output voltages are close to the input voltage; therefore the attenuation is low, and the signal is allowed to pass with little disturbance. As the frequency increases the attenuation begins to increase also, it can be observed that fc must lie between 28000Hz and 29000Hz, and the Vout value at this frequency is approximately 3.5V. The Vout also correlates with the -3.5dB , since 5×0.707=3.535V a value very close to the results, further proving that they’re accurate and any discrepancies can be determined to be caused by factors such as noise or component tolerances.

If this experiment was to be completed again there are some improvements to be made. The variations in the data can be easily explained by the inconsistent intervals at which the readings were taken and the likelihood of components having imperfections. The readings were taken at inconsistent frequency intervals primarily as a time saving measure, since taking the closer reading intervals required near the expected cutoff frequencies for the whole experiment would be impractical. However, there could still be readings taken with smaller intervals between a frequency range smaller than 100Hz to 300kHz to improve accuracy. The practical experiments were also completed on different days. This leaves a vulnerability of environmental factors to more likely affect results, as its possible for different components such as a different 10kΩ resistor with different tolerances to be used unintentionally.

# Conclusion

Overall, the experiment was successful. Despite having some anomalous results, the data presented is consistent and has no major discrepancies to suggest it’s unreliable. The high pass filter in this experiment exhibited expected behaviors and had a cutoff frequency of around 1000Hz. The low pass filter also behaved expectedly and had a cutoff frequency of approximately 28000Hz. These cutoff frequencies both corresponded to the Vout values expected, being close to if not equal to 70.7% of Vin. To conclude, the experiment was successful since both filters suppressed signals at unwanted frequencies and the data supports the validity of the circuits design, however if it were to be conducted again some minor improvements could be made.

# References

All about Circuits, Textbook summary of filters [online]

Available at: https://www.allaboutcircuits.com/textbook/alternating-current

[Accessed 04 June 2025]