| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 4 | LEFEL 4 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| April 2020 | EBRILL 2020 |

MODULE CODE: ACCB4001

COMPUTER ARCHITECTURE AND NETWORKS PRACTICAL

TIME:  3 HOURS

Student Name	: ______________________

Student ID		: ______________________

Date			: ______________________

This is an individual practical lab-test that must be completed and uploaded on Moodle within three hours. Any late submission will be capped at 40%. This paper is marked out of 100.

Part 1: IP Addressing (30 marks)

Using the Network address 192.168.10.0/24, VLSM the network to support the following requirements:

Subnet A: 15 hosts

Subnet B: 110 hosts

Subnet C: 50 hosts

Subnet D: 100 hosts

Subnet E: 25 hosts

Subnet F: 6 hosts

Fill Table 1 with IP address details and upload it on to Moodle.

Table 1

| Subnet | Network Address | Subnet mask | First Address | Last Address | Broadcast Address |
| --- | --- | --- | --- | --- | --- |

Part 2 (70 marks):

TOPOLOGY DIAGRAM

Table 2: IP Addressing

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | Gi0/0 | 192.168.1.1 | 255.255.255.0 |  |
|  | Gi0/1 | 192.168.2.1 | 255.255.255.0 |  |
| S1 | VLAN 1 | 192.168.2.20 | 255.255.255.0 | 192.168.2.1 |
| PC1 | NIC | 192.168.1.254 | 255.255.255.0 | 192.168.1.1 |
| PC2 | NIC | 192.168.2.253 | 255.255.255.0 | 192.168.2.1 |

Task 1: Network Creation (5 marks)

Using Cisco Packet Tracer create the topology shown above.

Task 2: Configure the Router (40 Marks)

- Assign a device name R1 to the router.

- Disable DNS lookup to prevent the router from attempting to translate incorrectly entered commands as though they were host names.

- Assign cisco as the console password and enable login.

- Assign cisco as the VTY password and enable login.

- Encrypt the clear text passwords.

- Create a banner that warns anyone accessing the device that unauthorized access is prohibited.

- Configure and activate both interfaces on the router. See Table 1 for IP address details.

- Configure an interface description for each interface indicating which device is connected to it.

- Establish SSH connection.

- Secure the console and VTY lines. Configure the Router to block login attempts for 30 seconds if someone fails two attempts within 120 seconds.

- Assign Enabl3p@ss as the privileged EXEC encrypted password. Set minimum of 10 character to be used for all passwords.

- Save the running configuration to the startup configuration file.

Task 3: Configure the Switch (15 Marks)

- Enter global configuration mode and assign the switch hostname.

- Assign class as the privileged EXEC encrypted password.

- Assign cisco as the console password and enable login.

- Encrypt the clear text passwords.

- Prevent unwanted Domain Name System (DNS) lookups.

- Configure a login message-of-the-day (MOTD) banner.

- Enter global configuration mode to set the SVI (VLAN 1) IP address to allow remote switch management. See Table 1 for IP address details

- Configure IP default gateway.

- Configure the virtual terminal (VTY) line for the switch to allow Telnet access. If you do not configure a VTY password, you will not be able to Telnet to the switch.

- Save the running configuration to the startup configuration file.

Task 4: Test Connectivity (10 Marks)

Task 4.1: Ping

| Task: Ping | Result |
| --- | --- |
| PC1 to PC2 and vice versa |  |
| PC2 to R1 |  |
| PC2 to S1 |  |
| PC1 to S1 |  |
| PC1 to R1 |  |

Task 4.2: Verify that your security measures have been implemented correctly. Use tera term to Telnet to R1.

Does R1 accept Telnet connection? _________________

Task 4.3: SSH to R1. Does R1 accept SSH connections? ________________

Submission Requirement

The packet tracer file should be saved as “studentname_id” and uploaded on Moodle. In addition to packet tracer file, you must also submit

- The output of show running-configuration command in a text file for all devices configured.

- Upload this exam paper with all your answers completed

All documentation for your solution should be submitted electronically on Moodle. It is your responsibility that you submit all the outputs and files correctly.