# Program: BSc Applied Computing

Module: Independent Project / ACCA6020/

Heart Rate Monitoring System – Arduino Source Code

Student Name: Eleonora Petrova

Student ID: 2217638

Lecturer’s Name: Koushik Modak

University of Wales

Trinity Saint David

London Campus

Section 1 — Main Sketch (Cloud_Variables_oct01a.ino)

This is the primary program file where setup(), loop(), sensor reading, smoothing, threshold detection, OLED updates, and cloud sync happen.

/*

Sketch generated by the Arduino IoT Cloud Thing "Untitled"

https://create.arduino.cc/cloud/things/fc255453-3bbd-4c64-bef7-f4e250d5b93a

Arduino IoT Cloud Variables description

The following variables are automatically generated and updated when changes are made to the Thing

int bpm_value_cloud;

bool email_alert_cloud;

Variables which are marked as READ/WRITE in the Cloud Thing will also have functions

which are called when their values are changed from the Dashboard.

These functions are generated with the Thing and added at the end of this sketch.

*/

#include "thingProperties.h"

#include <PulseSensorPlayground.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#include <Wire.h>

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, -1);

//  Variables

const int PulseWire = 0;              // PulseSensor PURPLE WIRE connected to ANALOG PIN 0

const int LED = LED_BUILTIN;          // Built-in LED blinks with heartbeat

int Threshold = 350;                  // Adjust based on Serial Plotter (~baseline + 10–20)

PulseSensorPlayground pulseSensor;

// BPM smoothing buffer

const int BUFFER_SIZE = 5;

int bpmBuffer[BUFFER_SIZE];

int bufferIndex = 0;

// Error detection

const int bpmThreshold = 150;

const unsigned long duration = 10000; // 10 secs

unsigned long lastHighTime = 0;

bool aboveThreshold = false;

//Variables if the sensor does not read data for more than 5 secs

unsigned long lastBeatDetected = 0;

const unsigned long noBeatTimeout = 5000; // 5 seconds

void setup() {

Serial.begin(115200);

// Defined in thingProperties.h

initProperties();

// Connect to Arduino IoT Cloud

ArduinoCloud.begin(ArduinoIoTPreferredConnection);

/*

The following function allows you to obtain more information

related to the state of network and IoT Cloud connection and errors

the higher number the more granular information you’ll get.

The default is 0 (only errors).

Maximum is 4

*/

setDebugMessageLevel(2);

ArduinoCloud.printDebugInfo();

//Pulse sensor setup

pulseSensor.analogInput(PulseWire);

pulseSensor.blinkOnPulse(LED);

pulseSensor.setThreshold(Threshold);

if (pulseSensor.begin()) {

Serial.println("We created a pulseSensor Object!");

}

//Display setup

if (!display.begin(SSD1306_SWITCHCAPVCC, 0x3C)) { // change to 0x3D if needed

Serial.println(F("SSD1306 allocation failed"));

for (;;);

}

//Set default text when display starts

display.clearDisplay();

display.setTextSize(2);

display.setTextColor(SSD1306_WHITE);

display.setCursor(0, 0);

display.println("Hello OLED");

display.display();

}

boolean email_alert = false;

void loop() {

ArduinoCloud.update();

// Your code here

if (pulseSensor.sawStartOfBeat()) {

int myBPM = pulseSensor.getBeatsPerMinute();

lastBeatDetected = millis();

// Put BPM into smoothing buffer

bpmBuffer[bufferIndex] = myBPM;

bufferIndex = (bufferIndex + 1) % BUFFER_SIZE;

// Calculate average BPM

int sum = 0;

for (int i = 0; i < BUFFER_SIZE; i++) {

sum += bpmBuffer[i];

}

int avgBPM = sum / BUFFER_SIZE;

//Set cloud variable "bpm_value_cloud" value

bpm_value_cloud = avgBPM;

//Printing BPM

Serial.print("♥ BPM: ");

Serial.println(avgBPM);

//Display last read value on OLED Display

display.clearDisplay();

display.setTextSize(2);

display.setTextColor(SSD1306_WHITE);

display.setCursor(0, 0);

display.print("BPM: "); display.println(avgBPM);

display.display();

// High BPM error detection

if (avgBPM > bpmThreshold) {

if (!aboveThreshold) {

aboveThreshold = true;

lastHighTime = millis();

} else if (millis() - lastHighTime >= duration ) {

Serial.println("⚠️ ERROR: BPM above 150 for over 10 seconds!");

email_alert_cloud = true;

email_alert = true;

//Display error message on OLED Display

display.clearDisplay();

display.setTextSize(2);

display.setTextColor(SSD1306_WHITE);

display.setCursor(0, 0);

display.println("ERROR: BPM above 150");

display.display();

}

} else {

//Set variable to default values

email_alert_cloud = false;

email_alert = false;

aboveThreshold = false;

}

}

// If no beat detected for >5s, reset BPM to 0

if (millis() - lastBeatDetected > noBeatTimeout) {

if (bpm_value_cloud != 0) {

bpm_value_cloud = 0;

Serial.println("No pulse detected for more than 5 secs.");

display.clearDisplay();

display.setTextSize(2);

display.setTextColor(SSD1306_WHITE);

display.setCursor(0, 0);

display.println("NO PULSE");

display.println("Touch");

display.println("sensor...");

display.display();

}

}

delay(20);

}

/*

Since BpmValueCloud is READ_WRITE variable, onBpmValueCloudChange() is

executed every time a new value is received from IoT Cloud.

*/

void onBpmValueCloudChange()  {

// Add your code here to act upon BpmValueCloud change

}

/*

Since EmailAlertCloud is READ_WRITE variable, onEmailAlertCloudChange() is

executed every time a new value is received from IoT Cloud.

*/

void onEmailAlertCloudChange()  {

// Add your code here to act upon EmailAlertCloud change

}

Section 2 — Cloud Properties File (thingProperties.h)

This file is automatically generated by the Arduino IoT Cloud. It defines the cloud variables, callback links, network configuration, and synchronization settings required for the IoT dashboard and alert system.

#include <ArduinoIoTCloud.h>

#include <Arduino_ConnectionHandler.h>

#include <Arduino_NetworkConfigurator.h>

#include "configuratorAgents/agents/BLEAgent.h"

#include "configuratorAgents/agents/SerialAgent.h"

void onBpmValueCloudChange();

void onEmailAlertCloudChange();

int bpm_value_cloud;

bool email_alert_cloud;

KVStore kvStore;

BLEAgentClass BLEAgent;

SerialAgentClass SerialAgent;

WiFiConnectionHandler ArduinoIoTPreferredConnection;

NetworkConfiguratorClass NetworkConfigurator(ArduinoIoTPreferredConnection);

void initProperties(){

NetworkConfigurator.addAgent(BLEAgent);

NetworkConfigurator.addAgent(SerialAgent);

NetworkConfigurator.setStorage(kvStore);

// For changing the default reset pin uncomment and set your preferred pin. Use DISABLE_PIN for disabling the reset procedure.

//NetworkConfigurator.setReconfigurePin(your_pin);

ArduinoCloud.setConfigurator(NetworkConfigurator);

ArduinoCloud.addProperty(bpm_value_cloud, READWRITE, ON_CHANGE, onBpmValueCloudChange);

ArduinoCloud.addProperty(email_alert_cloud, READWRITE, ON_CHANGE, onEmailAlertCloudChange);

}