ANSWER ALL THE QUESTIONS

Question 1 [25 Marks]

- For the circuit shown in fig1, Calculate the following

- Total resistance in the circuit			[6 Marks]

- Total current in the circuit.			[2 Marks]

- Current across each resistor		[10 Marks]

- The voltage across R7 resistor			[2 Marks]

- Power delivered by the battery			[2 Marks]

Fig: 1

- A solar yard light has a solar collector that can provide 1.0W of power for charging the 3.0V batteries. What is the maximum charging current the solar collector could supply to the fully discharged 3.0V batteries?							[3 Marks]

Question 2 [25 Marks]

- Calculate the current flowing when a charge of 60C flows past a point over a period of 2 minutes.										[2 Marks]

- What length of German silver wire, diameter 0.050 cm, is needed to make a 28 resistor, if the resistivity of German silver is 2.2×10-7  m?				[5 Marks]

- Determine the current across each resistor in the circuit shown in fig.2, using mesh analysis (Kirchhoff’s Voltage Law)	[14 Marks]

Fig.2

- A silicon diode in a circuit has a forward voltage drop of 0.7 V and is in series with a 1 kΩ resistor and a 5 V supply. Calculate the current through the diode.		[4 Marks]

Question 3 [25 Marks]

- Differentiate Step- up Transformer and Step-Down Transformer (minimum of two differences)									[4 Marks]

- Explain the following terms:

- Resistance									[2 Marks]

- Reactance									[2 Marks]

- Impedance									[2 Marks]

- Peak inverse voltage								[2 Marks]

- The alternating current is given by i=40 sin314t A. Determine the following

- Peak Value of current			[1 Mark]

- RMS Value of Current			[2 Marks]

- Frequency			[2 Marks]

- Time period			[1 Mark]

- An oscilloscope display shows the below voltage waveform. The oscilloscope settings are shown in the box next to the waveform.

Oscilloscope settings:

2V/div

10ms/div

Fig:3

- Estimate the period of the waveform.					[1 mark]

- From the estimated period, determine its frequency.			[2 marks]

- Determine its peak-peak amplitude					[2 marks]

- Determine the r.m.s. value of the waveform.				[2 marks]

Question 4 [25 Marks]

- An RLC circuit is shown in figure 4 below. Calculate the following

- The total circuit impedance 						[4 Marks]

- Total current flowing in the circuit.					[2 Marks]

- Determine the voltage drop across each component in the circuits.	[6 Marks]

Fig: 4

- Describe the construction and working of PN Junction diode with the aid of necessary diagrams.									[10 Marks]

- Determine the voltage across each resistor in the circuit shown in fig.5. 	[3 Marks]

Fig.5