| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 5 | LEFEL 5 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| January 2021 | IONAWR 2021 |

MODULE CODE: ACDA5006

Transducers and Signal Conditioning

TIME:  2 HOURS

answer all questions

Equipment or materials required – Computer with Microsoft Excel or similar spreadsheet application

Adnoddau neu ddefnyddiau sydd eu hangen - Cyfrifiadur gyda Microsoft Excel neu gymhwysiad taenlen debyg

ANSWER ALL QUESTIONS

- A humidity transducer is to be characterised.  The voltage measurements given in table Q1a were made using the transducer at 20°C and the measurements given in table Q1b were made using the transducer at 70°C.

Using this data, calculate:

  - sensitivity of the transducer at each temperature,

[6 Marks]

  - the zero offset of the transducer at each temperature,

[6 Marks]

  - maximum non-linearity of the transducer at each temperature

[8 Marks]

  - temperature sensitivity drift coefficient

[5 Marks]

  - temperature zero offset drift coefficient

[5 Marks]

| 20°C | Humidity (RH%) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 |
| Reading 1 | 0.041 | 1.779 | 2.620 | 2.666 | 3.010 | 3.655 | 5.016 | 5.931 | 7.103 |
| Reading 2 | 1.390 | 1.868 | 2.358 | 2.829 | 0.998 | 4.515 | 5.102 | 5.445 | 6.322 |
| Reading 3 | 1.138 | 0.992 | 1.535 | 2.947 | 1.735 | 2.974 | 4.853 | 5.722 | 5.972 |
| Reading 4 | 1.014 | 1.752 | 2.290 | 3.094 | 3.391 | 3.246 | 5.157 | 5.574 | 6.172 |
| Reading 5 | 1.108 | 1.137 | 2.835 | 3.660 | 2.320 | 3.801 | 5.055 | 6.491 | 6.972 |
| Reading 6 | 1.120 | 0.360 | 2.783 | 2.271 | 2.708 | 4.876 | 4.937 | 5.412 | 6.420 |
| Reading 7 | -0.149 | 0.468 | 1.913 | 3.675 | 2.863 | 4.896 | 3.793 | 5.909 | 4.775 |
| Reading 8 | -0.673 | 1.957 | 1.414 | 3.344 | 3.609 | 4.444 | 4.832 | 6.762 | 6.388 |
| Reading 9 | 1.778 | 2.178 | 1.945 | 3.349 | 2.718 | 5.081 | 5.978 | 5.094 | 5.900 |
| Reading 10 | 1.856 | 2.144 | 1.741 | 2.933 | 2.250 | 2.898 | 3.404 | 4.804 | 6.796 |
| Reading 11 | 0.530 | 1.003 | 1.995 | 2.499 | 2.981 | 5.581 | 4.751 | 6.024 | 6.980 |
| Reading 12 | 1.544 | 1.113 | 1.765 | 2.629 | 1.416 | 4.974 | 4.156 | 5.889 | 6.092 |
| Reading 13 | 0.699 | 1.385 | 3.628 | 1.485 | 2.095 | 5.287 | 4.618 | 6.258 | 4.954 |
| Reading 14 | 0.528 | 1.632 | 1.747 | 1.592 | 2.541 | 4.264 | 4.601 | 4.690 | 6.399 |
| Reading 15 | -0.149 | 2.209 | 1.776 | 2.723 | 1.693 | 3.812 | 4.217 | 6.024 | 7.153 |
| Reading 16 | 0.576 | 2.380 | 2.526 | 2.846 | 3.618 | 4.514 | 4.999 | 5.446 | 6.726 |
| Reading 17 | 1.615 | 1.124 | 1.639 | 2.281 | 2.898 | 4.314 | 5.749 | 5.996 | 6.667 |
| Reading 18 | 1.366 | 0.789 | 1.682 | 3.029 | 2.970 | 4.702 | 4.672 | 5.531 | 5.420 |
| Reading 19 | 0.913 | 1.745 | 2.546 | 1.210 | 2.431 | 6.398 | 5.149 | 5.274 | 6.919 |
| Reading 20 | 1.010 | 1.487 | 2.874 | 2.729 | 2.909 | 3.959 | 5.112 | 5.907 | 5.987 |

Table Q1a

| 70°C | Humidity (RH%) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 |
| Reading 1 | 1.300 | 2.448 | 3.856 | 3.760 | 5.377 | 5.855 | 7.865 | 8.675 | 9.425 |
| Reading 2 | 1.425 | 2.758 | 2.950 | 4.684 | 5.641 | 5.335 | 8.219 | 7.305 | 10.335 |
| Reading 3 | 0.702 | 2.882 | 2.643 | 4.841 | 6.260 | 5.716 | 8.992 | 8.265 | 8.293 |
| Reading 4 | 2.357 | 1.073 | 3.754 | 3.493 | 4.619 | 6.207 | 7.400 | 9.310 | 9.306 |
| Reading 5 | 2.000 | 1.596 | 3.381 | 5.123 | 5.488 | 6.572 | 7.981 | 8.024 | 8.883 |
| Reading 6 | 0.765 | 2.441 | 3.202 | 3.795 | 6.340 | 6.954 | 8.389 | 9.113 | 8.550 |
| Reading 7 | 1.224 | 1.756 | 2.406 | 3.112 | 5.111 | 6.198 | 8.355 | 7.998 | 9.273 |
| Reading 8 | 1.579 | 2.452 | 1.653 | 5.754 | 5.250 | 6.785 | 9.192 | 8.315 | 9.805 |
| Reading 9 | 2.586 | 2.671 | 3.052 | 4.234 | 5.543 | 6.893 | 7.722 | 7.190 | 9.363 |
| Reading 10 | 1.119 | 2.921 | 5.528 | 4.256 | 5.785 | 5.818 | 7.033 | 7.175 | 9.729 |
| Reading 11 | 2.372 | 2.480 | 4.710 | 3.826 | 4.093 | 5.931 | 6.842 | 8.237 | 8.990 |
| Reading 12 | 2.440 | 1.713 | 2.492 | 4.777 | 5.247 | 5.966 | 7.905 | 9.592 | 7.787 |
| Reading 13 | 1.818 | 2.116 | 3.384 | 3.852 | 4.701 | 6.306 | 7.362 | 8.403 | 9.179 |
| Reading 14 | 2.411 | 2.056 | 4.412 | 3.995 | 5.744 | 7.030 | 7.176 | 8.449 | 9.239 |
| Reading 15 | 1.538 | 1.338 | 3.819 | 4.413 | 5.376 | 6.100 | 8.436 | 7.716 | 10.247 |
| Reading 16 | 2.023 | 2.670 | 2.457 | 3.785 | 6.462 | 7.243 | 8.245 | 8.950 | 8.004 |
| Reading 17 | 2.502 | 2.199 | 2.629 | 4.908 | 5.557 | 5.470 | 7.462 | 8.553 | 8.221 |
| Reading 18 | 2.072 | 3.728 | 3.458 | 3.964 | 4.014 | 6.514 | 9.007 | 7.678 | 9.363 |
| Reading 19 | 2.030 | 2.212 | 4.119 | 4.746 | 5.207 | 7.000 | 7.909 | 8.923 | 10.597 |
| Reading 20 | 1.625 | 2.209 | 3.579 | 3.505 | 5.092 | 4.973 | 8.223 | 8.216 | 10.211 |

Table Q1b

- Compare and contrast the requirements and specifications for a temperature sensor for the following applications:

  - Air temperature measurement for a weather station

  - Coolant temperature of a car engine.

[10 Marks]

- Explain what methods could be used to reduce noise when transmitting transducer signals using wires in a noisy environment.  Give an example circuit and explain how it works.

[20 Marks]

- Explain how inductive and capacitive proximity sensors work, include a block diagram for each in your answer to illustrate their functionality. Give examples of applications where each could be used.

[20 Marks]

- Explain the need for and the process of calibration.

[20 Marks]

Diwedd/End