HEAT TRANSFER

CONVECTION

Extra exercise:

An electrically heated thin foil of length L = 25 mm and width W = 8 mm is to be used as a wind speed metre. Wind with a temperature Tf and velocity uf blows parallel to the longest side. The foil is internally heated by an electric heater dissipating Q (Watts) from both sides and is to be operated in air with:

Tf=20℃,    cp=1.005kJkg.K,   =1.522×10-5 m2s,     ρ=1.19 kgm3 and   Pr=0.72.

The surface temperature Ts of the foil is to be measured at the trailing edge – but can be assumed to be constant. Estimate the wind speed when Ts=32℃ and Q=0.5 W.