| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 4 | LEFEL 4 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| January 2024 | 2024 |

MODULE CODE: ACDA4006

Electrical and Electronic Principles

TIME:  2 HOURS

Maximum Marks : 100

answer All the questions

Equipment or materials required – Calculator, Formula sheet, Graph sheet,

Adnoddau neu ddefnyddiau sydd eu hangen - Cyfrifiannell

ANSWER ALL THE QUESTIONS

Question 1 [25 Marks]

- For the circuit shown in fig1, Calculate the following

- Total resistance in the circuit			[8 Marks]

- Total current in the circuit.			[3 Marks]

- Current across each resistor		[12 Marks]

- the voltage across R7 resistor			[2 Marks]

Fig: 1

Question 2 [25 Marks]

- Determine the current across each resistor in the circuit shown in fig.2, using mesh analysis (Kirchhoff’s Voltage Law)	[17 Marks]

Fig.2

- Three resistors (RA RB and Rc)are connected in series to a 200V Voltage Source. Where, Rc=25Ω, VB=50V and Total current = 3A. Calculate the following

- Voltage across RA 							[2 Marks]

- Voltage across RC 							[2 Marks]

- Resistance value of RA and RB						[4 Marks]

Question 3 [25 Marks]

- A power supply requires 80W of input power and produces an output power of 75W. Calculate its efficiency in percentage and the total power loss.		[5 Marks]

- State Thevenin’s Theorem with the aid of Thevenin’s equivalent circuit.	[5 Marks]

- The alternating current is given by i=100 sin314t. Determine the following

- Maximum Value of current			[1 Mark]

- RMS Value of Current			[2 Marks]

- Frequency			[2 Marks]

- In the RL circuit shown in fig 3. Calculate the following.

- Inductive Reactance (XL)		[3 Marks]

- Impedance (Z)		[3 Marks]

- Current across the circuit.		[2 Marks]

- Phase angle 		[2 Marks]

Fig. 3

Question 4 [25 Marks]

- Define the following terms

- Reactance									[2 Marks]

- Impedance								[2 Marks]

- Peak inverse Voltage							[2 Marks]

- A sine wave is observed on a oscilloscope as shown in fig 4. The time setting is 10ms/div and voltage is 2V / div. Estimate the following.

- Peak Voltage								[1 Mark]

- Peak to peak voltage							[2 Marks]

- Time period of the waveform						[2 Marks]

- Frequency									[2 Marks]

Fig.4

- A transformer steps up voltage from 110 V to 440 V. If the primary coil has 50 turns, how many turns does the secondary coil have?				[4 Marks]

- List any two applications of Transistor.					[2 Marks]

- What happens when a PN junction diode is forward biased?			[6 Marks]

Diwedd/End