Zsolt Varkonyi, BEng Motorcycle Engineering, P142563

Malcolm McDonald, Powertrain & Sustainability, M3X8824

02 OCT 2017

12

POWERTRAIN & SUSTAINABILITY
LEVEL 6

COMBUSTION CHEMISTRY CALCULATIONS

TABLE OF CONTENTS

1	Introduction	2

2	Calculations	2

(A)	Mass Percentages	2

(B)	Air-Fuel Mass Ratio	3

(C)	Calculation of Lower Heating Value	4

(D)	Enthalpy of Formation	5

(E)	Difference Between Higher and Lower Heating Values	5

(F)	Mass of CO2 Produced	5

(G)	Properties of an “Ideal” Fuel	6

3	Appendix	7

4	Bibliography	8

# Introduction

### Molar composition of blended bio-ethanol

Ethanol/Ethyl alcohol (National Center for Biotechnology Information, 2017)

Molecular Formula:	C2H6O

Molecular Weight:	46.069gmol

Iso-Octane/2,2,4-Trimethylpentane/Gasoline (National Center for Biotechnology Information, 2017)

Molecular Formula:	C8H18

Molecular Weight:	114.232gmol

Molar Composition of Blend		50%-50%

Atomic Weight of Components (Cayman Chemical, 2017)

Carbon, C		12.01078gmol

Hydrogen, H		  1.007947gmol

Oxygen, O		15.99943gmol

Nitrogen, N		14.00672gmol

# Calculations

## Mass Percentages

Molar weight of blended bio-ethanol:

0.5×46.0687gmol+0.5×114.2293gmol	=80.1490gmol	(Eq. A.1)

Mass percentages of components in blend:

50% ethanol in blend weights 	23.0345gmol	=28.74 %

50% iso-octane in blend weights 	57.116gmol	=71.26 %

Mass percentages of elements in blend:

Carbon 	5×12.011gmol	=60.0539gmol	=74.93 %

Hydrogen 	12×1.0079gmol	=  12.0954gmol	=15.09 %

Oxygen 	0.5×15.9994gmol	=  7.9997gmol	=  9.98 %

## Air-Fuel Mass Ratio

### Stochiometric Combustion

Equation of stoichiometry of combustion: (Kirk, 2018)

CnHm+n+m4O2+3.76N2→nCO2+m2H2O+3.76n+m4N2 	(Eq. B.1)

Stochiometric reaction of ethanol:

C2H6O+3.5O2+3.76 N2→2 CO2+3 H2O+13.16 N2+0.5O2	(Eq. B.2)

Stochiometric reaction of gasoline:

C8H18+12.5O2+3.76 N2→8 CO2+9 H2O+47 N2		(Eq. B.3)

Stochiometric reaction of the blend:

0.5 C2H6O+0.5 C8H18+0.5×3.5+0.5×12.5O2+3.76 N2

→0.5×2+0.5×8CO2+0.5×3+0.5×9 H2O

+0.5×3.5+0.5×12.5×3.76 N2+(0.5×0.5) O2	(Eq. B.4)

0.5 C2H6O+0.5 C8H18+8 O2+3.76 N2

→5 CO2+6 H2O+30.08 N2+0.25 O2		(Eq. B.4)

Ethanol can be considered as a partially oxidized hydrocarbon, because it contains an oxygen atom in its basic form. When a partially oxidized hydrocarbon is taking part in the combustion process, it provides more oxygen (see (Eq. B.2) and (Eq. B.4)) and leads to the so-called “leaning effect” and achieves better combustion. (Sadiq, Ali , & Noor, 2011)

### Mass Calculations

Molar weight of blend from previous calculation (Eq. A.1)

0.5 C2H6O+0.5 C8H18			=80.1490 g	(Eq. B.5)

Total weight of air needed for stochiometric combustion of 1 molecule of blend:

8 O2+3.76 N2	=8×2×15.9943g+3.76×2×14.0067g=1098.6352 g	(Eq. B.6)

Total weight of CO2 produced by burning 1 mol of blend:

5 CO2	=5×12.011g+2×15.9943g	=220.0482 g	(Eq. B.7)

Total weight of H2O produced by burning 1 mol of blend:

6 H2O	=6×2×1.008g+15.9943g	=108.0919 g	(Eq. B.8)

Total weight of N2 produced by burning 1 mol of blend:

30.08 N2	=30.08×2×14.0067g	=842.6443 g	(Eq. B.9)

Total weight of O2 produced by burning 1 mol of blend:

0.25 O2	=0.25×2×15.9943g	=7.9997 g	(Eq. B.10)

### Air-Fuel Ratio

Using previous calculations (Eq. B.5) and (Eq. B.6):

AF=1098.6352g80.1490g=13.7074	(Eq. B.11)

FA=80.1490g1098.6352g=0.0730	(Eq. B.12)

## Calculation of Lower Heating Value

Below calculations are based on (Eq. B.4).

0.5 C2H6O+0.5 C8H18+8 O2+3.76 N2→5 CO2+6 H2O+30.08 N2+0.25 O2

Assuming, that fuel is vaporized in the cylinder.

Enthalpy required to get reactions to zero level for 1 kmol blend:

0.5×-234.6 MJkmol+0.5×-224.1MJkmol	=-229.35 MJ	(Eq. C.1)

Enthalpy of 1 kmol blend released from zero level:

5×-393.5 MJkmol+6×-241.8MJkmol+0MJkmol+0MJkmol	=-3418.30 MJ	(Eq. C.2)

Net enthalpy of 1 kmol blend:

3418.30 MJ-229.35 MJ		=3188.95 MJ	(Eq. C.3)

Lower Heating Value of 1 kg blend, using value of previous calculation 0.5 C2H6O+0.5 C8H18			=80.1490 g	(Eq. B.5):

3188.95 MJ80.149 kg			=39.7878 MJkg	(Eq. C.4)

## Enthalpy of Formation

Enthalpy of formation of a compound is the enthalpy required to make 1 kmol of compound from its constituent building blocks (C (graphite), H2, N2 and O2) at a constant pressure of 1 bar and a constant temperature of 298.15°K. It is an activation energy which does occur when appropriate number of molecules are dividing from their stable structure into unstable atoms, then energy is released when these atoms are forming new molecules, which are the products of combustion (CO2, H2O)

## Difference Between Higher and Lower Heating Values

The heating value of a fuel is the amount of heat released during the combustion of a specified amount of it.

The higher heating value (HHV) is determined by bringing all the products of combustion back to the original pre-combustion temperature, and in particular condensing any vapor produced.

The lower heating value (LHV) is the net heat released per kg of a fuel and the products are permitted to escape and when the product of water is in the form of gas.

HHV = LHV + Latent heat of vaporization of water vapor. (THI LUONG DINH & VU NGUYEN HOANG, 2014)

## Mass of CO2 Produced

The stochiometric condition of the blend (Eq. B.4) shows, that

1 molecule of fuel makes 10 molecules of CO2.

Using previous calculations, (Eq. B.5) and (Eq. B.7), it means, that

1 kmol fuel is 80.149 kg and is producing 220.0482 kg CO2.

In other words,

1 kg fuel is producing 2.7455 kg CO2.

## Properties of an “Ideal” Fuel

Characteristics of a liquid fuel, marked as “ideal”, should fulfil the following:

- High calorific value.

- Moderate ignition temperature.

- High octane rating to resist self-ignition.

- High compression ratio to make the engine more powerful and economical.

- High laminar flame speed to make the power more useable.

- High flashpoint to make transportation and storage safer.

- Cleaner/Better burning to reduce air pollution and decrease HC and CO emissions.

- Non-poisonous or residue by products on combustion.

- Other important features in the name of sustainability:

  - Availability is easy, at low cost, and in plenty.

  - Transporting and storage is easy and convenient.

# Appendix

All the calculations included in this assignment are calculated in Excel to ensure their correctness. This is the calculator used:

# Bibliography

Cayman Chemical. (2017, 11 18). Molar Mass and Mass Percent Composition Calculator. Retrieved from Molar Mass Calculator Tool: https://www.caymanchem.com/chemAssistant/Tool/4002

CONCAWE Fuels Quality and Emissions Management Group. (2006). Assessment of the impact of ethanol content in gasoline on fuel consumption, including a literature review up to 2006. Brussels.

Flagan, R. C., & Seinfeld, J. H. (1988). Combustion Fundametnals. In C. I. Technology, Fundamentals of Air Pollution Engineering (pp. 59-166). New Jersey: Prentice-Hall Inc.

Kirk, D. R. (2018, 01 01). Advanced Propulsion - Introduction and Chemical Equilibrium. Retrieved 11 18, 2017, from Slideplayer.com: http://slideplayer.com/slide/5284369/

National Center for Biotechnology Information. (2017, 11 18). 2,2,4-Trimethylpentane. Retrieved from PubChem Compound Database: https://pubchem.ncbi.nlm.nih.gov/compound/10907

National Center for Biotechnology Information. (2017, 11 18). Carbon. Retrieved from PubChem Compound Database: https://pubchem.ncbi.nlm.nih.gov/compound/5462310

National Center for Biotechnology Information. (2017, 11 18). Ethanol. Retrieved from PubChem Compound Database: https://pubchem.ncbi.nlm.nih.gov/compound/702

National Center for Biotechnology Information. (2017, 11 18). Home. Retrieved from PubChem Compound Database: https://www.ncbi.nlm.nih.gov/pccompound

National Center for Biotechnology Information. (2017, 11 18). Hydrogen. Retrieved from PubChem Compound Database: https://pubchem.ncbi.nlm.nih.gov/compound/783

National Center for Biotechnology Information. (2017, 11 18). Oxygen. Retrieved from PubChem Compound Database: https://pubchem.ncbi.nlm.nih.gov/compound/977

Sadiq, M. A., Ali , Y. K., & Noor, A. R. (2011). Effects of Ethanol-Gasoline Blends on Exhaust and Noise Emissions from 4-Stroke S.I.Engine. Eng. & Tech. Journal, 1438-1450.

THI LUONG DINH, & VU NGUYEN HOANG. (2014). Determination of C/H/O fractions and lower heating values for diesel-biodiesel blends derived from Vietnam. International Journal of Renewable Energy and Environmental Engineering.