Lab - Accessing Network Devices with SSH

Topology

Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/1 | 192.168.1.1 | 255.255.255.0 | N/A |
| S1 | VLAN 1 | 192.168.1.11 | 255.255.255.0 | 192.168.1.1 |
| PC-A | NIC | 192.168.1.3 | 255.255.255.0 | 192.168.1.1 |

Objectives

Part 1: Configure Basic Device Settings

Part 2: Configure Basic Security Measures on the Router and Switch

Part 3: Configure the Router for SSH Access

Part 4: Configure the Switch for SSH Access

Part 5: SSH from the CLI on the Switch

Background / Scenario

In the past, Telnet was the most common network protocol used to remotely configure network devices. Telnet does not encrypt the information between the client and server. This allows a network sniffer to intercept passwords and configuration information.

Secure Shell (SSH) is a network protocol that establishes a secure terminal emulation connection to a router or other networking device. SSH encrypts all information that passes over the network link and provides authentication of the remote computer. SSH is rapidly replacing Telnet as the remote login tool of choice for network professionals. SSH is most often used to log in to a remote device and execute commands; however, it can also transfer files using the associated Secure FTP (SFTP) or Secure Copy (SCP) protocols.

The network devices that are communicating must be configured to support SSH in order for SSH to function. In this lab, you will enable the SSH server on a router and then connect to that router using a PC with an SSH client installed. On a local network, the connection is normally made using Ethernet and IP.

Note: The routers used with CCNA hands-on labs are Cisco 1941 Integrated Services Routers (ISRs) with Cisco IOS Release 15.2(4)M3 (universalk9 image). The switches used are Cisco Catalyst 2960s with Cisco IOS Release 15.0(2) (lanbasek9 image). Other routers, switches, and Cisco IOS versions can be used. Depending on the model and Cisco IOS version, the commands available and the output produced might vary from what is shown in the labs. Refer to the Router Interface Summary Table at the end of this lab for the correct interface identifiers.

Note: Make sure that the routers and switches have been erased and have no startup configurations. If you are unsure, contact your instructor.

Required Resources

1 Router (Cisco 1941 with Cisco IOS Release 15.2(4)M3 universal image or comparable)

1 Switch (Cisco 2960 with Cisco IOS Release 15.0(2) lanbasek9 image or comparable)

1 PC (Windows 7 or 8 with terminal emulation program, such as Tera Term, and Wireshark installed)

Console cables to configure the Cisco IOS devices via the console ports

Ethernet cables as shown in the topology

Configure Basic Device Settings

In Part 1, you will set up the network topology and configure basic settings, such as the interface IP addresses, device access, and passwords on the router.

Cable the network as shown in the topology.

  - Initialize and reload the router and switch.

  - Configure the router.

    - Console into the router and enable privileged EXEC mode.

    - Enter configuration mode.

    - Disable DNS lookup to prevent the router from attempting to translate incorrectly entered commands as though they were host names.

    - Assign class as the privileged EXEC encrypted password.

    - Assign cisco as the console password and enable login.

    - Assign cisco as the VTY password and enable login.

    - Encrypt the plaintext passwords.

    - Create a banner that will warn anyone accessing the device that unauthorized access is prohibited.

    - Configure and activate the G0/1 interface on the router using the information contained in the Addressing Table.

    - Save the running configuration to the startup configuration file.

Configure PC-A.

Configure PC-A with an IP address and subnet mask.

Configure a default gateway for PC-A.

Verify network connectivity.

Ping R1 from PC-A. If the ping fails, troubleshoot the connection.

- Configure Basic Security Measures on the Router

  - Encrypt the clear text passwords.

R1(config)# service password-encryption

  - Strengthen passwords.

An administrator should ensure that passwords meet the standard guidelines for strong passwords. These guidelines could include combining letters, numbers and special characters in the password and setting a minimum length.

Note: Best practice guidelines require the use of strong passwords, such as those shown here, in a production environment. However, the other labs in this course use the cisco and class passwords for ease in performing the labs.

    - Change the privileged EXEC encrypted password to meet guidelines.

R1(config)# enable secret Enablep@55

    - Require that a minimum of 10 characters be used for all passwords.

R1(config)# security passwords min-length 10

  - Secure the console and VTY lines.

    - You can set the router to log out of a connection that has been idle for a specified time. If a network administrator was logged into a networking device and was suddenly called away, this command automatically logs the user out after the specified time. The following commands cause the line to log out after five minutes of inactivity.

R1(config)# line console 0

R1(config-line)# exec-timeout 5 0

R1(config-line)# line vty 0 4

R1(config-line)# exec-timeout 5 0

R1(config-line)# exit

R1(config)#

    - The following command impedes brute force login attempts. The router blocks login attempts for 30 seconds if someone fails two attempts within 120 seconds. This timer is set especially low for the purpose of this lab.

R1(config)# login block-for 30 attempts 2 within 120

What does the 2 within 120 mean in the above command?

If the user fails to login more than one time (2 in this case), the user would then be stopped and blocked form attempting to log in for 120 seconds (aka 2 minutes).

What does the block-for 30 mean in the above command?

This command is used for blocking users to gain access to the router. The router will wait 30 seconds in this example and once those 30 seconds are up the user can attempt to log in again.

  - Verify that all unused ports are disabled.

Router ports are disabled by default, but it is always prudent to verify that all unused ports are in an administratively down state. This can be quickly checked by issuing the show ip interface brief command. Any unused ports that are not in an administratively down state should be disabled using the shutdown command in interface configuration mode.

R1# show ip interface brief

Interface                  IP-Address      OK? Method Status                Protocol

Embedded-Service-Engine0/0 unassigned      YES NVRAM  administratively down down

GigabitEthernet0/0         unassigned      YES NVRAM  administratively down down

GigabitEthernet0/1         192.168.1.1     YES manual up                    up

Serial0/0/0                unassigned      YES NVRAM  administratively down down

Serial0/0/1                unassigned      YES NVRAM  administratively down down

Configure the Router for SSH Access

Using Telnet to connect to a network device is a security risk because all information is transmitted in a clear text format. SSH encrypts the session data and provides device authentication, which is why SSH is recommended for remote connections. In Part 2, you will configure the router to accept SSH connections over the VTY lines.

Configure device authentication.

The device name and domain are used as part of the crypto key when it is generated. Therefore, these names must be entered prior to issuing the crypto key command.

Configure the domain for the device.

R1(config)# ip domain-name ccna-lab.com

Configure the encryption key method.

R1(config)# crypto key generate rsa modulus 1024

The name for the keys will be: R1.ccna-lab.com

% The key modulus size is 1024 bits

% Generating 1024 bit RSA keys, keys will be non-exportable...

[OK] (elapsed time was 1 seconds)

R1(config)#

*Jan 28 21:09:29.867: %SSH-5-ENABLED: SSH 1.99 has been enabled

Configure a local database username.

R1(config)# username admin privilege 15 secret adminpass

Note: A privilege level of 15 gives the user administrator rights.

Enable SSH on the VTY lines.

Enable Telnet and SSH on the inbound VTY lines using the transport input command.

R1(config)# line vty 0 4

R1(config-line)# transport input telnet ssh

Change the login method to use the local database for user verification.

R1(config-line)# login local

R1(config-line)# end

R1#

Save the running configuration to the startup configuration file.

R1# copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

R1#

Establish an SSH connection to the router.

Start Tera Term from PC-A.

Use Tera Term to telnet to R1.

Does R1 accept the Telnet connection? Explain.

My router did not accept the telnet connection. This is because the router was disabled with an SSH command. Once I enabled the command, the router could then connect.

Use Tera Term, establish an SSH session to R1. Use the username admin and password adminpass. You should be able to establish an SSH session with R1.

Does R1 accept the SSH connection? Yes it does.

Intentionally mistype the user and password information to see if login access is blocked after two attempts.

What happened after you failed to login the second time?

Once the user has failed to login a second time, they will be removed and will be unable to login again for 30 seconds. Once 30 seconds has gone the user can then reattempt the login.

From your console session on the router, issue the show login command to view the login status. In the example below, the show login command was issued within the 30 second login blocking period and shows that the router is in Quiet-Mode. The router will not accept any login attempts for 14 more seconds.

R1# show login

A default login delay of 1 second is applied.

No Quiet-Mode access list has been configured.

Router enabled to watch for login Attacks.

If more than 2 login failures occur in 120 seconds or less,

logins will be disabled for 30 seconds.

Router presently in Quiet-Mode.

Will remain in Quiet-Mode for 14 seconds.

Denying logins from all sources.

R1#

After the 30 seconds has expired, SSH to R1 again and login using the SSHadmin username and Admin1p@55 for the password.

After you successfully logged in, what was displayed? Once the user has logged in, all they will see is the banner.

Enter privileged EXEC mode and use Enablep@55 for the password.

If you mistype this password, are you disconnected from your SSH session after two failed attempts within 120 seconds? Explain.

If you mistype the password or get the password wrong more than 2 times, the user would be disconnected from the SSH and a 30 second timer will start blocking any sort of login. If you fail to login in again that timer will increase to 120 seconds.

Issue the show running-config command at the privileged EXEC prompt to view the security settings you have applied.

Configure the Switch for SSH Access

In Part 3, you will configure the switch in the topology to accept SSH connections. After the switch has been configured, establish an SSH session using Tera Term.

Configure the basic settings on the switch.

Console into the switch and enable privileged EXEC mode.

Enter configuration mode.

Disable DNS lookup to prevent the router from attempting to translate incorrectly entered commands as though they were host names.

Assign class as the privileged EXEC encrypted password.

Assign cisco as the console password and enable login.

Assign cisco as the VTY password and enable login.

Encrypt the plain text passwords.

Create a banner that will warn anyone accessing the device that unauthorized access is prohibited.

Configure and activate the VLAN 1 interface on the switch according to the Addressing Table.

Save the running configuration to the startup configuration file.

Configure the switch for SSH connectivity.

Use the same commands that you used to configure SSH on the router in Part 2 to configure SSH for the switch.

Configure the device name as listed in the Addressing Table.

Configure the domain for the device.

S1(config)# ip domain-name ccna-lab.com

Configure the encryption key method.

S1(config)# crypto key generate rsa modulus 1024

Configure a local database username.

S1(config)# username admin privilege 15 secret adminpass

Enable Telnet and SSH on the VTY lines.

S1(config)# line vty 0 15

S1(config-line)# transport input telnet ssh

Change the login method to use the local database for user verification.

S1(config-line)# login local

S1(config-line)# end

Establish an SSH connection to the switch.

Start Tera Term from PC-A, and then SSH to the SVI interface on S1.

Are you able to establish an SSH session with the switch?

Yes

SSH From the CLI on the Switch

The SSH client is built into the Cisco IOS and can be run from the CLI. In Part 4, you will SSH to the router from the CLI on the switch.

View the parameters available for the Cisco IOS SSH client.

Use the question mark (?) to display the parameter options available with the ssh command.

S1# ssh ?

-c    Select encryption algorithm

-l    Log in using this user name

-m    Select HMAC algorithm

-o    Specify options

-p    Connect to this port

-v    Specify SSH Protocol Version

-vrf  Specify vrf name

WORD  IP address or hostname of a remote system

SSH to R1 from S1.

You must use the –l admin option when you SSH to R1. This allows you to log in as user admin. When prompted, enter adminpass for the password.

S1# ssh -l admin 192.168.1.1

Password:

***********************************************

Warning: Unauthorized Access is Prohibited!

***********************************************

R1#

You can return to S1 without closing the SSH session to R1 by pressing Ctrl+Shift+6. Release the Ctrl+Shift+6 keys and press x. The switch privileged EXEC prompt displays.

R1#

S1#

To return to the SSH session on R1, press Enter on a blank CLI line. You may need to press Enter a second time to see the router CLI prompt.

S1#

[Resuming connection 1 to 192.168.1.1 ... ]

R1#

To end the SSH session on R1, type exit at the router prompt.

R1# exit

[Connection to 192.168.1.1 closed by foreign host]

S1#

What versions of SSH are supported from the CLI?

To find this out, I had to type in a command to see what version are supported. I found out that there are two versions that are supported. Version 1 is used by protocol 1 and the same goes for protocol 2 having version 2.

How would you provide multiple users, each with their own username, access to a network device?

The best way to do this is to setup a database. Having a database set up can help users get a username and password too.

Reflection (max 800 words):

Write a research report comparing various Encryption standards, e.g. AES, SHA, DES, RSA.

Introduction:

This report will cover a various amount of encryption standards. This report will cover: AES, SHA, DES and RSA and the differences between all the encryption standards and it will also go in depth about each encryption’s history.

AES:

AES also known as Advanced Encryption Standard is a fast and secure encryption method that was originally developed to be used by the US government. Ever since it has been used by the US government, the AES encryption has now become one of the most commonly used encryption methods today being used by most organizations in the world [1], for example: messaging apps like WhatsApp use the AES encryption to keep their data secure and to make sure its encrypted so it’s not easy to obtain said data / encrypt the data. [2]

Advances encryption standard has a lot of pros going for it too, lets take the cost to use AES. The cost to use AES is free, AES is to be released under a global, royalty-free and nonexclusive basis. [3] This is good as users who are interested in giving AES a shot can do so for no cost.

SHA:

SHA also known as Secure Hash Algorithms is a family of different encryptions that was developed by the National Security Agency (The NSA) [4]. Unlike the formerly known AES, SHA is not actually encryption. SHA is in fact a one-way hash function. [4] This makes SHA different when compared to encryption methods because SHA isn’t encryption and rather a one-way hash function unlike the recently mentioned AES. SHA isn’t just one thing, but rather as mentioned SHA is a family of different things. Within the SHA family there is SHA-1 and SHA-0. [5]

SHA-0 was the version made before SHA-1, created and developed in 1993 by the US National Security Agency (NSA) and SHA-1 is a type of algorithm that was created and developed by the National Security Agency in the US (NSA) in the year 1995. The SHA-1 is not a part of the DSS (Digital Signature Standard. [6]

DES:

DES also known as Data Encryption Standard is an outdated type of symmetric-key method of data encryption. DES has a key length of 567-bits and it was released known as Federal Information Processing Standards (also known as FIPS) in 1977, but now known as DES, it was officially withdrawn in 2005. [7] The way DES works is rather simple as all it will need to do is use the same key to encrypt and decrypt a message [8]. This could be bad as if someone was to find out the key then that user then knows that key to encrypt and decrypt the data, this is bad as once a key has been found, anyone could easily obtain that data and then decrypt it too for others to gain access to.

RSA:

RSA also known as Rivest-Shamir-Adleman is a type of encoded method that is mostly used for things like sending a user a coded message. [9] This is a really good way of encrypting as you can send a message to another user across the country encrypted and no one will be able to see what message you are sending to the receiving end.

Conclusion:

In conclusion, you can clearly see that all four encryption methods listed above are different in their own way and all have their own uses for different situations. Not all methods are around today but all have had their uses over the years and some like the AES are used commonly in the present day by big global companies when they want to encrypt data.

References:

| [1] | I. P. Team, “What is AES Encryption?,” ITPro, 30 September 2019. [Online]. Available: https://www.itpro.co.uk/security/29671/what-is-aes-encryption. [Accessed 30 September 2019]. |
| --- | --- |
| [2] | J. Lake, “What is AES Encryption and How Does it Work?,” CompariTech, 5 October 2018. [Online]. Available: https://www.comparitech.com/blog/information-security/what-is-aes-encryption/. [Accessed 5 October 2018]. |
| [3] | M. Rouse, “Advanced Encryption Standard (AES),” TechTarget, March 2017. [Online]. Available: https://searchsecurity.techtarget.com/definition/Advanced-Encryption-Standard. [Accessed March 2017]. |
| [4] | J. Moser, “What's The Diffrences Between SHA and AES Encryption,” StackOverflow, 13 June 2013. [Online]. Available: https://stackoverflow.com/questions/990705/whats-the-difference-between-sha-and-aes-encryption. |
| [5] | Techopedia, “Secure Hash Algorithm (SHA),” Techopedia, 2019. [Online]. Available: https://www.techopedia.com/definition/10328/secure-hash-algorithm-sha. |
| [6] | Techopedia, “Secure Hash Algorithm 1 (SHA-1),” Techopedia, 2019. [Online]. Available: https://www.techopedia.com/definition/30570/secure-hash-algorithm-1-sha-1. |
| [7] | T. Security, “AES vs. DES Encryption: Why Advanced Encryption Standard (AES) has replaces DES, 3DES and TDEA,” Syncsort, 21 August 2018. [Online]. Available: https://blog.syncsort.com/2018/08/data-security/aes-vs-des-encryption-standard-3des-tdea/. [Accessed 21 August 2018]. |
| [8] | M. Rouse, “Data Encryption Standard (DES),” Techtarget, November 2014. [Online]. Available: https://searchsecurity.techtarget.com/definition/Data-Encryption-Standard. [Accessed November 2014]. |
| [9] | J. Lake, What is RSA encryption and how does it work?, 2018. |