Microsoft Office User

[Email address]

Abstract

[Draw your reader in with an engaging abstract. It is typically a short summary of the document. 
When you’re ready to add your content, just click here and start typing.]

Transmission Matching

Powertrain Technology M4X02196

Powertrain Technology M4X02196

Transmission Matching

Subith Joseph 1902458

(Pinterest )

# Abstract

The purpose of this report is to study and analysis the performance data on a chosen vehicle; The vehicle chosen is the BMW S1000RR. The BMW S1000RR is a street legal Motorsports purposed motorcycle. The Vehicle will be put onto a motorcycle rolling road dynamometer and the following areas of performance will be studied and analysed: transmission Gearing, Engine/Wheel Torque and Power, Engine deceleration and more.

Table of Contents

Abstract	2

List of Figures	4

Introduction	7

Research / Literature review	8

BMW S1000RR	8

Engine	8

Drivetrain	8

Dynamometer	8

Methodology	9

Dynamometer	9

Excel	10

Analysis	11

Conclusion	12

# List of Figures

Figure 1 // Wheel Torque	5

Figure 2 // Tractive Effort	5

Figure 3// Velocity/Pdrag	6

Figure 4 // Gearing	6

| Ratios |  | Max Tractive Effort | Theoretical Gear Ratio | Max Road Speed | Max Road Speed |
| --- | --- | --- | --- | --- | --- |
| Primary | 1.652 | N |  | [m/s] | [Km/h] |
| 1st | 2.65 | 3777.74726 |  | 40.37622625 | 145.3544145 |
| 2nd | 2.09 | 2979.430858 | 2.122611827 | 51.19473663 | 184.3010519 |
| 3rd | 1.73 | 2466.227457 | 1.700181498 | 61.84797662 | 222.6527158 |
| 4th | 1.5 | 2138.347506 | 1.361820889 | 71.33133304 | 256.7927989 |
| 5th | 1.36 | 1938.768405 | 1.090798915 | 78.67426438 | 283.2273518 |
| 6th | 1.26 | 1796.211905 | 0.873714218 | 84.91825361 | 305.705713 |

Figure 4 // Gearing

# Introduction

The vehicle chosen, BMW S1000RR is a Street legal motorsports Focused motorcycle. The Powerplant of the vehicle is a 999cc 4 stroke 16 Valve DOHC inline 4. Since the Vehicle is a Motorsports Focused motorcycle it is built to handle high power and high stress with all components. The Transmission is a 6 speed Constant mesh sequential gearbox, for motorcycle sequential gearbox is most common, this is for weight saving and also the complete gearbox is compact.

# Research / Literature review

## BMW S1000RR

### Engine

The 1000cc engine which BMW has chosen for this vehicle was a race orientated engine made by the BMW Motorrad to compete for the 2009 Superbike World Championship  (Carroll, 2008).

The Engine is a Water/ Oil-cooled four-cylinder four stroke in-line engine. The bore x stroke of the cylinder are 80mm x 49.7mm. Rated to output 152kW (207 hp) at 13,500 RPM with a compression ratio of 13.3:1. The Engine has adopted various performance enhancing features; Variable intake camshafts, Variable intake manifold length, Drive by wire throttle and also Titanium Valves per cylinder (S1000RR Technical Data, n.d.)

### Drivetrain

The Transmission on the vehicle is a Straight cut 6 speed sequential gearbox, which is combined with a multi disc wet clutch system (BMW, n.d.). BMW has also added a quick shifter but as an optional extra. Quick shifter is an electronic controller that enables clutch-less ultra-fast gear changes by the engine cutting off ignition when the shifter pedal is moved either up or down in conjunction of sensors to detect movement. For downshift the engine quick shifter the engine would auto blip (raise engine RPM to match the transmission speed).

## Dynamometer

The Dynamometer used for this vehicle is the MAHA MSR 400. This Dynamometer is designed for motorcycle and can be used to measure a wide variety of data:

- Static power measurement at constant RPM

- Static power measurement at constant speed

- Static power measurement at constant tractive force

- Dynamic power measurement with adjustable acceleration

- MAHA towing power measurement guarantees the highest degree of accuracy when measuring power: Precision calculation of the parasitic losses of the dynamometer, of the vehicle's drive train and the tyre to roller friction and flex losses

- Tachometer testing with up to ten freely selectable test points

- Distance measurement included

- Stopwatch function for measurement of acceleration between optional speed markers as standard

- Optional load simulation with freely programmable load profile

- Optional driving simulation with freely programmable speed profile

- Option of storing programmed profiles in database

(Single Roller Dynamometer msr 400, n.d.)

# Methodology

## Dynamometer

- Before starting anything, wearing the correct PPE (Personal Protective Equipment) is mandatory for the equipment’s that are going to be used in this report. (Steel cap boots, Fire resistance overalls, Ear plugs + defender, Safety goggles)

- Checking the Area of the dynamometer facility for any risk is mandatory before stating the operation of the dynamometer. This includes moving item that can cause a personal to trip and fall or anything that can possibly interfere with the dynamometer and motorcycle which could cause damage

- Using a selected dynamometer application which would be used to read the data from the dynamometer roller. Selecting the right type of data is important

- Selecting vehicle data, set to 250 horse power, set ISO for power correction

- Secure the motorcycle onto the dynamometer

- Place the extraction pipe on the exhaust if the facility located is enclosed, if not open doors for ventilation due to exhaust fumes (Carbon Monoxide, Nitrogen Monoxide, Carbon Oxide)

- Warming the motorcycle engine up to operating temperature, this can be confirmed by checking the vehicle dash where the coolant temperature gauge is indicated, also plugging the ODB 2 Cable from the computer which controls the Dynamometer to the motorcycle

- Select sweep test on the dynamometer program

- Select gear on the motorcycle closest to 1:1 (fifth gear)

- Run the bike full throttle to redline

- After testing acceleration and power, let the motorcycle reduce engine speed as it is without any inputs to the brakes

- Doing this the dynamometer records the deceleration of the vehicle

## Excel

- To make the results more scientifically accurate converting the velocity to [m/s] from [km/h] is needed

- Velocity m/s=Velocity [km/h]*10003600

- Calculate Transmission efficiency, η

- η=Power @ wheelspower @ Engine

- Calculate Wheel Torque (T) [Nm]

- Wheel Torque = Engine Torque*1st Gear Ratio*Final Drive Ratio*5st Gear Ratio*Transmission efficency

- Calculate Tractive Effort (TE) [N]

- Tractive Effort=wheel TorqueRolling Radius

- Calculate Final Drvie (f)

- Final Drive=(2π * Rplling Radius * RPM)(60 * elocitym/s * 5th Gear Ratio * Primary Gear Ratio)

- Find the total Average of the Final Drive

- Average of Final Drive = AVERAGE (select all variables of Final Drive)

- Calculate the Speed Ratio

- Speed Ratio=Max Torque engine speedMax Power engine speed

- Calculate Theoretical Intermediate gear ratio

- 1st gear = n1

- 2nd gear =n1*Speed Ratio

- 3rd gear =n1*Speed Ratio2

- 4th gear =n1*Speed Ratio3

- 5th gear =n1*Speed Ratio4

- Calculate Max Road Speed

- Max road speed [ms]=2π*Rolling Radius*1420060*Primary Gear Ratio*Final Drive*1st Gear Ratio

- maxRoad speed kmh= Max Road speed [ms]*36001000

- Create a visual scatter line graph showing wheel torque

- Select all variable for Engine speed [ rpm] as x-axis and wheel Torque [Nm] as y-axis

- Construct a visual scatter line graph showing Tractive effort

- Select all variables for engine speed [rpm] and Tractive effort [N] as y-axis

# Analysis

Figure 1 // Wheel Torque

This graph shows the wheel torque at specific engine speed. The graphs show at around 11000 rpm it has reached max wheel torque then onwards it dips downwards, this shows the engine. This graph also shows that from 2000 – 4000 rpm it aggressively increases but then straightens out up till 8000 rpm it increases steeply; this can be due to the Variable intake manifold the engine decides to open up the valve in the manifold to allow more air to inter the cylinder.

Figure 3// Velocity/Pdrag

This graph shows that as the velocity increases drag increases, this is a clear representation of the faster the vehicle is moving the more power needed to overcome drag.

# Conclusion

In conclusion the using the dynamometer was a great learning experience and can also be used in future tasks when diagnosing or engineering engines. The BMW S1000RR is a high-performance race orientated street bike, from the data collected it shows that this bike has a high-power output and since is a powerful motorcycle it also has a high deceleration. Using a new motorcycle given me a greater understanding in the current technology which in involved in the motorsports focused vehicle.

# Bibliography

(n.d.). Retrieved from Pinterest : https://www.pinterest.co.uk/pin/838232549368473034/

BMW. (n.d.). S1000RR Technical Data. Retrieved from BMW Motorrad: https://www.bmw-motorrad.de/de/models/sport/s1000rr/technicaldata.html

Carroll, M. (2008, April 16). BMW offically unveils World Superbike contender. Retrieved from Motorcycle News: http://www.motorcyclenews.com/MCN/News/newsresults/mcn/2008/april/14-20/apr1608bmwoficiallyunveilsworldsuperbikecontender/

Single Roller Dynamometer msr 400. (n.d.). Retrieved from MAHA: https://www.maha.co.uk/single-roller-dynamometer-msr-400.htm

# Works Cited

(n.d.). Retrieved from Pinterest : https://www.pinterest.co.uk/pin/838232549368473034/

BMW. (n.d.). S1000RR Technical Data. Retrieved from BMW Motorrad: https://www.bmw-motorrad.de/de/models/sport/s1000rr/technicaldata.html

Carroll, M. (2008, April 16). BMW offically unveils World Superbike contender. Retrieved from Motorcycle News: http://www.motorcyclenews.com/MCN/News/newsresults/mcn/2008/april/14-20/apr1608bmwoficiallyunveilsworldsuperbikecontender/

Single Roller Dynamometer msr 400. (n.d.). Retrieved from MAHA: https://www.maha.co.uk/single-roller-dynamometer-msr-400.htm