73%
APPENDIX GA36b

#### LEVEL 5 ASSESSMENT SPECIFICATION

| Student name: | Matthew John Permintella | Student ID number: | PER18841948 |
| --- | --- | --- | --- |
| Programme: | HND LEVEL 5 |  |  |
| Module: | MECHANICAL PRINCIPLES |  |  |
| Module code: | M5X00926 | ## Contribution to Overall Module Assessment (%): | 50% |
| Lecturer: | EMILY NYATHI | Internal Verifier: | EDWARD HORNAGOLD |
| Assignment Title: | Power Transmission Systems | Word count (or equivalent): | (Nominal 2500) |
| Submission deadline: | 1/05/2020 | Return date of provisional marks & written feedback: | 8/05/2020 |
| Submission method: | All written assessments, where practical and possible, must be submitted via Turnitin unless otherwise instructed by the Lecturer. (Please DO NOT put this assessment specification into Turnitin or it will match many similarities with other students’ submissions.) Alternative submission method (if applicable): Late submission of the assessment will result in a late penalty mark. Penalties for late submission: Up to one-week late, maximum mark of 40%. Over one week late, 0%. Only the Extenuating Circumstances Panel may approve a change to submission dates. |  |  |
| Academic honesty / referencing: | Academic honesty is required. In the main body of your submission you must give credit to authors on whose research and ideas your work is based. Append to your submission a reference list that indicates the books, articles, etc. that you have used, cited or quoted in order to complete this assessment. |  |  |

Student FEEDBACK FORM

| Generic Assessment Criteria | Marks available | Marks awarded |
| --- | --- | --- |
| 1. Engagement with Literature Skills . |  |  |
| You have engaged with a good range of literature relevant to the subject, however, the quality of your referencing needs to improve. |  | 16 |
|  | 25 |  |
| 2. Knowledge and Understanding Skills |  |  |
| The report shows good effort and good understanding of the topics covered. Very good application of the governing equations. However, take extra care when rounding off your figures and transposing formulae. |  | 23 |
|  | 30 |  |
| 3. Cognitive and Intellectual Skills |  |  |
| You have analysed your data using tables and graphs and reached some sound conclusions. You have demonstrated that you can summarise scientific facts into your own words. There is a logical flow in the writing and the calculation process, and the analysis of the results is easy to follow. You have made an attempt to reach conclusions on your results. |  | 19 |
|  | 25 |  |
| 4. Practical Application Skills |  |  |
| You have identified uses of the different types of gears used in industry and their advantages and disadvantages compared to each other. A good and appropriate application of established methods and techniques. You have demonstrated that you have a clear understanding of studies to date and you have the ability to work as an individual. |  | 12 |
|  | 15 |  |
| 5. Transferable Skills for Life and Professional Practice |  |  |
| You have demonstrated basic skills for employment applicable to the exercise of personal responsibility and decision-making with a few minor errors. | 5 | 3 |

| Assessment Mark (Assessment marks are subject to ratification at the Exam Board. These comments and marks are to give feedback on module work and are for guidance only until they are confirmed. ) | Late Submission Penalties (tick if appropriate) |  | 73% |
| --- | --- | --- | --- |
|  | Up to 1 week late (40% Max) |  |  |
|  | Over 1 week late (0%) |  |  |

# Belt Drive Friction

Matthew John Permintella | HND Mechanical Engineering | 03/04/2020

Contents

1.0 Introduction 								Page 2

2.0 Objectives								Page 3

3.0 Theory									Page 4

4.0 Equipment 								Page 5

5.0 Procedure								Page 5

6.0 Data, analysis, calculations, results					Page 5-11

7.0 Conclusions								Page 12

8.0 References 								Page 12

9.0 Bibliography								Page 14

1.0 Introduction

Belt drives are used in vast range of machines, they have been used in washing machines, cars.  They are an effective solution of transmitting power from one machine to another.  Flat belt are made of synthetic rubber and can utilise composite steel in there design,  when using belt drives consideration needs to be made as these belt tend to slip, the co-efficient of friction plays a vital role between the pulley and the belt, the co-efficient of friction needs to be high enough in order to stop any slipping occurring during in operation. (Grosjean, 1986).

The aim of this lab session is to investigate the relationship between the tensions in a slipping pulley system and compare these with any theoretical valves calculated.

2.0 Objectives

This experiment has three objectives:

- To show that a constant angle of lap θ  has a constant tension ratio of T1T2

- To verify the relationship of  T1T2= eμθ

- To determine the friction μ between the Cast Iron pulley and the belt.

(Lab sheet handout).

3.0 Theory

As given in the lab sheet;

For a flat belt drive:  T1T2=eμθ

T1 = Tight side tension

T2 = Slack side tension

µ = Coefficient of friction

θ = Angle of lap (radians)

If a dependence of T1 is plotted against T2 for a constant angle of lap θ then a straight line should pass through the origin because eμθ is constant.

T1T2= eμθ=k    or         T1 = k.T2

The slope k can be measured, and it can yield a value for the coefficient of friction μ.

T1T2= eμθ=k                        ln k = μθ                   = μ=1θlnk

If ln T1T2 is plotted against θ then the resulting curve should be a straight line that passes through the origin.

The slope for this line equals µ: ln T1T2=µθ

(Lab sheet handout).

3.1 Derivation of capstan equation

For a flat belt drive:  T1T2=eμθ

Let,

T1 = Tight side tension of the belt

T2 = Slack side tension of the belt

µ = Coefficient of friction between belt and pulley

θ = Angle of lap of belt over the pulley(radians)

Consider a short length of belt subtending an angle δθ at the centre of the pulley as shown in Fig(1).

Fig (1): Tensions in Belt. (Fawcett and Burdess, 2011)

Let,

N = Normal reaction between the element length of a belt and pulley.

δT = Increase in tension in tight side that that of slack side.

T+ δT= Tension on the tight side of the element.

Resolve tangential forces,

μN+Tcosδ2-T-δTcosδθ2=0

As,

δθ is very small, cosδθ2≡1

μN+T-T- δT=0        or        δT= μN   (Equation 1).

Resolve radial forces,

N-Tsinδθ2-T+δTsinδθ2=0

As,

δθ  is very small, sinδθ2 ≡δθ2

N-Tδθ2-Tδθ2- δTδθ2=0

Neglecting the product of two small quantities,

N=T δθ 	(Equation 2).

From Equations (1+2).

δT= μT δθ  Or δTT= μδθ

Integrating, we get

T2T1δTT= 0θμδθ   	             T1T2=eμθ

(Fawcett and Burdess, 2011).

4.0 Apparatus

Below is a list of the Apparatus needed for the experiment to be completed:

- Belt friction rig

- Dial test indicator

- Weight carrier

- 2 – 25 kg weights

Fig(1): Dial test indicator	Fig(2): Belt friction rig with weight
 carrier and weights

(Lab sheet handout).

5.0 Procedure

As given in lab sheet;

5.1 Cantilever calibration

- Turn angle of lap protractor to 0.

- Hang belt and weight carrier on the cantilever.

- Set the dial test indicator to 0.

- Measure the deflection of the cantilever for masses from 0 – 24kg in steps of 3kg.

- Plot T2 against deflection for the cantilever.

- Use the plot as a calibration curve to determine T2 on the slack side against deflection.

(Lab sheet handout).

5.2 Constant angle of lap

- Set lap θ to 90⁰, attach the belt and set the dial indicator to 0.

- Switch the pulley motor on.

- Measure the deflection for masses 0-20kg.

- Using the calibration curve compute T2 and plot T1 against T2.

- Calculate µ using first method in theory section.

5.3 Variable angle of lap

- Attach belt and set the test indicator to 0.

- Attach 20kg to the belt.

- Switch pulley motor on.

- Measure deflection for angle of lap θ from 240⁰ to 10⁰.

- Determine T2 for each angle with aid of the calibration curve.

- Plot lnT1T2 against θ, rad.

- Record µ from the slope of the curve.

(Lab sheet handout).

6.0 Results

6.1 Cantilever calibration

Table 1: Cantilever calibration results.(Lab results).

6.1.1 Calculations

Example (1)

T2=m*g					
T2=24kg*9.81 
T2=235.44 N

Example (2)

T2=m*g
T2=12kg*9.81 
T2=117.72 N

6.1.2 Graph

Fig (3) Graph for slack side Vs. Deflection. (Excel).

6.2 Constant angle of lap

Table 2: Constant angle of lap results(Lab results).

6.2.1 Graph

Fig (4) Graph for T1 Vs. T2. (Excel).

6.2.2 Calculation

μ=1θlnk

T1=k.T2   				    T1/T2=K

K = 21.8639.24=1.794

T2=k* δ

T2=1.794*11

T2=19.73

Using Equation from lab sheet;

T1T2= eμθ=k

T1=39.24
T2=21.86

k=1.794

θ= π/2

eμπ2=k

eμπ2=1.794

μ*π2=ln⁡(1.794)

μ*2π=0.584

μ=0.371

For flat belt, since T1= T2eμθ, compare to y=mx,

T1=y
T2=x
eμθ= m1 (gradient of the graph)
Therefore,  T1T2= eμθ= m1

Because the graph is T2 against T1, thus the gradient of the graph will be 1m1.

(Fawcett and Burdess, 2011).

6.3 Variable angle of lap

Table 3: Variable angle of lap results(Lab results).

6.3.1 Graph

Fig (4) Graph for T1/T2 Vs. Angle of lap (θ).(Excel).

The above graph allows you to get values for T1/T2, this is represented by row 5 table: (3).

Fig (4) Graph for lnT1/T2 Vs. Angle of lap (θ) (Excel).

The above graph allows you to get values for T1/T2, this is represented by row 6 table: (3).

If ln T1T2 is plotted against θ the angle of lap (θ) the resulting curve should be a straight line that passes through the origin. The graph shows this linear relationship which relates to y=mx+c.

6.3.2 Calculations

To calculate sliding coefficient, μ for a flat belt, since:

T1= T2eμθ		           T1=m*g	  	                 T2=K* δ

lnT1T2= μθ			T1=20kg*9.81                        T2=1.794* 26

μ =ln(T1T2) θ			T1=196.2 N	          		        T2=46.64 N

μ =ln196.246.6θ

μ =1.443.84=0.375

(Fawcett and Burdess, 2011).

7.0 Conclusion

Throughout the experiment I realised several things could have been improved so the results were more accurate:

- Apparatus error could be minimised by utilising more accurate measurement devices, or maintaining equipment more often

- In-order to aid better results each experiment should be completed 3 times and average readings taken.

- An electronic device could have been used to aid more accurate results.

- Hysteresis may have occurred within the experiment this could have been prevented if a new belt were used.

- Random human error could have played a part during the experiment.

(Belt Drive | Belt (Mechanical) | Physics & Mathematics, 2020)

7.1 Is tension ratio of T1/T2  constant for the given lap angle?  Confirm relationship between T1= T2eμθ is constant?

From table 2 you can see the tension ratio for a given lap angle is constant, this was solidified further during experiment 2.  The coefficient of friction remained constant for all masses used.  T1= T2eμθ confirms that μ is indeed constant.  Graph 2 confirms this further that there is a relationship between T1 and T2 as the straight line is linear. However, with experiment 3 when the angle is changed the ratio of T1/T2 reduces as the lap angle increases. (Belt Drive | Belt (Mechanical) | Physics & Mathematics, 2020).

7.2 Determine the friction μ between the Cast Iron pulley and the belt?

As you can see from experiment 2 and 3 the value μ has a slight percentage error.  However, they should be the same.  This error can be improved by implementing changes/ improvements and completing the experiment once again. (Belt Drive | Belt (Mechanical) | Physics & Mathematics, 2020).

8.0 References

[1] Bird, J., 2015. Science for engineering. 5th ed. 2 Park Square, Milton Park, Abingdon, Oxon OX14 4RN: Routledge.

[2] Fawcett, J. and Burdess, J., 2011. Basic Mechanics with Engineering Applications. London: Routledge.

[3] Grosjean, J., 1986. Principles of Dynamics. Cheltenham: Thorne

[4] R. C. Hibbeler, 2010. Engineering Mechanics. 12th ed. Person Education south Asia Pte. Ltd.

9.0 Bibliography

Book used for research are below:

[1] Bird, J., 2015. Science for engineering. 5th ed. 2 Park Square, Milton Park, Abingdon, Oxon OX14 4RN: Routledge.

[2] Fawcett, J. and Burdess, J., 2011. Basic Mechanics with Engineering Applications. London: Routledge.

[3] Grosjean, J., 1986. Principles of Dynamics. Cheltenham: Thornes.

[4] R. C. Hibbeler, 2010. Engineering Mechanics. 12th ed. Person Education south Asia Pte. Ltd.

[5] Ferdinand, P. Beer, E., 1996. Vector Mechanics for Engineers, Static and Dynamics. International Ed. New York: McGraw-Hill Co.

[6] Scribd. 2020. Belt Drive | Belt (Mechanical) | Physics & Mathematics. [online] Available at: <https://www.scribd.com/document/397273498/Belt-Drive> [Accessed 14 April 2020].

[7] Scribd. 2020. Belt Friction Full Report | Belt (Mechanical). [online] Available at: <https://www.scribd.com/doc/290820876/Belt-Friction-Full-report> [Accessed 23 April 2020].

[8] Scribd. 2020. Lab 3 Belt Friction Experiment | Belt (Mechanical) Scribd. [online] Available at: <https://www.scribd.com/doc/127522615/Lab-3-Belt-Friction-Experiment> [Accessed 23 April 2020].

nbTask II:  Gear trains

Introduction

Gear trains refer to a mechanism with two or more gears meshed in series to transmit power from from one shaft to another. The nature of the train used depends upon the velocity ratio required and the relative position of the axis of the shafts. A gear train may consist of bevel, spur or spiral gears.(Handout).

Research

Discribe in detail the the following types of gear trains, giving typical applications in industry. (1000 words)

Simple gear train

A simple gear train generally consists of only one gear on any shaft fig(1).  The gears have pitch circles, when there is a small distance bewteen any two gears is small, gear 1 and gear 2 mesh together in order to transmit motion to the other shaft. Gear 1 will be known as the driver gear as it transmits motion to gear 2, gear 2 will be known as the driven or follower.  The driven gear will have opposite rotation from driver gear unless a idler gear is implimented in order for the driver and driven to have the same orientation of motion. (Iitism.ac.in. (2020); Moodlearn.ariel.ac.il. (2020)).

Fig(1): Simple gear train. (Moodlearn.ariel.ac.il. (2020)).

Simple gear train is a system which consists of gears with different module are meshed together to provide max torque can be transmitted without slip.  If a simple gear train is used to give a large speed reduction, the last gear must be very large. (Bird, 2015).

Some of its applications in industry include gearbox of automobiles, heavy press machine and lathe machine. (Moodlearn.ariel.ac.il. (2020)).

Compound gear train

A compound gear train consists of more that one gear on any given shaft.  Idler gear are useful in bridging a given distance bewteen the driver and driven gears.  When the distance bewteen the driven and driver has to be bridged over by intermediate gears and at the same then speed ratio is needed, the advantage of ultilzing intermataite gears is intensified by providing compound gears on intermediate shafts.  This system setup will have two gears fixed on it so they have the same speed.  One of the two gears will mesh together with the driver and the other will attached to the next gears shaft as shown in fig(2).  The advantage of using a compound gears system over a simple gear train system is a much larger speed reduction can be obtained, normally if the speed reduction is in excess of 7:1 then a compound gear train is used. (Iitism.ac.in. (2020); Moodlearn.ariel.ac.il. (2020)).

Fig(2): Compound gear train. (Moodlearn.ariel.ac.il. (2020)).

Some of its applications in industry include gearbox of automobiles, lathe machine, gear levers, clocks and watches; A device which needs extremely low rate of rotation in it. (Bird, 2015).

Reverted gear train

A reverted gear is shown in Fig(3).  The axes of first gear  (driver 1) and the last (driven) are co-axial.  We can see that first gear  drives second gear in the opposite direction.  Secondly and third gear are mounted on the same shaft, they form a compound gear and the third gear will have the same orientation as that of the second gear.  The third gear is now the scond driven gear it drives the forth gear which is driven in the same orentation of that of the first gear.  This allow the motion of the first and last gear to be like. (Iitism.ac.in. (2020); Moodlearn.ariel.ac.il. (2020)).

Fig(3): Reverted gear train. (Moodlearn.ariel.ac.il. (2020)).

Epicyclic gear train

In an epicycle gears train the axes of shafts wich are connected to the gear can move relative to a fixed axis.  A epicycle gear train is shown in fig(4). Gear A and the arm C rest on the same axis which they can rotate about 01.   As you can see gear A meshes with gear B has its axis on the arm at 02 which allows rotation at that point.  If the arm is fixed then gear A can drive gear B or vice-versa. However if gear A is fixed then arm arm will rotate about this gear axis(01), which forces gear B to rotate around that gear.  This motion is known as epicycle, gear trains are arrange so that one or more of their members move around or upon another member, these conbined are known as an epicycle gear train.  They can be simple of compound.  The advantages of using such a system is for trainsmitting high velocity ratios with gears of a standard size in a reletivity small space.  (Iitism.ac.in. (2020).

Fig(4): Reverted gear train. (Moodlearn.ariel.ac.il. (2020)).

These  gear train system is used in many application some of these include: gear of lathes, hoists, pilley blocks and watches. (Iitism.ac.in. (2020).

Problem solving

- The gearing of  a machine tool is shown in Figure 1 below. The motor shaft is connected to gear A and rotates at 1000r.p.m The gear wheels B,C,D and E are fixed to parallel shafs rotating together. The final gear F is fixed on the output shaft. What is the speed of gear F?.

Fig(5): Gear train.

From Fig(5) we see that gears A, C and E are drivers while gears B, D and F are driven or followers.  Let the gear A rotates in clockwise direction.  Since the gears B and C are mounted on the same shaft, therefore it is a compound gear and the direction or rotation of both these gears is same (e.g. anticlockwise). Similarly, the gears D and E are mounted on the same shaft, therefore it is also a compound gear and the direction of rotation of both these gears is same (e.g. clockwise). The gear F will rotate in anticlockwise direction. (Iitism.ac.in. (2020))

The numbers of of teeth are given on each gear are shown in the table below.

| Gear | A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- | --- |
| No. Of teeth | 20 | 50 | 25 | 85 | 28 | 65 |

Fig(5a): Gear teeth Number’s.

Given:

TA=20 ; TB=50 ; TC=25 ; TD=85  ; TE=28 ; TF=65

Let, 		NF=Speed of gear F, i.e last driven or follwer.

Thus,

Speed of the first driverSpeed of the last driven[follower]= Product of no. of teeth on drivensProduct of no. of teeth on drivers

Thus,

NANF= TB* TD* TFTA* TC* TE

So,

50*85*6520*25*28=19.73

NANF=19.73

So,

NF= 100019.73

NF=50.68 r.p.m

(Iitism.ac.in. (2020)).

2 i.  What type of gears are the best for applications that require speed reduction and torque multiplication?

Compound gears

2 ii. Two parallel shafts, about 800mm apart are to be connected by spur gears. One shaft is required to run at 360r.p.m and the other at 140r.p.m. Design the gears, given that the pitch is to be 25mm.

Given: x=800mm ; N1=360 r.p.m ; N2=140 r.p.m ; Pc=25mm

Let 	d1=Pitch circle diameter of the first gear,
d2= Pitch circle diameter of the second gear.

We know that speed ratio,

N1N2= d2d1= 360140= 2.57        or         d2= 2.57d1             ........(1)

Centre distance between the shafts (x),

800= 12 (d1+ d2)           or           d1+d2=1600		.....(2)

From equation (1) and (2), we find that

d1+2.57d1=3.57d1

16003.57=448.179mm

d1=448.2mm

d2 = d1*2.57=1151.82mm

d2=1151.8mm

∴Number of teeth on the first gear,

T1= πd1PC= π*448.225=56.32

∴Number of teeth on the second gear,

T2= πd2PC= π*1151.825=144.73

Since the number of teeth on both gears need to be complete whole numbers, we shall take first gear 57mm and second gear 146mm.

Calculate pitch circle diameter of first gear,

d1= T1* PC π=57*25π=453.59 mm

Pitch circle diameter of second gear,

d2= T2* PC π=146*25π=1161.83 mm

∴ Exact distance between the two shafts,

x= d1+d22=453.59+1161.832=807.71 mm

Thus,

First gear must have 57 teeth, second gear 146 teeth.  Pitch circle diameter of first gear must be 453.59 mm and second gear must be 1161.83 mm.  The exact distance needed bewteen the two shafts must be 807.71 mm. (Moodlearn.ariel.ac.il. (2020)).

3.A riverted gear train is to be designed such that its speed ratio is 12, as shown in Figure 2.The module pitch of gears A and B is 3.13mm and of geras C and D is 2.5mm. What are the suitable numbers of teeth for the gears? No gear should have less than 24 teeth.

200mm

Fig (6). Reverted gear train.

Given:

Speed ratio, NAND=12 ; mA= mB=3.13 mm ; mC= mD=2.5 mm

Let,

NA=Speed of gear A,

TA=Number of teeth on gear A,
rA=Pitch circle radius of gear A,
NB, NC,ND=Speed of respective gears,
TB,TC,TD=Number of teeth on repective gears,
rB,rC,rD=Pitch circle radii of respective gears.

We know that circular pitch,

pC= 2πrT= πm             or             r= mT2     (m= the module)

∴

r1= mT12 ; r2= mT22 ; r3= mT32 ; r4= mT42 		.......(1)

From equation (1),

mT12+ mT22= mT32+ mT42

T1+T2= T3+T4

Since the speed ratio bewteen the gears are equal, therefore,

NANB= NCND= 12=3.464

Also the speed ratio of any pair of gears is the inverse of their number of teeth, therefore,

TBTA= TDTC=3.464

We know that the distance bewteen the shafts,

x= rA+rB= rC+rD=200mm

Or,

mA*TA2+ mB*TB2= mC*TC2+ mD*TD2=200mm

{∴ r= m*T2}					{∴ mA=mB, and mC=mD}

3.13TA+TB=2.5 TC+TD=400

∴ TA+TB=4003.13=127.79 	….(2)

TC+ TD=4002.5=160 	   ….(3)

From equation (1), TB=3. 464 TA. Substituting this value of TB in equation 2.

TA=127.794.464=28.62 round up (29 Teeth)

TA=29 Teeth

TB=127.79-29=98.79 round up (99 Teeth)

TB=99 Teeth

Again from equation (1), TD=3.464 TC.  Substituting this value of TD in equation (3)

TC=1604.464=35.84 round up (36 Teeth)

TC=36 Teeth

TD=160-36=124

TD=124 Teeth

Check:

NAND= TB*TDTA*TC= 99*12429*36= 11.87

The speed ratio of the reverted gear train=11.87

The value of 11.87 is very close to the value given in the question 12 proving that the answer is highely likely to be correct slightly off due to rounding errors.

(Fawcett. and Burdess (2011); Moodlearn.ariel.ac.il. (2020)).

References

[1] Fawcett, J. and Burdess, J., 2011. Basic Mechanics with Engineering Applications. London: Routledge.

[2] Grosjean, J., 1986. Principles of Dynamics. Cheltenham: Thornes.

[3] Iitism.ac.in. 2020. [online] Available at: <https://www.iitism.ac.in/~shibayan/MCC%2013103%20Kinematics%20of%20Machines/TOM%203BT%2014-15%20lecture%20set-6%20_Gear%20Train_KG.pdf> [Accessed 7 April 2020].

[4] Moodlearn.ariel.ac.il. 2020. [online] Available at: <https://moodlearn.ariel.ac.il/pluginfile.php/837913/mod_resource/content/0/%D7%9E%D7%9E%D7%A1%D7%A8%D7%95%D7%AA.pdf> [Accessed 8 April 2020].

Biblography

[1] Bird, J., 2015. Science for engineering. 5th ed. 2 Park Square, Milton Park, Abingdon, Oxon OX14 4RN: Routledge.

[2] Fawcett, J. and Burdess, J., 2011. Basic Mechanics with Engineering Applications. London: Routledge.

[3] Grosjean, J., 1986. Principles of Dynamics. Cheltenham: Thornes.

[4] Iitism.ac.in. 2020. [online] Available at: <https://www.iitism.ac.in/~shibayan/MCC%2013103%20Kinematics%20of%20Machines/TOM%203BT%2014-15%20lecture%20set-6%20_Gear%20Train_KG.pdf> [Accessed 7 April 2020].

[5] Moodlearn.ariel.ac.il. 2020. [online] Available at: <https://moodlearn.ariel.ac.il/pluginfile.php/837913/mod_resource/content/0/%D7%9E%D7%9E%D7%A1%D7%A8%D7%95%D7%AA.pdf> [Accessed 8 April 2020].

[6]https://books.google.co.uk/books?id=hpmA4Ch2bfwC&pg=PA441&lpg=PA441&dq=simple+gear+train+industry+application&source=bl&ots=OxoqJVvJdU&sig=ACfU3U2RqFtWkvFN7TEw202804KpYXQqnw&hl=en&sa=X&ved=2ahUKEwiUlrrnodjoAhWBVBUIHSI_DgkQ6AEwEXoECA4QKQ#v=onepage&q=simple%20gear%20train%20industry%20application&f=false