Router R1 Configurations

Unauthorised Access Is Strictly Prohibited

User Access Verification

Password:

R1> en

Password:

R1#show running-config

Building configuration...

Current configuration : 1438 bytes

!

! Last configuration change at 10:42:45 UTC Tue Mar 27 2018

! NVRAM config last updated at 10:43:21 UTC Tue Mar 27 2018

!

version 12.4

service timestamps debug datetime msec

service timestamps log datetime msec

service password-encryption

!

hostname R1

!

boot-start-marker

boot-end-marker

!

logging message-counter syslog

enable password 7 14321C0A0E08793B043B2075

!

no aaa new-model

!

dot11 syslog

ip source-route

!

!

ip cef

!

!

no ip domain lookup

no ipv6 cef

!

multilink bundle-name authenticated

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

voice-card 0

!

!

!

!

!

archive

log config

hidekeys

!

!

!

!

!

!

!

!

!

interface GigabitEthernet0/0

description PC-B L PORT CONNECTED TO ROUTER GE0/0 PORT

ip address 172.16.2.1 255.255.255.192

duplex auto

speed auto

!

interface GigabitEthernet0/1

description SWITCHPORT FA0/1 CONNECTED TO ROUTER GE0/1 PORT

ip address 172.16.2.65 255.255.255.240

duplex auto

speed auto

!

interface Serial0/0/0

no ip address

shutdown

no fair-queue

clock rate 2000000

!

interface Serial0/0/1

no ip address

shutdown

clock rate 2000000

!

ip forward-protocol nd

no ip http server

no ip http secure-server

!

!

!

!

!

!

!

!

!

control-plane

!

!

!

!

!

!

!

!

!

banner motd ^C

Unauthorised Access Is Strictly Prohibited

Unauthorised Access Is Strictly Prohibited ^C

!

line con 0

password 7 05080F1C2243

login

line aux 0

line vty 0 4

password 7 121A0C041104

login

!

scheduler allocate 20000 1000

end

R1#show ip interface brief

Interface                  IP-Address      OK? Method Status                Protocol

GigabitEthernet0/0         172.16.2.1      YES manual up                    up

GigabitEthernet0/1         172.16.2.65     YES manual up                    up

Serial0/0/0                unassigned      YES unset  administratively down down

Serial0/0/1                unassigned      YES unset  administratively down down

R1#show ip route

Codes: C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route

Gateway of last resort is not set

172.16.0.0/16 is variably subnetted, 2 subnets, 2 masks

C       172.16.2.0/26 is directly connected, GigabitEthernet0/0

C       172.16.2.64/28 is directly connected, GigabitEthernet0/1

R1#