show running-config

DLS2#show running-config

Building configuration...

Current configuration : 3473 bytes

!

version 12.2(37)SE1

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname DLS2

!

!

enable secret 5 $1$mERr$9cTjUIEqNGurQiFU.ZeCi1

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

vtp domain SoAC

vtp mode transparent

!

spanning-tree mode pvst

spanning-tree vlan 123,234 priority 24576

spanning-tree vlan 12,434,800,1010,1111,3456 priority 28672

!

!

!

!

!

!

vlan 12

name executives

!

vlan 123

name cubes

!

vlan 234

name guest

!

vlan 434

name parking

!

vlan 567

name accounting

!

vlan 800

name native

!

vlan 1010

name voive

!

vlan 1111

name videonet

!

vlan 3456

name management

!

interface Loopback0

ip address 1.1.1.1 255.255.255.255

!

interface Port-channel1

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface Port-channel12

!

interface FastEthernet0/1

shutdown

!

interface FastEthernet0/2

shutdown

!

interface FastEthernet0/3

shutdown

!

interface FastEthernet0/4

shutdown

!

interface FastEthernet0/5

shutdown

!

interface FastEthernet0/6

switchport access vlan 1010

switchport mode access

switchport nonegotiate

shutdown

!

interface FastEthernet0/7

switchport trunk native vlan 800

switchport trunk encapsulation dot1q

switchport mode trunk

channel-group 1 mode active

!

interface FastEthernet0/8

switchport trunk native vlan 800

switchport trunk encapsulation dot1q

switchport mode trunk

channel-group 1 mode active

!

interface FastEthernet0/9

switchport trunk native vlan 800

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/10

switchport trunk native vlan 800

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/11

switchport trunk encapsulation dot1q

switchport mode trunk

channel-group 12 mode passive

shutdown

!

interface FastEthernet0/12

switchport trunk encapsulation dot1q

switchport mode trunk

channel-group 12 mode passive

shutdown

!

interface FastEthernet0/13

shutdown

!

interface FastEthernet0/14

shutdown

!

interface FastEthernet0/15

switchport access vlan 1111

switchport mode access

switchport nonegotiate

shutdown

!

interface FastEthernet0/16

switchport access vlan 567

switchport mode access

switchport nonegotiate

shutdown

!

interface FastEthernet0/17

switchport access vlan 567

switchport mode access

switchport nonegotiate

shutdown

!

interface FastEthernet0/18

switchport access vlan 567

switchport mode access

switchport nonegotiate

shutdown

!

interface FastEthernet0/19

shutdown

!

interface FastEthernet0/20

shutdown

!

interface FastEthernet0/21

shutdown

!

interface FastEthernet0/22

shutdown

!

interface FastEthernet0/23

shutdown

!

interface FastEthernet0/24

shutdown

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

interface Vlan12

mac-address 000a.f30a.3501

ip address 172.16.12.1 255.255.255.0

!

interface Vlan123

mac-address 000a.f30a.3502

ip address 172.16.123.1 255.255.255.0

!

interface Vlan234

mac-address 000a.f30a.3503

ip address 172.16.234.1 255.255.255.0

!

interface Vlan1010

mac-address 000a.f30a.3504

ip address 172.16.10.1 255.255.255.0

!

interface Vlan1111

mac-address 000a.f30a.3505

ip address 172.16.11.1 255.255.255.0

!

interface Vlan3456

mac-address 000a.f30a.3506

ip address 172.16.56.1 255.255.255.0

!

ip classless

!

ip flow-export version 9

!

!

!

!

!

!

!

!

line con 0

!

line aux 0

!

line vty 0 4

login

!

!

!

ntp server 172.16.56.1

!

end

Show IP Interface Brief

Show VLAN Brief

Show VTP Status

Show etherchannel

Show spanning-tree Summary

Show standby brief