# Design of an Unsigned two Number uneven Parallel Multiplier and the design of a FSM in hardware and VHDL

# Name: Morgan jones

Student number: 1706863

Module : (CDA6000)

Course: Electric and Electronic Engineering

Contents

Name: Morgan jones	1

Different types of multipliers	3

Picture and details	3

Booth multiplier.	3

Combinational multiplier.	4

Braun Multiplier.	4

Wallace tree multiplier.	4

Array multiplier.	5

Sequential multiplier.	5

Multiplier Table with advantages	5

1 . Aims and Objectives:	6

Code screenshots with description	6

Test bench screenshots	6

2. Aims and Objectives:	6

VHDL Test Bench	9

Requirements	13

References	14

# Figures

Figure 1 The flow chart of Booth multiplication algorithm. [1]	5

Figure 2 Combinational multiplier [3]	6

Figure 3 Wallace tree multiplier [4]	7

Figure 4 array multiplier [5]	7

Figure 5 Sequential multiplier [6]	8

Figure 6 Braun multiplier [7]	9

Figure 7  state diagram	45

# Tables

Table 1 multipliers	10

Table 2	15

Table 3 line x in implementation window	22

Table 4 line 0 test bench	28

Table 5	35

Table 6	39

Table 7	44

# Introduction

This report will contain information on several multipliers and how each one works. There two separate tasks. Task one will contain code for a 4 bit by 12 bit multiplier with observations. Task 2 will contain a VHDL FSM and a discrete gate solution.

# Different types of multipliers

## Booth multiplier.

Figure 1 The flow chart of Booth multiplication algorithm. [1]

The Booth multiplier is a technique used to multiply binary numbers in signed -2’s full representation. [2]

## Combinational multiplier.

Figure 2 Combinational multiplier [3]

Combinational Multipliers are capable of doing multiplication using two unsigned binary numbers. It has also been used in the multiplication of two signed number. It works by having each part of the multiplier multiplied by a multiple and the result location is dependant upon the location of the bit inside the multiplier and the ensuing values are then combined to create the final value. [2]

## Wallace tree multiplier.

Figure 3 Wallace tree multiplier [4]

A Wallace tree multiplier is an effective physical implementation of a circuit that multiply two integers. “Wallace tree reduces the no. of partial products and use carry select adder for the addition of partial products.” [2]

## Array multiplier

Figure 4 array multiplier [5]

Array multiplier is well established because of its consistent structure. The multiplier is established upon repetitive addition and shifting. Each  partial result value is being created using the multiplication of a multiple with one figure multiplier. The partial results are altered depending on their bit sequences which are subsequently added. “The summation can be performed with normal carry propagation adder. N-1 adders are required where N is the no. of multiplier bits.” [2]

## Sequential multiplier.

Figure 5 Sequential multiplier [6]

If you need to multiply two binary numbers by using a single n bit adder, you will be able to create a sequential multiplier that handles a single partial value at a time and then cycle the multiplier a certain number of times. This type of system is called a sequential multiplier. In a sequential multiplier, the process is separated into multiple steps. In each step a value will be created, added to an amassed sum and partial sum these will automatically be shifted to line up the collected sum with partial values. Consequently, every step of the multiplier involves three distinct procedures which generate partial values, adding the partial products to the partial sum, and shifting the partial sum. [2]

## Braun multiplier

Figure 6 Braun multiplier [7]

“It is a simple parallel multiplier generally called as carry save array multiplier. It has been restricted to perform signed bits. The structure consists of array of AND gates and adders arranged in an iterative manner and no need of on n*n bit Braun multiplier is constructed with n (n-1) adders, n2 AND gates and (n-1) rows of Carry Save Adder.”  [7]

“Each value can be produced in parallel by using AND gates. Each partial value  can be added with the sum of partial product which has previously produced by using the row of adders. The carry out will shift one bit to the left or right and then it would be included in the sum which is produced by the first adder and the recently created value.” [7]

# Multiplier Table with advantages

| Multipliers | Advantages |
| --- | --- |
| Booth | regular structure easy layout small size. Quick design time ease of design for a pipelined architecture. |
| Combinational | unrolling multiplier loop carry save addition regular and modular |
| Wallace tree | small delay. Low logic levels required to perform the summation |
| Array | reduces the number of partial products |
| [2] |  |

Table 1 multipliers

# 1 . Aims and Objectives:

The aim of task 1 is to design, simulate and synthesise a 12 bit x 4 bit unsigned number multiplier using the Xilinx design software in order to achieve your aims.

## Half adder

Module Name: half_adder - Behavioral -- Project Name: -- Target Devices: -- Tool versions: -- Description: -- -- Dependencies: -- -- Revision: -- Revision 0.01 - File Created -- Additional Comments: -- ---------------------------------------------------------------------------------- library IEEE; use IEEE.STD_LOGIC_1164.ALL; -- Uncomment the following library declaration if using -- arithmetic functions with Signed or Unsigned values --use IEEE.NUMERIC_STD.ALL; -- Uncomment the following library declaration if instantiating -- any Xilinx primitives in this code. --library UNISIM; --use UNISIM.VComponents.all; entity half_adder is Port ( A : in STD_LOGIC; B : in STD_LOGIC; Sum : out STD_LOGIC; Carry : out STD_LOGIC); end half_adder; architecture Behavioral of half_adder is begin Sum <= A xor B; Carry <= A and B; end Behavioral;

Project Name: half_adder -- Target Device: -- Tool versions: -- Description: -- -- VHDL Test Bench Created by ISE for module: half_adder -- -- Dependencies: -- -- Revision: -- Revision 0.01 - File Created -- Additional Comments: -- -- Notes: -- This testbench has been automatically generated using types std_logic and -- std_logic_vector for the ports of the unit under test. Xilinx recommends -- that these types always be used for the top-level I/O of a design in order -- to guarantee that the testbench will bind correctly to the post-implementation -- simulation model. -------------------------------------------------------------------------------- LIBRARY ieee; USE ieee.std_logic_1164.ALL; -- Uncomment the following library declaration if using -- arithmetic functions with Signed or Unsigned values --USE ieee.numeric_std.ALL; ENTITY half_adder_TB IS END half_adder_TB; ARCHITECTURE behavior OF half_adder_TB IS -- Component Declaration for the Unit Under Test (UUT) COMPONENT half_adder PORT( A : IN std_logic; B : IN std_logic; Sum : OUT std_logic; Carry : OUT std_logic ); END COMPONENT; --Inputs signal A : std_logic := '0'; signal B : std_logic := '0'; --Outputs signal Sum : std_logic; signal Carry : std_logic; -- No clocks detected in port list. Replace <clock> below with -- appropriate port name BEGIN -- Instantiate the Unit Under Test (UUT) uut: half_adder PORT MAP ( A => A, B => B, Sum => Sum, Carry => Carry ); -- Stimulus process stim_proc0: process begin -- hold reset state for 100 ns. wait for 10 ns; A <= not A; end process; stim_proc1: process begin -- hold reset state for 100 ns. wait for 20 ns; B <= not B; end process; END;

Table 2

## Full adder

Module Name: full_adder - Behavioral -- Project Name: -- Target Devices: -- Tool versions: -- Description: -- -- Dependencies: -- -- Revision: -- Revision 0.01 - File Created -- Additional Comments: -- ---------------------------------------------------------------------------------- library IEEE; use IEEE.STD_LOGIC_1164.ALL; -- Uncomment the following library declaration if using -- arithmetic functions with Signed or Unsigned values --use IEEE.NUMERIC_STD.ALL; -- Uncomment the following library declaration if instantiating -- any Xilinx primitives in this code. --library UNISIM; --use UNISIM.VComponents.all; entity full_adder is Port ( A : in STD_LOGIC; B : in STD_LOGIC; C : in STD_LOGIC; Sum : out STD_LOGIC; Carry : out STD_LOGIC); end full_adder; architecture Behavioral of full_adder is signal AxB : STD_LOGIC; signal AxBnC: STD_LOGIC; signal AnB : STD_LOGIC; begin AxB <= A xor B; AxBnC <= AxB and C; AnB <= A and B; Sum <= AxB xor C; Carry <= AxBnC or AnB; end Behavioral;

Project Name: full_adder -- Target Device: -- Tool versions: -- Description: -- -- VHDL Test Bench Created by ISE for module: full_adder -- -- Dependencies: -- -- Revision: -- Revision 0.01 - File Created -- Additional Comments: -- -- Notes: -- This testbench has been automatically generated using types std_logic and -- std_logic_vector for the ports of the unit under test. Xilinx recommends -- that these types always be used for the top-level I/O of a design in order -- to guarantee that the testbench will bind correctly to the post-implementation -- simulation model. -------------------------------------------------------------------------------- LIBRARY ieee; USE ieee.std_logic_1164.ALL; -- Uncomment the following library declaration if using -- arithmetic functions with Signed or Unsigned values --USE ieee.numeric_std.ALL; ENTITY full_adder_TB IS END full_adder_TB; ARCHITECTURE behavior OF full_adder_TB IS -- Component Declaration for the Unit Under Test (UUT) COMPONENT full_adder PORT( A : IN std_logic; B : IN std_logic; C : IN std_logic; Sum : OUT std_logic; Carry : OUT std_logic ); END COMPONENT; --Inputs signal A : std_logic := '0'; signal B : std_logic := '0'; signal C : std_logic := '0'; --Outputs signal Sum : std_logic; signal Carry : std_logic; -- No clocks detected in port list. Replace <clock> below with -- appropriate port name BEGIN -- Instantiate the Unit Under Test (UUT) uut: full_adder PORT MAP ( A => A, B => B, C => C, Sum => Sum, Carry => Carry ); -- Stimulus process stim_proc: process begin -- hold reset state for 10 ns. wait for 10 ns; A <= not A; end process; stim_proc1: process begin -- hold reset state for 20 ns. wait for 20 ns; B <= not B; end process; stim_proc2: process begin -- hold reset state for 40 ns. wait for 40 ns; C <= not C; end process; END;

## Line 0

Module Name: line_0 - Behavioral -- Project Name: -- Target Devices: -- Tool versions: -- Description: -- -- Dependencies: -- -- ---------------------------------------------------------------------------------- library IEEE; use IEEE.STD_LOGIC_1164.ALL; -- Uncomment the following library declaration if using -- arithmetic functions with Signed or Unsigned values --use IEEE.NUMERIC_STD.ALL; -- Uncomment the following library declaration if instantiating -- any Xilinx primitives in this code. --library UNISIM; --use UNISIM.VComponents.all; entity line_0 is Port ( A1 : in STD_LOGIC; A2 : in STD_LOGIC; A3 : in STD_LOGIC; A4 : in STD_LOGIC; B1 : in STD_LOGIC; Q0 : out STD_LOGIC; Q1 : out STD_LOGIC; Q2 : out STD_LOGIC; Q3 : out STD_LOGIC; Q4 : out STD_LOGIC; Q5 : out STD_LOGIC; Q6 : out STD_LOGIC; Q7 : out STD_LOGIC; Q8 : out STD_LOGIC; Q9 : out STD_LOGIC; Q10 : out STD_LOGIC; Q11 : out STD_LOGIC); end line_0; architecture Behavioral of line_0 is begin Q0 <= A1 and B1; Q1 <= A1 and B1; Q2 <= A1 and B1; Q3 <= A2 and B1; Q4 <= A2 and B1; Q5 <= A2 and B1; Q6 <= A3 and B1; Q7 <= A3 and B1; Q8 <= A3 and B1; Q9 <= A4 and B1; Q10 <= A4 and B1; Q11 <= A4 and B1; end Behavioral;

The Behavioral location

Synthesis confirmed and no errors found

RTL Schematic

Table 3 line x in implementation window

multi4x12_line0/line0_TB.vhd -- Project Name: multi4x12_line0 -- Target Device: -- Tool versions: -- Description: -- -- VHDL Test Bench Created by ISE for module: line_0 -- -- Dependencies: -- -- Revision: -- Revision 0.01 - File Created -- Additional Comments: -- -- Notes: -- This testbench has been automatically generated using types std_logic and -- std_logic_vector for the ports of the unit under test. Xilinx recommends -- that these types always be used for the top-level I/O of a design in order -- to guarantee that the testbench will bind correctly to the post-implementation -- simulation model. -------------------------------------------------------------------------------- LIBRARY ieee; USE ieee.std_logic_1164.ALL; -- Uncomment the following library declaration if using -- arithmetic functions with Signed or Unsigned values --USE ieee.numeric_std.ALL; ENTITY line0_TB IS END line0_TB; ARCHITECTURE behavior OF line0_TB IS -- Component Declaration for the Unit Under Test (UUT) COMPONENT line_0 PORT( A1 : IN std_logic; A2 : IN std_logic; A3 : IN std_logic; A4 : IN std_logic; B1 : IN std_logic; Q0 : OUT std_logic; Q1 : OUT std_logic; Q2 : OUT std_logic; Q3 : OUT std_logic; Q4 : OUT std_logic; Q5 : OUT std_logic; Q6 : OUT std_logic; Q7 : OUT std_logic; Q8 : OUT std_logic; Q9 : OUT std_logic; Q10 : OUT std_logic; Q11 : OUT std_logic ); END COMPONENT; --Inputs signal A1 : std_logic := '0'; signal A2 : std_logic := '0'; signal A3 : std_logic := '0'; signal A4 : std_logic := '0'; signal B1 : std_logic := '0'; --Outputs signal Q0 : std_logic; signal Q1 : std_logic; signal Q2 : std_logic; signal Q3 : std_logic; signal Q4 : std_logic; signal Q5 : std_logic; signal Q6 : std_logic; signal Q7 : std_logic; signal Q8 : std_logic; signal Q9 : std_logic; signal Q10 : std_logic; signal Q11 : std_logic; BEGIN -- Instantiate the Unit Under Test (UUT) uut: line_0 PORT MAP ( A1 => A1, A2 => A2, A3 => A3, A4 => A4, B1 => B1, Q0 => Q0, Q1 => Q1, Q2 => Q2, Q3 => Q3, Q4 => Q4, Q5 => Q5, Q6 => Q6, Q7 => Q7, Q8 => Q8, Q9 => Q9, Q10 => Q10, Q11 => Q11 ); -- Stimulus process stim_proc: process begin -- hold reset state for 10 ns. wait for 10 ns; A1 <= not A1; end process; stim_proc2: process begin -- hold reset state for 20 ns. wait for 20 ns; A2 <= not A2; end process; stim_proc3: process begin -- hold reset state for 40 ns. wait for 40 ns; A3 <= not A3; end process; stim_proc4: process begin -- hold reset state for 40 ns. wait for 80 ns; A4 <= not A4; end process; stim_proc5: process begin -- hold reset state for 80 ns. wait for 160 ns; B1 <= not B1; end process; END;

Table 4 line 0 test bench

## Line X

Module Name: line_X - Behavioral -- Project Name: -- Target Devices: -- Tool versions: -- Description: -- -- Dependencies: -- -- Revision: -- Revision 0.01 - File Created -- Additional Comments: -- ---------------------------------------------------------------------------------- library IEEE; use IEEE.STD_LOGIC_1164.ALL; -- Uncomment the following library declaration if using -- arithmetic functions with Signed or Unsigned values --use IEEE.NUMERIC_STD.ALL; -- Uncomment the following library declaration if instantiating -- any Xilinx primitives in this code. --library UNISIM; --use UNISIM.VComponents.all; entity line_X is Port ( A1 : in STD_LOGIC; A2 : in STD_LOGIC; A3 : in STD_LOGIC; B : in STD_LOGIC; P1 : in STD_LOGIC; P2 : in STD_LOGIC; P3 : in STD_LOGIC; Q1 : out STD_LOGIC; Q2 : out STD_LOGIC; Q3 : out STD_LOGIC; Q4 : out STD_LOGIC); end line_X; architecture Behavioral of line_X is COMPONENT half_adder PORT( A : IN std_logic; B : IN std_logic; Sum : OUT std_logic; Carry : OUT std_logic ); END COMPONENT; COMPONENT full_adder PORT( A : IN std_logic; B : IN std_logic; C : IN std_logic; Sum : OUT std_logic; Carry : OUT std_logic ); END COMPONENT; signal s0 : STD_LOGIC; signal s1 : STD_LOGIC; signal s2 : STD_LOGIC; signal c1 : STD_LOGIC; signal c2 : STD_LOGIC; begin s0 <= A1 and B; s1 <= A2 and B; s2 <= A3 and B; HA1: half_adder Port map ( s0 , p1 , Q1 , c1 ); FA1 : full_adder Port map ( s1 , p2 , c1 , Q2, c2 ); FA2 : full_adder Port map ( s2 , P3 , c2 , Q3, Q4 ); end behavioral;

roject Name: line_X -- Target Device: -- Tool versions: -- Description: -- -- VHDL Test Bench Created by ISE for module: line_X -- -- Dependencies: -- -- Revision: -- Revision 0.01 - File Created -- Additional Comments: -- -- Notes: -- This testbench has been automatically generated using types std_logic and -- std_logic_vector for the ports of the unit under test. Xilinx recommends -- that these types always be used for the top-level I/O of a design in order -- to guarantee that the testbench will bind correctly to the post-implementation -- simulation model. -------------------------------------------------------------------------------- LIBRARY ieee; USE ieee.std_logic_1164.ALL; -- Uncomment the following library declaration if using -- arithmetic functions with Signed or Unsigned values --USE ieee.numeric_std.ALL; ENTITY line_X_TB IS END line_X_TB; ARCHITECTURE behavior OF line_X_TB IS -- Component Declaration for the Unit Under Test (UUT) COMPONENT line_X PORT( A1 : IN std_logic; A2 : IN std_logic; A3 : IN std_logic; B : IN std_logic; P1 : IN std_logic; P2 : IN std_logic; P3 : IN std_logic; Q1 : OUT std_logic; Q2 : OUT std_logic; Q3 : OUT std_logic; Q4 : OUT std_logic ); END COMPONENT; --Inputs signal A1 : std_logic := '0'; signal A2 : std_logic := '0'; signal A3 : std_logic := '0'; signal B : std_logic := '0'; signal P1 : std_logic := '0'; signal P2 : std_logic := '0'; signal P3 : std_logic := '0'; --Outputs signal Q1 : std_logic; signal Q2 : std_logic; signal Q3 : std_logic; signal Q4 : std_logic; -- No clocks detected in port list. Replace <clock> below with -- appropriate port name BEGIN -- Instantiate the Unit Under Test (UUT) uut: line_X PORT MAP ( A1 => A1, A2 => A2, A3 => A3, B => B, P1 => P1, P2 => P2, P3 => P3, Q1 => Q1, Q2 => Q2, Q3 => Q3, Q4 => Q4 ); -- Stimulus process stim_proc: process begin -- hold reset state for 100 ns. wait for 10 ns; A1 <= not A1; end process; -- Stimulus process stim_proc2: process begin -- hold reset state for 100 ns. wait for 20 ns; A2 <= not A2; end process; -- Stimulus process stim_proc3: process begin -- hold reset state for 100 ns. wait for 40 ns; A3 <= not A3; end process; -- Stimulus process stim_proc4: process begin -- hold reset state for 100 ns. wait for 80 ns; P1 <= not P1; end process; -- Stimulus process stim_proc5: process begin -- hold reset state for 100 ns. wait for 160 ns; P2 <= not P2; end process; stim_proc6: process begin -- hold reset state for 100 ns. wait for 320 ns; P3 <= not P3; end process; -- Stimulus process stim_proc7: process begin -- hold reset state for 100 ns. wait for 640 ns; B <= not B; end process; END;

Table 5

## Multi Full

Module Name: multi4x12_full - Behavioral -- Project Name: -- Target Devices: -- Tool versions: -- Description: -- -- Dependencies: -- -- Revision: -- Revision 0.01 - File Created -- Additional Comments: -- ---------------------------------------------------------------------------------- library IEEE; use IEEE.STD_LOGIC_1164.ALL; -- Uncomment the following library declaration if using -- arithmetic functions with Signed or Unsigned values --use IEEE.NUMERIC_STD.ALL; -- Uncomment the following library declaration if instantiating -- any Xilinx primitives in this code. --library UNISIM; --use UNISIM.VComponents.all; entity multi4x12_full is Port ( A1 : in STD_LOGIC; A2 : in STD_LOGIC; A3 : in STD_LOGIC; B1 : in STD_LOGIC; B2 : in STD_LOGIC; B3 : in STD_LOGIC; Q0 : out STD_LOGIC; Q1 : out STD_LOGIC; Q2 : out STD_LOGIC; Q3 : out STD_LOGIC; Q4 : out STD_LOGIC; Q5 : out STD_LOGIC); end multi4x12_full; architecture Behavioral of multi4x12_full is COMPONENT line_0 PORT( A1 : IN std_logic; A2 : IN std_logic; A3 : IN std_logic; A4 : IN std_logic; B1 : IN std_logic; Q0 : OUT std_logic; Q1 : OUT std_logic; Q2 : OUT std_logic; Q3 : OUT std_logic; Q4 : OUT std_logic; Q5 : OUT std_logic; Q6 : OUT std_logic; Q7 : OUT std_logic; Q8 : OUT std_logic; Q9 : OUT std_logic; Q10 : OUT std_logic; Q11 : OUT std_logic ); END COMPONENT; COMPONENT line_X PORT( A1 : IN std_logic; A2 : IN std_logic; A3 : IN std_logic; B : IN std_logic; P1 : IN std_logic; P2 : IN std_logic; P3 : IN std_logic; Q1 : OUT std_logic; Q2 : OUT std_logic; Q3 : OUT std_logic; Q4 : OUT std_logic ); END COMPONENT; signal p1_1 : STD_LOGIC; signal p1_2 : STD_LOGIC; signal p2_1 : STD_LOGIC; signal p2_2 : STD_LOGIC; signal p2_3 : STD_LOGIC; begin line1 : line_0 Port Map ( A1, A2 , A3 , B1 , Q0 , p1_1 , p1_2 ); line2 : line_X Port Map ( A1 , A2 , A3 , B2 , p1_1 , p1_2 , P3 , Q1 , p2_1 , p2_2 , p2_3 ); line3 : line_X Port Map ( A1 , A2 , A3 , B3 , p2_1 , p2_2 , p2_3, Q2 , Q3 ,Q4 , Q5 ); end Behavioral;

Table 6

Project Name: multi4x12_full -- Target Device: -- Tool versions: -- Description: -- -- VHDL Test Bench Created by ISE for module: multi4x12_full -- -- Dependencies: -- -- Revision: -- Revision 0.01 - File Created -- Additional Comments: -- -- Notes: -- This testbench has been automatically generated using types std_logic and -- std_logic_vector for the ports of the unit under test. Xilinx recommends -- that these types always be used for the top-level I/O of a design in order -- to guarantee that the testbench will bind correctly to the post-implementation -- simulation model. -------------------------------------------------------------------------------- LIBRARY ieee; USE ieee.std_logic_1164.ALL; -- Uncomment the following library declaration if using -- arithmetic functions with Signed or Unsigned values --USE ieee.numeric_std.ALL; ENTITY full_TB IS END full_TB; ARCHITECTURE behavior OF full_TB IS -- Component Declaration for the Unit Under Test (UUT) COMPONENT multi4x12_full PORT( A1 : IN std_logic; A2 : IN std_logic; A3 : IN std_logic; B1 : IN std_logic; B2 : IN std_logic; B3 : IN std_logic; Q0 : OUT std_logic; Q1 : OUT std_logic; Q2 : OUT std_logic; Q3 : OUT std_logic; Q4 : OUT std_logic; Q5 : OUT std_logic ); END COMPONENT; --Inputs signal A1 : std_logic := '0'; signal A2 : std_logic := '0'; signal A3 : std_logic := '0'; signal B1 : std_logic := '0'; signal B2 : std_logic := '0'; signal B3 : std_logic := '0'; --Outputs signal Q0 : std_logic; signal Q1 : std_logic; signal Q2 : std_logic; signal Q3 : std_logic; signal Q4 : std_logic; signal Q5 : std_logic; -- No clocks detected in port list. Replace <clock> below with -- appropriate port name BEGIN -- Instantiate the Unit Under Test (UUT) uut: multi4x12_full PORT MAP ( A1 => A1, A2 => A2, A3 => A3, B1 => B1, B2 => B2, B3 => B3, Q0 => Q0, Q1 => Q1, Q2 => Q2, Q3 => Q3, Q4 => Q4, Q5 => Q5 ); -- Stimulus process stim_proc: process begin -- hold reset state for 100 ns. wait for 10 ns; A1 <= not A1; end process; stim_proc2: process begin -- hold reset state for 100 ns. wait for 20 ns; A2 <= not A2; end process; stim_proc3: process begin -- hold reset state for 100 ns. wait for 40 ns; A3 <= not A3; end process; stim_proc4: process begin -- hold reset state for 100 ns. wait for 80 ns; B1 <= not B1; end process; stim_proc5: process begin -- hold reset state for 100 ns. wait for 160 ns; B2 <= not B2 ; end process; stim_proc6: process begin -- hold reset state for 100 ns. wait for 320 ns; B3 <= not B3; end process; END;

Table 7

# 2. Aims and Objectives:

The task 2 is to design, simulate and synthesise the FSM described by the state diagram in figure 4 as both a VHDL and discrete gate solution.

Figure 7  state diagram

# Discrete gate solution

|  | State |  |  |  |  |  | Next state |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| state | A | B | C | X |  | A | B | C | Z |
| A | 0 | 0 | 0 | 0 |  | 1 | 0 | 0 |  |
|  | 0 | 0 | 0 | 1 |  | 0 | 0 | 1 |  |
| B | 0 | 0 | 1 | 0 |  | 0 | 1 | 1 |  |
|  | 0 | 0 | 1 | 1 |  | 0 | 1 | 0 |  |
| C | 0 | 1 | 0 | 0 |  | 1 | 1 | 0 |  |
|  | 0 | 1 | 0 | 1 |  | 0 | 1 | 1 |  |
| D | 0 | 1 | 1 | 0 |  | 1 | 1 | 0 |  |
|  | 0 | 1 | 1 | 1 |  | 1 | 0 | 1 |  |
| E | 1 | 0 | 0 | 0 |  | 1 | 0 | 1 |  |
|  | 1 | 0 | 0 | 1 |  | 0 | 1 | 1 |  |
| F | 1 | 0 | 1 | 0 |  | 1 | 1 | 1 |  |
|  | 1 | 0 | 1 | 1 |  | 0 | 1 | 0 |  |
| G | 1 | 1 | 0 | 0 |  | 1 | 1 | 0 |  |
|  | 1 | 1 | 0 | 1 |  | 0 | 0 | 1 |  |
| H | 1 | 1 | 1 | 0 |  | 1 | 1 | 0 |  |
|  | 1 | 1 | 1 | 1 |  | 0 | 0 | 0 | 1 |
|  |  | AB |  |  |  |
|  | Da | _00 | _01 | 11 | 10 |
| CX | _00 | 1 | 1 | 1 | 1 |
|  | _01 | 0 | 0 | 0 | 0 |
|  | 11 | 0 | 1 | 0 | 0 |
|  | 10 | 0 | 1 | 1 | 1 |

Da = /C/X + B/X + A/X + /A.B.C

|  |  | AB |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Db | _00 | _01 | 11 | 10 |
| CX | _00 | 0 | 1 | 1 | 0 |
|  | _01 | 0 | 1 | 0 | 1 |
|  | 11 | 1 | 0 | 0 | 1 |
|  | 10 | 1 | 1 | 1 | 1 |

Db =B/X + /B. C + /A. B/ C + A /B. X

|  |  |  | AB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Dc |  | _00 | _01 | 11 | 10 |
| CX | _00 |  | 0 | 0 | 0 | 1 |
|  | _01 |  | 1 | 1 | 1 | 1 |
|  | 11 |  | 0 | 1 | 0 | 0 |
|  | 10 |  | 1 | 0 | 0 | 1 |

Dc = /C.X+ /B.C/X+A/B/X+/A.B.X

# FSM

## VHDL Module

Module Name: fm1 - Behavioral ---------------------------------------------------------------------------------- library IEEE; use IEEE.STD_LOGIC_1164.ALL; -- Uncomment the following library declaration if using -- arithmetic functions with Signed or Unsigned values --use IEEE.NUMERIC_STD.ALL; -- Uncomment the following library declaration if instantiating -- any Xilinx primitives in this code. --library UNISIM; --use UNISIM.VComponents.all; entity fm1 is Port ( x : in STD_LOGIC; Clk : in STD_LOGIC; Z : out STD_LOGIC); end fm1; architecture Behavioral of fm1 is type state_type is (state_A, state_B, state_C, state_D, state_E, state_F, state_G, state_H); signal present_state : state_type := state_A; signal next_state : state_type := state_A; begin State: process (present_state,X) begin if x='0' Then case present_state is when state_A => next_state <= state_E;Z<='0'; when state_B => next_state <= state_D;Z<='0'; when state_C => next_state <= state_G;Z<='0'; when state_D => next_state <= state_G;Z<='0'; when state_E => next_state <= state_D;Z<='0'; when state_F => next_state <= state_C;Z<='0'; when state_G => next_state <= state_G;Z<='0'; when state_H => next_state <= state_A;Z<='0'; end case; else if x='1' Then case present_state is when state_A => next_state <= state_B;Z<='0'; when state_B => next_state <= state_C;Z<='0'; when state_C => next_state <= state_D;Z<='0'; when state_D => next_state <= state_F;Z<='0'; when state_E => next_state <= state_D;Z<='0'; when state_F => next_state <= state_C;Z<='0'; when state_G => next_state <= state_B;Z<='0'; when state_H => next_state <= state_G;Z<='1'; when OTHERS => NULL; end case; end if; end if; end Process state; clocked: Process (Clk) begin if ((Clk='1') and (Clk'last_value='0')) Then present_state <= next_state; end if; end process clocked; end Behavioral;

## VHDL Test Bench

VHDL Test Bench Created by ISE for module: fm1 -- -- Dependencies: -- -- Revision: -- Revision 0.01 - File Created -- Additional Comments: -- -- Notes: -- This testbench has been automatically generated using types std_logic and -- std_logic_vector for the ports of the unit under test. Xilinx recommends -- that these types always be used for the top-level I/O of a design in order -- to guarantee that the testbench will bind correctly to the post-implementation -- simulation model. -------------------------------------------------------------------------------- LIBRARY ieee; USE ieee.std_logic_1164.ALL; -- Uncomment the following library declaration if using -- arithmetic functions with Signed or Unsigned values --USE ieee.numeric_std.ALL; ENTITY fm1_TB IS END fm1_TB; ARCHITECTURE behavior OF fm1_TB IS -- Component Declaration for the Unit Under Test (UUT) COMPONENT fm1 PORT( x : IN std_logic; Clk : IN std_logic; Z : OUT std_logic ); END COMPONENT; --Inputs signal x : std_logic := '0'; signal Clk : std_logic := '0'; --Outputs signal Z : std_logic; -- Clock period definitions constant Clk_period : time := 10 ns; BEGIN -- Instantiate the Unit Under Test (UUT) uut: fm1 PORT MAP ( x => x, Clk => Clk, Z => Z ); -- Clock process definitions Clk_process :process begin Clk <= '0'; wait for Clk_period/2; Clk <= '1'; wait for Clk_period/2; end process; -- Stimulus process stim_proc: process begin -- hold reset state for 100 ns. wait for 100 ns; wait for Clk_period*10; -- insert stimulus here wait; end process; END;

# References

| [1] | “The flow chart of Booth multiplication algorithm.,” [Online]. Available: https://www.researchgate.net/figure/The-flow-chart-of-Booth-multiplication-algorithm_fig3_308574775. [Accessed March 2021]. |
| --- | --- |
| [2] | “A Review ofDifferent Type of Multipliers and Multiplier-Accumulator Unit.,” International Journal of Emerging Trends & Technology in Computer Science, [Online]. Available: https://www.ijettcs.org/Volume2Issue4/IJETTCS-2013-08-25-115.pdf. [Accessed February 2021]. |
| [3] | “Combinational multiplier diagram,” [Online]. Available: https://ocw.mit.edu/courses/electrical-engineering-and-computer-science/6-004-computation-structures-spring-2017/c8/c8s1/Slide18.png. [Accessed March 2021]. |
| [4] | [Online]. Available: https://www.google.com/url?sa=i&url=http%3A%2F%2Fwww.ijarse.com%2Fimages%2Ffullpdf%2F1523964041_IIMT19.pdf&psig=AOvVaw2Gd5nypjDnAZfAN9aHYCfe&ust=1621505354063000&source=images&cd=vfe&ved=0CAkQjhxqFwoTCKjnu6rA1fACFQAAAAAdAAAAABAD. |
| [5] | [Online]. Available: https://www.google.com/url?sa=i&url=http%3A%2F%2Fpeople.ee.duke.edu%2F~krish%2Fteaching%2FLectures%2FArithmeticCircuits_2.pdf&psig=AOvVaw1GLR9GXnlJnoVE6osA5nxb&ust=1621505455363000&source=images&cd=vfe&ved=0CAkQjhxqFwoTCKCrstnA1fACFQAAAAAdAAAAABAD. |
| [6] | [Online]. Available: https://www.google.com/url?sa=i&url=https%3A%2F%2Fdigitalsystemdesign.in%2Fsequential-multiplier%2F&psig=AOvVaw3dRFPhvls3qvU0n2S4nHXC&ust=1621506144808000&source=images&cd=vfe&ved=0CAkQjhxqFwoTCOC46qfD1fACFQAAAAAdAAAAABBL. |
| [7] | [Online]. Available: https://www.ijser.org/paper/Design-of-Braun-Multiplier-with-Kogge-Stone-Adder-Its-Implementation-on-FPGA.html. [Accessed May 2021]. |