UWTSD

Embedded C Assignment

Proteus Programme

Louise Caygill (2110765) 03/11/2022

//AUTHOR:	L. Caygill

//DATE:		October 2022

//DESCRIPTION:	Code designed to run 4 different scenarios depending on the position of SW1

//		Position A: Shows the value of RV1 on the LEDs, LCD screen and virtual terminal

//		Position B: Uses the value of RV1 to drive the motor clockwise, anti-clockwise or hold it stationary, prints the value of RV1 and direction of spin on LCD screen and virtual terminal

//		Position C: The motor rotates clockwise so long as the button is pressed, and prints the button status to the LCD and virtual terminal

//		Position D: LEDs flash left to right with 150ms intervals a la Knight Rider. Displays "Kitt Mode" on the LCD and virtual terminal

#include <main.h>

//--------------------------------------------------------------------------------------//DEFINE PINS

#define LCD_ENABLE_PIN PIN_D2

#define LCD_RS_PIN PIN_D0

#define LCD_RW_PIN PIN_D1

#define LCD_DATA4 PIN_D4

#define LCD_DATA5 PIN_D5

#define LCD_DATA6 PIN_D6

#define LCD_DATA7 PIN_D7

#define SW_A PIN_A2

#define SW_B PIN_A3

#define SW_C PIN_A4

#define SW_D PIN_A5

#define BUTTON PIN_A1

//--------------------------------------------------------------------------------------//MOTOR PINS

#define M_A PIN_C0

#define M_B PIN_C1

#include <lcd.c>

//--------------------------------------------------------------------------------------//DEFINE FUNCTIONS

void analog(void);									//READ VALUE FROM RV1

void SwitchA(void);									//PRINT VALUE OF RV1 ON THE SCREEN AND ON LEDs

void SwitchB();										//DRIVE MOTOR WITH POT INPUT

void SwitchC();										//SPIN MOTOR CLOCKWISE INDEFINITELY WHILST BUTTON HELD ON SWITCH POSITION C

void SwitchD();										//KNIGHT RIDER ROUTINE

void BinaryLEDs();									//TRANSLATE RV1 INTO BINARY ROUTINE FOR LEDs

unsigned int adc;									// declare global variables

char direction[14];									//SET direction AS OUTPUT WITH 14 CHARACTERS

//--------------------------------------------------------------------------------------------------

//--------------------------------------------------------------------------------------//MAIN CODE

void main()

{

setup_adc_ports(sAN0);									// initialise adc channel

setup_adc(ADC_CLOCK_INTERNAL|ADC_TAD_MUL_0);						// setup adc operation

lcd_init();										// initialise the LCD

while(TRUE)										//MAIN WHILE LOOP

{

analog();										// read the analog input

SwitchA();									//RUN FUNCTION FOR SWITCH POSITION A

SwitchB();									//RUN FUNCTION FOR SWITCH POSITION B

SwitchC();									//RUN FUNCTION FOR SWITCH POSITION C

SwitchD();									//RUN FUNCTION FOR SWITCH POSITION D

}											//MAIN WHILE LOOP END

}											//PROGRAM END

//--------------------------------------------------------------------------------------------------

//--------------------------------------------------------------------------------------//FUNCTIONS

//--------------------------------------------------------------------------------------------------

//--------------------------------------------------------------------------------------//SWITCH A POSITION

void SwitchA(void)									//DISPLAY VALUE OF RV1 ON LCD & VIRTUAL TERMINAL + SHOW BINARY VALUE OF RV1 ON LEDS

{

if(input(SW_A))

{

BinaryLEDs();								//CALLS FUNCTION TO SHOW VALUE OF RV1 ON LEDs IN BINARY

printf(lcd_putc,"\f");							//CLEAR SCREEN

lcd_gotoxy(6,1);								//SET POSITION OF TEXT

printf("\fRV1 = %u", adc);							//CLEAR TERMINAL AND PRINT RV1 VALUE

printf(lcd_putc,"RV1 = %u", adc);						//PRINT RV1 VALUE TO LCD

output_low(M_A);								//STOP THE MOTOR TURNING WHEN RETURNING FROM SWITCH B

output_low(M_B);

delay_ms(250);								//DELAY TO SLOW THE SPAMMING OF THE TERMINAL

}

}

//--------------------------------------------------------------------------------------------------

//--------------------------------------------------------------------------------------//SWITCH B POSITION

void SwitchB()										//CONTROL MOTOR DIRECTION WITH POT

{

if(input(SW_B))									//CHECK IF INPUT FROM SWITCH B

{

printf(lcd_putc,"\f");							//CLEAR SCREEN

lcd_gotoxy(1,2);	 							//SET POSITION ON LCD SCREEN, NO SPACE BETWEEN VALUES TO FIT "ANTI-CLOCKWISE" ON ONE LINE

printf(lcd_putc,"RV1=%u%s", adc, direction);					//PRINT RV1 VALUE TO LCD SCREEN + DIRECTION

printf("\fRV1=%u %s", adc, direction);					//CLEAR TERMINAL AND PRINT RV1 VALUE + DIRECTION

delay_ms(250);								//DELAY 250uS IN ORDER TO MAKE SCREENS LEGIBLE

if(adc>175)									//RV1 IS GREATER THAN 175 - MOTOR SPINS ANTI-CLOCKWISE

{

direction = "AntiClockwise";

output_low(M_A);

output_high(M_B);

}

if(adc<75)									//RV1 IS LESS THAN 75 - MOTOR SPINS CLOCKWISE

{

direction = " Clockwise";							//SPACE BETWEEN QUOTE MARKS AND WORD TO SPACE OUT ON SCREEN

output_high(M_A);

output_low(M_B);

}

if(adc>76&&adc<174)								//RV1 IS BETWEEN 75 AND 175 - MOTOR IS STATIONARY

{

direction = " Stationary";							//SPACE BETWEEN QUOTE MARKS AND WORD TO SPACE OUT ON SCREEN

output_low(M_A);

output_low(M_B);

}

}

}

//--------------------------------------------------------------------------------------------------

//--------------------------------------------------------------------------------------//SWITCH C POSITION

void SwitchC()										//SPIN MOTOR CLOCKWISE INDEFINITELY WHILST BUTTON HELD ON SWITCH POSITION C

{

if(input(SW_C))									//CHECK IF INPUT FROM SWITCH C

{

if(!input(BUTTON)&&(input(SW_C)))						//INPUT FROM SWITCH C, BUT NOT BUTTON

{

printf(lcd_putc,"\f");							//CLEAR SCREEN

lcd_gotoxy(24,1);							//SET POSITION OF TEXT

printf(lcd_putc,"MOTOR INACTIVE");					//PRINT BUTTON STATUS ON LCD SCREEN

printf("\fMOTOR INACTIVE");						//CLESR TERMINAL AND PRINT BUTTON STATUS

while(!input(BUTTON)&&(input(SW_C)))					//MOTOR CONTROL FOR INPUT FROM SWITCH C, BUT NO BUTTON PRESS

{

output_low(M_A);

output_low(M_B);

}

}

if(input(BUTTON)&&(input(SW_C)))						//BUTTON PRESSED AND SWITCH IN POSITION C

printf(lcd_putc,"\f");							//CLEAR SCREEN

lcd_gotoxy(25,1,);							//SET POSITION OF TEXT

printf(lcd_putc,"MOTOR ACTIVE");					//PRINT BUTTON STATUS ON LCD SCREEN

printf("\fMOTOR ACTIVE");						//CLEAR TERMINAL AND PRINT BUTTON STATUS

{

while(input(BUTTON)&&(input(SW_C)))					//MOTOR CONTROL FOR INPUT FROM SWITCH C, WITH BUTTON PRESSED

{

output_high(M_A);

output_low(M_B);

}

}

}

}

//--------------------------------------------------------------------------------------------------

//--------------------------------------------------------------------------------------//SWITCH D POSITION

void SwitchD()										//KNIGHT RIDER ROUTINE (I'M SORRY I DON'T KNOW HOW TO SHORTEN IT...)

{

if(input(SW_D))

{

printf(lcd_putc,"\f");								//CLEAR SCREEN

lcd_gotoxy(24,2);								//SET POSITION OF TEXT

printf(lcd_putc,"**KITT MODE**");						//PRINT ON LCD SCREEN

printf("\f**KITT MODE**");							//CLEAR TERMINAL AND PRINT STATUS

while(input(SW_D))

{

output_high(PIN_B7);    delay_ms(150);    output_low(PIN_B7);		//LEDs LEFT TO RIGHT

output_high(PIN_B6);    delay_ms(150);    output_low(PIN_B6);

output_high(PIN_B5);    delay_ms(150);    output_low(PIN_B5);

output_high(PIN_B4);    delay_ms(150);    output_low(PIN_B4);

output_high(PIN_B3);    delay_ms(150);    output_low(PIN_B3);

output_high(PIN_B2);    delay_ms(150);    output_low(PIN_B2);

output_high(PIN_B1);    delay_ms(150);    output_low(PIN_B1);

output_high(PIN_B0);    delay_ms(150);    output_low(PIN_B0);

output_high(PIN_B1);    delay_ms(150);    output_low(PIN_B1);		//LEDs RIGHT TO LEFT

output_high(PIN_B2);    delay_ms(150);    output_low(PIN_B2);

output_high(PIN_B3);    delay_ms(150);    output_low(PIN_B3);

output_high(PIN_B4);    delay_ms(150);    output_low(PIN_B4);

output_high(PIN_B5);    delay_ms(150);    output_low(PIN_B5);

output_high(PIN_B6);    delay_ms(150);    output_low(PIN_B6);

output_high(PIN_B7);    delay_ms(150);    output_low(PIN_B7);

}

}

}

//--------------------------------------------------------------------------------------------------

//--------------------------------------------------------------------------------------//READ VALUE OF RV1 - ALREADY IN PROGRAMME FROM GARY

void analog(void)

{

set_adc_channel(0);								//read adc channel 0

delay_us(30);									// wait 30uS

adc = read_adc();								// store value in variable ADC

}

//--------------------------------------------------------------------------------------------------

//--------------------------------------------------------------------------------------//TRANSLATE VALUE OF RV1 INTO BINARY TO DISPLAY ON LEDs

void BinaryLEDs()									//https://create.arduino.cc/projecthub/Madhur_Bajpai/binary-counter-using-leds-2089d9

{

if(adc & 1)

output_high(PIN_B0);

if(!(adc & 1))

output_low(PIN_B0);

if(adc & 2)

output_high(PIN_B1);

if(!(adc & 2))

output_low(PIN_B1);

if(adc & 4)

output_high(PIN_B2);

if(!(adc & 4))

output_low(PIN_B2);

if(adc & 8)

output_high(PIN_B3);

if(!(adc & 8))

output_low(PIN_B3);

if(adc & 16)

output_high(PIN_B4);

if(!(adc & 16))

output_low(PIN_B4);

if(adc & 32)

output_high(PIN_B5);

if(!(adc & 32))

output_low(PIN_B5);

if(adc & 64)

output_high(PIN_B6);

if(!(adc & 64))

output_low(PIN_B6);

if(adc & 128)

output_high(PIN_B7);

if(!(adc & 128))

output_low(PIN_B7);

}

//--------------------------------------------------------------------------------------------------

//--------------------------------------------------------------------------------------//END FUNCTIONS