Advanced networking lab 3

Fill the tables with appropriate information/ details.

| Specification | Network: A | Network: B | Network: C |
| --- | --- | --- | --- |
| Network Address | 192.168.100.0 | 192.168.100.96 | 192.168.100.64 |
| Subnet Mask | 255.255.255.192 | 255.255.255.252/30 | 255.255.255.224/27 |
| No. of asked hosts/subnet | 50 | 2 | 2 0 |
| No. of usable hosts/ subnet | 62 | 2 | 30 |
| First Usable Host (FUH) | 192.168.100.1 | 192.168.100.97 | 192.168.100.65 |
| Last Usable Host (LUH) | 192.168.100.62 | 192.168.100.98 | 192.168.100.94 |
| Broadcast Address | 192.168.100.63 | 192.168.100.99 | 192.168.100.95 |

| Addressing table |  |  |  |  |
| --- | --- | --- | --- | --- |
| Device | Requirement | IP address | Subnet Mask | Gateway |
| PC1 | 1st Usable Host Network: A | 192.168.100.1 | 255.255.255.192 | 192.168.100.62 |
| Laptop1 | 2nd Usable Host Network: A | 192.168.100.2 | 255.255.255.192 | 192.168.100.62 |
| PC2 | 1st Usable Host Network: B | 192.168.100.97 | 255.255.255.252 | 192.168.100.98 |
| PC3 | 1st Usable Host Network: C | 192.168.100.65 | 255.255.255.224 | 192.168.100.94 |
| Laptop0 | 2nd Usable Host Network: C | 192.168.100.66 | 255.255.255.224 | 192.168.100.94 |
| RAI G0/0/0 | Last Usable Host Network: A | 192.168.100.62 | 255.255.255.192 | N/A |
| RIBC G0/0/1 | Last Usable Host Network: B | 192.168.100.98 | 255.255.255.252 | N/A |
| RIBC G0/0/0 | Last Usable Host Network: C | 192.168.100.94 | 255.255.255.224 | N/A |

Initialise All the switches

Delete the old VLAN Database in All switches (S0, S1 &SW2) and reload

Erase startup-config

Reload Switch

Show Flash

Delete.vlan.dat

Sh flash – confirm that the file has been deleted

Reload

Check the switches with the appropriate command to find the old vlan database is deleted.

Sh Vlan (The only vlans should be in the database is 1002 to 1005 as legacy vlans) You are able to create up to 1001 vlans and then the extended vlan database ranges from 1006 to 4094 and then also the native vlan which is 1

Line console password – ciscocon

Line VTY password – ciscovty

Username – admin – password: cisco1234