Main device

#include <ESP8266WiFi.h>

#include <ESP8266WiFiMulti.h>

#include <MQUnifiedsensor.h>

ESP8266WiFiMulti WiFisetup;// Variable for WIFI connection

const int echoPin = 5;//Defines the digital pin for the ultrasonic echo

const int trigPin = 4;//defines the digital pin for the ultrasonic trigger

const int speedpinent = 0;

int speedpinentval, speedpinexitval;

const int speedpinexit = 2;

const int led1 = 13;

const int led2 = 15;

long duration;

int distance;

int distancecount = 0;

unsigned long time1, time2;

int timeresult;

float vspeed;

int timeout, counter;

#define pin A0

#define type 135

float CO, CO2;

MQUnifiedsensor MQ135(pin, type);

void setup() {

// put your setup code here, to run once:

pinMode(trigPin, OUTPUT); // Sets the trigPin as an Output for the ultrasonic sensor

pinMode(echoPin, INPUT); // Sets the echoPin as an Input for the ultrasonic sensor

pinMode(speedpinent, INPUT_PULLUP);//enterance speed

pinMode(speedpinexit, INPUT_PULLUP);//exit speed

pinMode(led1, OUTPUT);//turns on led 1

pinMode(led2, OUTPUT);//turns on led 2

Serial.begin(115200);

delay(15);//delay of 15 microseconds

WiFi.mode(WIFI_STA);//Wifi mote client

WiFisetup.addAP("ESP-588643", "");// This connectes to the wifi access point.

Serial.println();

Serial.print("Initialising connection to Access Point..... ");

while (WiFisetup.run() != WL_CONNECTED)// While the connection is not equal to WL_CONNECTED print '.' and wait 500ms.

{

Serial.print(".");//print ,

delay(500);//delay of 500 microseconds

}

Serial.println("");

Serial.println("Access point connected");

Serial.println("IP address: ");//print the ip address of the device

Serial.println(WiFi.localIP());

delay(500);

MQ135.inicializar();

digitalWrite(led1, LOW);//sets led to low

digitalWrite(led1, HIGH);//sets lef to high

digitalWrite(led2, LOW);//sets led to low

digitalWrite(led2, HIGH);//sets led to high

}

void loop()

{

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

// Sets the trigPin on HIGH state for 10 micro seconds

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

// Reads the echoPin, returns the sound wave travel time in microseconds

duration = pulseIn(echoPin, HIGH);

// Calculating the distance

distance= duration*0.034/2;

// Prints the distance on the Serial Monitor

Serial.print("Distance: ");

Serial.println(distance);

delay(10);

if(distance < 3)// when the distance between the ultrasonic sensor and the floor is les than 3

{

Serial.println("Add +1 to counter");

distancecount = distancecount + 1;// when a vehicle is detected add 1 to the counter

Serial.println(distancecount);

speedpinentval = digitalRead(speedpinent);//set the pin to digital read

speedpinexitval = digitalRead(speedpinexit);//set the pin to digital read

while(speedpinentval != 1)

{

speedpinentval = digitalRead(speedpinent);

Serial.println(speedpinentval);

Serial.println("Waiting for read 1");

delay(1);// delay of 1 microseconds is required to stop the system from crashing

timeout++;

if(timeout >= 10000)//if there is a delay of more than 10000 microseconds, break out of the loop

{

break;

}

}

Serial.println("Sensor tripped 1");

time1 = micros();

timeout = timeout - timeout;//set the timeout to 0

while(speedpinexitval != 1)

{

speedpinexitval = digitalRead(speedpinexit);

Serial.println("Waiting for read 2");

delay(1);

timeout++;

if(timeout >= 10000)

{

break;

}

}

Serial.println("Sensor tripped 2");

time2 = micros();

timeout = timeout - timeout;

timeresult = time2 - time1;

vspeed = 100000.0 / (float) timeresult;

Serial.println(vspeed, 4);

Serial.print("M/s");

delay(5000);

}

//GAS sensor detecting CO and CO2.

MQ135.update();

CO =  MQ135.readSensor("CO");

CO2 =  MQ135.readSensor("CO2");

Serial.print("CO2: ");Serial.print(CO2,2);Serial.println(" ppm");

Serial.print("CO: ");Serial.print(CO,2);Serial.println(" ppm");

Serial.println("loop.......................................");

delay(100);

if(CO2 >= 700)//if the polution is over a certain thereshold reduce the speed of the traffic

{

Serial.print("Carbon Dioxide levels too high! Reduce Speed by 10");

}

if(CO >= 1500)//if the polution is over a certain thereshold reduce the speed of the traffic

{

Serial.print("Carbon Monoxide levels too high! Reduce Speed by 10");

}

////the results will then be sent to a database every 10 cars that pass

}

Access point

#include <ESP8266WiFi.h>

#include <WiFiClient.h>

#include <ESP8266WebServer.h>

/* Set these to your desired credentials. */

const char *ssid = "PoliceVan";

const char *password = "12345";

ESP8266WebServer server(80);

/* Just a little test message.  Go to http://192.168.4.1 in a web browser

connected to this access point to see it.

*/

void handleRoot() {

server.send(200, "text/html", "<h1>You are connected</h1>");

}

void setup() {

delay(1000);

Serial.begin(9600);

Serial.println();

Serial.print("Configuring access point...");

/* You can remove the password parameter if you want the AP to be open. */

WiFi.softAP(ssid, password);

IPAddress myIP = WiFi.softAPIP();

Serial.print("AP IP address: ");

Serial.println(myIP);

server.on("/", handleRoot);

server.begin();

Serial.println("HTTP server started");

Serial.println(ssid);

Serial.println(password);

}

void loop() {

server.handleClient();

}