Basic Command List

Always use cisco or class as password

? 	Help

Tab	autofill

Three modes of opearation

Router> 		User mode

Router#		Privilege  user exec mode

Router(config)#		global config mode

Would like to enter initial configuration mode[yes/no]: no

If you entered yes to above question press Ctrl+c at any time to exit.

Commands

Router>?						Help

Router>sh? 						List you commands with sh

Router>enable or en					user to privliege exec  mode

Router#show running-config or sh run			to view running config

Router#show startup-config or sh start			to view start config

Router#show flash: 					to view flash memory

Router#copy running-config  startup-config		to copy current config into NVRAM

Router#copy run start					to copy current config into NVRAM

Router#wr						to copy current config into NVRAM

Router#copy startup-config  running-config		to retrieve NVRAM contant

Router#erase startup					to erase NVRAM

Router#reload						to restart the router

Router#show ip route					to view routing table

Router#show ip interface brief				to view interface in brief

Router#show ip interface <int no>			to view interface in detail

Router#show controllers  interface <int no>		to view DCE/DTE interface

Router#debug  ip <>					to troubleshoot

Router#undebug all or u all				to stop all debugging

Router#configure terminal or conf t			to enter golbal config mode

Router(config)# hostname ####				to set host name

Router(config)# enable password ______		to set clear password

Router(config)# enable secret ______			to set encrypt  password

Router(config)#no ip domain lookup			disable dns lookup

Router(config)# ip routing				enable routing

Router(config)# banner motd				to set Banner

Router(config)#line con 0				to set console password

Router(config-line)#login

Router(config-line)#password cisco

Router(config-line)#exit

Router(config)#line vty  0 4				to set vty line password

Router(config-line)#login

Router(config-line)#password cisco

Router(config-line)#exit

Router(config)#interface Serial 0 or (0/0)		to configure serial interface

Router(config-int)# ip address A.A.A.A M.M.M.M

Router(config-int)# line description <your description>

Router(config-int)#clockrate 64000			Only at DCE end

Router(config-int)#no shutdown

Router(config-int)#exit

Router(config)#interface ethernet0 or fastethernet0/0	to configure ethernet interface

Router(config-int)# ip address A.A.A.A M.M.M.M

Router(config-int)# line description <your description>

Router(config-int)#no shutdown

Router(config-int)#exit

If you have done a mistake and want to remove the configuration, add NO in front.

Example: Router(config)# hostname R1

R1(config)# no hostname R1

Router(config)#

To configure static router

Router(config)# ip route N.N.N.N  M.M.M.M (destination network) S0/0 (local interface) or Next hop ip address

To configure routing protocol RIP v1

Router(config)# router rip

Router(config-router)#network N.N.N.N

Router(config-router)# exit

To configure routing protocol RIP v2

Router(config)# router rip

Router(config)# version 2

Router(config-router)#network N.N.N.N

Router(config-router)# exit