MDARZINS ID 1909722

Lab 4

Task 1

PC0 = 192.168.72.30

PC1 = 192.168.72.40

PC2 = 192.168.72.50

PC3 = 192.168.72.70

All with subnet mask = 255.255.255.0

Ipconfig & ping to all PC’s successful.

Qu 2 Give HOST a name

Change HOST name

At Config tab change Hostname to SwTMyFirstSwitch

Qu. 3 Passwords

On Switch click select Command Line CLI

Place cursor in IOS Command Line Interface box     and then press ENTER

At Switch>type    enable    then press ENTER

Switch (hatch) appears

Type    config t    and then press ENTER

NOTE: config t NOT required if HOSTNAME changed in Config box

Now Switch (config) hatch appears

Type    Line console 0   and then press ENTER

Now Switch (config-line) hatch appears

Type   password cisco123   and then press ENTER

At next line/hatch type   login   and then press ENTER

At next line/hatch type     Exit   and then press ENTER

Config vty password

Same process as above, just change details to

Line vty 0 15

Password 1CiSc3

Login

Exit

Enable privileged password

Type                        enable password cisco    a         nd press ENTER

Once fully exited

Place cursor in IOS Command Line Interface box

Press ENTER

At User Access Verification Password

Type                  cisco123                    ………….this will NOT be displayed

SwTMyFirstSwitch will be displayed

Type             enable     at the >prompt  and press ENTER

You will be requested for a further password

Type     cisco      and press ENTER…this pw will NOT be displayed

Now you should see

SwTMyFirstSwitch in (hatch) mode

Observe running configurations

Type    show running-config           press ENTER

Privileged password is shown

Remove privileged password

Privileged password now encrypted

Login in cisco123 then cisco

Enable

Config t

Type no enable password

Type enable secret cisco

Exit

Once out of (config) mode

Type show running-config

Password is now encrypted

Before = 1199 bytes				After = 1228 bytes (with service pw encryption)

Do show running-config

Do show interfaces

Lab 4

Task 2

Wireless router

GUI

Setup tab

Scrolldown and change IP address to 192.168.10.1

Enable DHCP

Change Set Start IP to 192.168.10.20

Max users = 3

SAVE

Next in Wireless Tab (basic settings)

Network mode = mixed

SSID = MyNetwork

Broadcast = enabled

SAVE

Still in Wireless tab (now Wireless security tab)

Change security mode = WPA2 Personal

Encryption = AES

Set PW = 123456789

SAVE

Now in ADMIN Mgmt tab

Change router PW = Cisco

Enable remote Mgmt

SAVE

Back to main PT topology

Click on each laptop in turn to change to wireless cards WPC 300 N

Remembering to first turn off to remove & replace then turn back on.

Click on laptop and in Physical exchange cards

Now to Laptop 0

Click on laptop 0

Select Desktop tab, then PC wireless icon

Select Connect tab

Refresh

Select MyNetwork

Connect

Next screen enter PW = 123456789

Press connect….nothing will be seen here…but topology will show ))))))))) link to router

Repeat for laptops 1, 2 & 3 until all linked ))))))))))))))))) to router

Laptop 0

IP address = 192.168.10.2

Subnet Mask = 255.255.255.240

Default gateway = 192.168.10.1

Ping successful

Laptop 1

IP Address = 192.168.10.3

Subnet Mask = 255.255.255.240

Default Gateway = 192.168.10.1

Ping successful

Laptop 2

IP address totally different = 169.254.45.38

Subnet mask = 255.255.0.0

No Default Gateway !!!

Ping Unsuccessful

Laptop 3

IP address totally different = 169.254.119.210

Subnet mask = 255.255.0.0

No Default Gateway !!!

Ping Unsuccessful

Unable to PING due to maximum number of USERs = 3 (Router + 2 laptops = max)

All laptops gained an IP address but only the first 2 laptops gained the DHCP (default gateway address).

Changed maximum users to 5 (from 3) to enable laptops 2 & 3 to become part of this Network, in Router setup & SAVE.

Laptop 2 now

IP Address = 192.168.10.5

Subnet Mask = 255.255.255.240

Default Gateway = 192.168.10.1

Ping successful

Laptop 3 now

IP Address = 192.168.10.4

Subnet Mask = 255.255.255.240

Default Gateway = 192.168.10.1

Ping successful

Use ipconfig /all to find MAC physical address

Laptop 0

Wireless MAC Address = 00 30 F2 D8 92 91

Laptop 1

Wireless MAC Address = 00 02 16 2A 40 B5

In Router / GUI / Wireless / Wireless MAC Filter tab

Click on    Enable          and then           Prevent PC’s

Enter laptop 0 & 1 MAC address details and SAVE

These laptops are NO longer connected to the network