Lab: IPv4, Subnetting, VLSM and IPv6 Addressing

Understanding IPv4 and IPv6 addressing (word count 1000)

This part of lab worksheet require you research and write a report on “Features of IPv6”. You may discuss but not limited to structure, address types, advanced functionalities and security.

Report:

Features of IPv6

1. Introduction

2. IPv6

3. Conclusion

4. References

Subnetting and VLSM

NOTE: To complete this part you would need full understanding of Lab 10 to 14.

Using the Network address 172.16.2.0/24, VLSM the network to support the following requirements:

Subnet A: 16 hosts

Subnet B: 2000 hosts

Subnet C: 6 hosts

Subnet D: 500 hosts

Subnet E: 60 hosts

Subnet F: 180 hosts and Serial links between routers

Table 1

| Subnet | Network Address | Subnet mask | First Address | Last Address | Broadcast Address |
| --- | --- | --- | --- | --- | --- |
| B = /21 | 172.16.2.0 | 255.255.248.0 | 172.16.2.1 | 172.16.9.254 | 172.16.9.255 |
| D = /23 | 172.16.10.0 | 255.255.254.0 | 172.16.10.1 | 172.16.11.254 | 172.16.11.255 |
| F = /24 | 172.16.12.0 | 255.255.255.0 | 172.16.12.1 | 172.16.12.254 | 172.16.12.255 |
| E = /26 | 172.16.13.0 | 255.255.255.192 | 172.16.13.1 | 172.16.13.62 | 172.16.13.63 |
| A = /27 | 172.16.13.64 | 255.255.255.224 | 172.16.13.65 | 172.16.13.94 | 172.16.13.95 |
| C = /29 | 172.16.13.96 | 255.255.255.248 | 172.16.13.97 | 172.16.13.102 | 172.16.13.103 |
| R1 > R2 /30 | 172.16.13.104 | 255.255.255.252 | 172.16.13.105 | 172.16.13.106 | 172.16.13.107 |
| R2 > R3 /30 | 172.16.13.108 | 255.255.255.252 | 172.16.13.109 | 172.16.13.110 | 172.16.13.111 |