| LOCATION Swansea | LLEOLIAD abertawe |
| --- | --- |
| LEVEL 4 | LEFEL 4 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| may 2016 | mai 2016 |

MODULE CODE: M4X01123

IP INTERNETWORKING

TIME:  2 HOURS

answer Three quESTION out of Four

Equipment or materials required – None

Adnoddau neu ddefnyddiau sydd eu hangen - Dim

ANSWER THREE QUESTION OUT OF FOUR

  - With reference to routing loops;

    - What are the conditions that can lead to routing loops?

    - How routing loops can impact network performance includes?

(8 marks)

  - Define 	Zinn’s three routing principles.				(9 marks)

  - Define a Metric and list the metrics used in IP routing protocols. 	(8 marks)

  - Link state routing protocols:

    - What are the requirements for using a link state routing protocol? (6 marks)

    - What are the two link state routing protocols used in IP routing? (4 marks)

  - Explain OSPF packet types. 						(10 marks)

  - What is the purpose of TTL filed in an IP header and how it works? (5 marks)

  - A small network has been set up as shown in figure 2. You are asked to configure a suitable routing that gives you full control of path selection.

    - Which routing you would configure? 			(2 marks)

    - List advantages and disadvantages of your choice of routing. (6 marks)

    - Write the full commands to establish end-to-end connectivity. (4 marks)

Figure 2

  - Compare classful and classless routing protocols 			 (8 marks)

  - With examples explain interior and exterior gateway routing protocols.										(5 marks)

  - Compare link state and distance vector routing protocols.	 	(8 marks)

  - Refer to partial output shown in figure 3 answer the following:

    - Which routing protocol is configured on router R1? 		(1 mark)

    - What are the remote networks learned by R1 using the

protocol? 							(3 marks)

    - For each remote networks list the metric used by the protocol.

(3 marks)

Figure 3

  - What is the purpose of Administrative Distance? Write the AD value for protocols listed below

    - Connected interface

    - Static

    - Default

    - EIGRP

    - OSPF

    - RIP

    - Unknown							(10 marks)