HEAT TRANSFER

RADIATION

Exercise 1:

Calculate the nett rate of radiant energy loss from a pond of water (10 m x 10 m) at 5C on a clear night in winter.

Exercise 2:

The Sun may be regarded as a blackbody with a surface temperature of 5800 K at a mean distance of  149×106 km from the earth. The diameter of the Sun is 1.4×109 km and that of the Earth 12.8×103 km. Calculate:

- Total energy radiated by the Sun.

- Energy received per m2 just outside the Earth’s atmosphere.

- Total energy the Earth would receive if no energy were blocked by the Earth’s atmosphere.

- Energy received by a solar collector (1m x 1m) whose perpendicular is inclined at 30 to the Sun, if the energy loss through the atmosphere is 40% and diffuse radiation is 20% of direct radiation.

Exercise 3:

A steel plate is placed on non-conducting opaque surface, normal to incident solar radiation (direct + diffuse) of 0.75 kW/m2. Neglecting convection, calculate the equilibrium temperature of the plate when it is:

- Oxidised,

- Polished.

Assume grey body behaviour (a surface with an emissivity less than 1, but which remains constant over all wavelengths, is called a “grey body”).

Notes:

Kirchhoff’s law states that at thermal equilibrium, the emissivity of a body (or surface) equals its absorptivity  (fraction of incident light that is absorbed by the body or surface).

Energy absorbed=absorptivity×irradiance

The irradiance is the radiant energy falling per unit area.

Thermal equilibrium: a state in which there is no net flow of heat. If 2 bodies are in thermal equilibrium, then they have the same temperature.

Exercise 4

The crankcase of a motor vehicle is 300 mm wide x 100 mm deep x 456 mm long (see exercise 4, “convection”), and has a mean surface temperature of 75C. If the re-radiation temperature from the atmosphere is assumed to be 0C, and the surface emissivity is 0.8, calculate the corresponding rate of heat radiation.

Exercise 5:

(i) An unlagged pipe, 200 mm diameter, has a surface temperature of 115C, and has an oxidised surface. If it is enclosed in a bare room with walls at a temperature of 15C, calculate the net radiated rate of heat loss per meter length of pipe.

(ii) If the pipe were then lagged with a 30 mm insulation secured by an anodised aluminium skin of negligible thickness, such that the external temperature reduces to 30C, calculate the new rate of radiated heat loss.

Exercise 6:

A “solar collection plate” is angled perpendicular to the incident sun’s rays on a day when the total solar radiation (direct + diffuse) is 0.95 W/m2. Calculate the temperature reached by the plate assuming it has a grey surface with emissivity 0.8.