#include <main.h>

#case

#define LCD_ENABLE_PIN PIN_D2

#define LCD_RS_PIN     PIN_D0

#define LCD_RW_PIN     PIN_D1

#define LCD_DATA4      PIN_D4

#define LCD_DATA5      PIN_D5

#define LCD_DATA6      PIN_D6

#define LCD_DATA7      PIN_D7

#include <lcd.c>

#define SW_A 1

#define SW_B 2

#define SW_C 4

#define SW_D 8

#define MOTA PIN_C0

#define MOTB PIN_C1

#define BUT PIN_A1

/*

* Read the value of an adc channel

*/

int8 read_adc_chnnl(unsigned channel) {

set_adc_channel(channel);   /* select the channel */

delay_us(10);               /* delay to configure channel */

read_adc(ADC_START_ONLY);       /* start the ADC conversion */

sleep();                        /* sleep whilst the ADC converts */

return read_adc(ADC_READ_ONLY); /* read the converted ADC value */

}

/*

* Switch A & B share outputs for their ADC values

*/

void show_adc(unsigned adc) {

output_b(adc);  /* Show the ADC value on the LEDs */

lcd_gotoxy(9,1);                /* move the LCD cursor to the middle of the first line */

printf(lcd_putc, "%03u", adc);  /* print the ADC value with padding on the LCD */

printf("%03u\r\n", adc);        /* print the ADC value to RS232 */

}

/*

* Switch A

* Using the potentiometer (channel 0)

*     Display its value

*     Rotate the motor

*         <75:       clockwise

*         >175:      anti-clockwise

*         otherwise: stop

*/

void task_a(void) {

int8 pot = read_adc_chnnl(0);   /* read the potentiometer (channel 0) */

/*

* Truth table:

*  pot < 75 | pot > 175 | result

* ----------+-----------+--------

*     0     |     0     |   nr

*     0     |     1     |   acw

*     1     |     0     |   cw

*     1     |     1     |   N/A

* ----------+-----------+--------

*

* nr  - no rotation

* acw - anti-clockwise

* cw  - clockwise

*/

int1 cw = pot > 175;

int1 acw = pot < 75;

output_bit(MOTA, cw);

output_bit(MOTB, acw);

lcd_gotoxy(4,2);    /* goto Line 2 */

if (cw) {           /* if motor moving clockwise */

printf(lcd_putc, "  clockwise   ");

} else if (acw) {   /* if motor moving anti-clockwise */

printf(lcd_putc, "anti-clockwise");

} else {            /* otherwise motor has stopped */

printf(lcd_putc, "     none     ");

}

show_adc(pot);  /* show the potentiometer in the required methods */

}

/*

* Switch B

* Using the potentiometer (channel 0)

*     Display its value

*/

void task_b(void) {

show_adc(read_adc_chnnl(0));    /* show the potentiometer (channnel 0) in the required methods */

}

/*

* Switch C

* When the button (A1) is pressed

*     Rotate the motor clockwise

*     Print the status of the button on the LCD and RS232 once

*/

void task_c(void) {

static unsigned int8 prev_but = 2;   /* initialise prev_but as unpressed */

int1 but = input(BUT);      /* read the state of the button (A1) */

/*

* Truth table:

*  but | result

* -----+--------

*   0  |   nr

*   1  |   cw

* -----+--------

*

* nr  - no rotation

* cw  - clockwise

*/

output_bit(MOTA, but);

output_low(MOTB);

lcd_gotoxy(27,1);   /* move the LCD cursor to the middle of the third line */

if (but) {  /* check to see if button is pressed */

if (prev_but != but) {  /* check to see if the button has just been pressed or released */

printf(lcd_putc, " pressed ");  /* print "pressed" on the LCD */

printf("pressed\r\n");  /* print "pressed" to RS232 */

}

} else {    /* button is not pressed */

if (prev_but != but) {  /* check to see if the button has just been pressed or released */

printf(lcd_putc, "unpressed");  /* print "unpressed" on the LCD */

printf("unpressed\r\n");    /* print "unpressed" to RS232 */

}

}

prev_but = but; /* store the current state of the button */

}

/*

* Switch D

* Illuminate one LED that moves backwards and forwards along a row of LED's connected via PORT B

*/

void task_d(void) {

static unsigned int8 sleft = 1,led = 1;

if (sleft) {    /* check to see if the LED is moving left */

led <<= 1;              /* move the LED left */

sleft = led != 0x80;    /* change direction when LED at far left */

} else {        /* the LED is moving right */

led >>= 1;              /* move the LED right */

sleft = led == 1;       /* change direction when LED at far right */

}

output_b(led);  /* show the illuminated LED */

delay_ms(200);  /* wait 200ms */

}

void main(void)

{

unsigned int8 prev_sw = 0;

setup_adc_ports(sAN0);                          /* set the potentiometer pin to anologue */

setup_adc(ADC_CLOCK_INTERNAL|ADC_TAD_MUL_0);    /* configure the ADC clock and clock multiplier */

enable_interrupts(INT_AD);  /* this interrupt flag goes high when an ADC conversion completes */

enable_interrupts(PERIPH);  /* this interrupt allows the PIC to awake from sleep when INT_AD goes high */

lcd_init(); /* setup the lcd */

while(1) {  /* loop forever */

unsigned int8 sw = (input_a() >> 2) & 0xf;   /* read PORT A and mask off switch pins */

if (sw != prev_sw) {    /* check to see if the switch has changed position */

prev_sw = sw;       /* store the current state of the switch */

/* turn off the motor */

output_low(MOTA);

output_low(MOTB);

output_b(0);    /* turn off the LEDs */

lcd_putc('\f'); /* clear the LCD */

if (sw == SW_D) {   /* check to see if the new switch position is D */

lcd_gotoxy(26,2);               /* move the LCD cursor to the middle of the fourth line */

printf(lcd_putc, "Kitt mode");  /* print "Kitt mode" on the LCD */

printf("Kitt mode\r\n");        /* print "Kitt mode" to RS232 */

}

}

/* dependent on the switch position, perform the relevant task */

switch (sw) {

case SW_A:

task_a();

break;

case SW_B:

task_b();

break;

case SW_C:

task_c();

break;

case SW_D:

task_d();

break;

}

}

}

Comments

2.. 20   3.. 30    4.. 40   5.. 10

****************************************

2.. Your code has a number of your own functions and all work, you should always put your functions (subroutines) after the main function to follow convention. Your statements indicate what each line does 70%

3.. Your program flows nicely 72%

4.. Your program achieves the desired results. 75%

5.. Your program is nicely laid out and well indented. 75%