| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 3 | LEFEL 3 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| JULY 2018 | Gorffennaf 2018 |

MODULE CODE: m3x01513

SCIENCE - RESIT

TIME:  2 HOURS

answer all questions

Equipment or materials required – calculator

Adnoddau neu ddefnyddiau sydd eu hangen - cyfrifiannell

ANSWER ALL QUESTIONS

- Write down the units for the following:					[5]

a)	Current

b)	Resistance

c)	Velocity

d)	Force

e)	Weight

- Given that the equation for momentum can be written as;

moment=force×distance

Write down the units of a moment						[2]

- A body of weight 750N rests on a rough inclined plane which is at an angle of 260 to the horizontal.

a)	Draw a free body diagram in your answer booklet showing the weight, normal reaction, friction and the component of weight that acts down the slope	[2]

b)	Calculate the magnitude of frictional force					[4]

c)	Calculate the magnitude of the normal reaction to the surface.		[4]

- By considering figure 1

Calculate the value of the distance d so that the object is in equilibrium	[4]

- Figure 2 shows a 14m uniform beam of weight 3.5KN which is at rest. The beam is supported by two pillars. Pillar X is positioned 0.70m from the left hand side of the beam, pillar Y is positioned 1.8m from the right hand side of the beam.

X

Y

Figure 2

a)	Copy this diagram on your answer booklet, showing all forces.		[2]

b)	Determine both support forces.						[6]

6.	Define the following electrical characteristics.				[2]

a)	Resistance

b)	Power

7.	The current in a certain wire is 0.5A. Calculate the charge passing a point in the wire in i) 15s and ii) 15 minutes.							[4]

8.

a)	State Ohm’s law								[2]

b)	Calculate the current that flows through a wire of resistance 12.5Ω when the potential difference across the wire is 1.5V						[4]

9.	The circuit shown in figure 3 shows three identical resistors placed in series.

a)	Using Ohms law, calculate the voltage of the cell if a current of 0.5A flows.

b)	What value of I would you expect if the cell has a voltage of 12V 		[4]

10.	Two 20Ω resistors are connected in series with each other. The series combination is connected in parallel with a 40Ω resistor and a 6V battery of negligible internal resistance, as shown in figure 4.

Calculate:

a)	the total resistance of the circuit						[2]

b)	the current leaving the cell							[2]

c)	the power supplied to each resistor						[3]

d)	the power supplied by the battery						[2]

Diwedd/End