University of Wales Trinity Saint David

School of Engineering

Engine Simulation and Analysis

Jeffrey Elliss

1802644

Motorsport Engineering Beng

20/1/20

Powertrain And Control,  M5X00404

# Contents

Introduction

- – Engine Analysis

Simulation

# Introduction

The following work is to analyse an example engine, create a simulation to replicate its characteristics and then use the real data to validate the simulation process. The simulation will be based on first law thermodynamics processes.

# Engine Analysis

Figure 1-Volume graph from data.

The volume of each cylinder changes in a roughly sinusoidal manner, though the curve is slightly fatter than a basic sin wave. This is due to the relationship between the angle of the crankshaft and the angle of the conrod. As the conrod is longer than the crank offset the effective vertical length of it changes more slowly than the effective vertical length of the crank offset. This relationship means that the rate of change of volume varies differently than a simple sin curve.

Figure 2-Mean pressure from data

At the beginning of the cycle the pressure is approximately the same as the ambient. This drops as the piston moves down increasing the volume, the valves are open at this time as the air moves in its velocity increases, but its total energy remains the same. This means that the pressure drops as Bernoulli’s equation shows.

On the compression stroke the pressure peaks a few degrees after top dead center when the volume in the cylinder is still small and combustion pressures have taken over the process. The start of combustion can be approximated with this as -5 degrees, though it is difficult to tell and only an approximation by the fact that the rate of pressure change increases here.

Figure 3-Log pressure from data

The reduction in pressure can be more clearly seen here as well as the more noticeable change in rate of change of pressure around the start of combustion. The graph becomes much less chaotic after the -90-degree point, this may be to do with the piston velocity as seen in figure 4 -90 degrees is the point at which the piston is starting to decelerate sharply.

The change in spikiness of the graph may also be a result of flow characteristics in the cylinder. As the gas is drawn into the cylinder around the valve it is accelerated and pushed into an obstruction. As the air flows around the valve it will try to fill the low pressure directly behind the valve and detach from the valve, becoming turbulent as it enters the cylinder. As the gas is compressed the turbulence evens out throughout the cylinder as the air is no longer flowing and the low-pressure area that was left by the valve will be roughly equalized with the rest of the cylinder.

Figure 4 - Rate of change of volume from data

This plot was made to examine whether the jaggedness of both the pressure and log pressure graphs was related to piston acceleration. It suggests that the points where those graphs smooth out are linked with the points where the acceleration of the piston changes the most. Although as these points are also where the piston is changing direction and the state of the valves is changing it is more likely that the more jagged parts are related to airflow characteristics more so than piston acceleration.

Figure 5-PV from data

Figure 5 shows a pressure volume cycle which is a deviation from an otto cycle as each process takes time and has sources of energy loss through heat rejection and friction of the piston rings against the cylinder walls. It clearly shows each stroke, the lower two lines are the intake and exhaust strokes while the banana shape above are the compression and power strokes.

Figure 6-Log pressure log volume from data

The straight line sections of the graph have a gradient which closely approximates the polytropic index of the compression and expansion parts of the cycle.

Figure 7-Polytropic Index, calculated from data

Figure 7 shows the polytropic index at any given crank angle, the huge peaks are where the system is no longer a polytropic process, such as during combustion and the moment when the piston changes direction moving from the intake stroke to the compression stroke, and vice versa. The moment when the valves open the system is no longer a constant mass thermodynamic system, this change of system state means that the polytropic model no longer applies. Any system constant mass or constant volume can be polytropic, however the change in state between the two means that the process stops being adiabatic. For example, when the exhaust valves open heat is lost through the exhaust.

Figure 8-Mass fuel burned, simulated blue, from data orange

Figure 8 shows the portion of the fuel burned against crank angle, starting slowly with the majority of the fuel then being burned between 0 and 29 degrees. At the ignition point only the fuel around the spark plug is ignited, as this flame ignites the area around it combustion accelerates until a point when the majority of the fuel air mix has been burned. The simulated number was found using matlab to curve fit, with an a of 5.333 and an m of 5.675. Note that the simulated curve peaks slightly earlier than the data and that it is a more continuous curve than the data. This is because the graph of the data only has a small number of points to plot with.

Figure 9-Work done against crank angle from data

Figure 9 shows the majority of the energy is released during combustion at the point which the pressure is highest as the crank angle changes roughly constantly. As the gas is compressed and expands some work is done to compress it and it does work to expand. The expansion phase is a gentler downward slope than the compression phase. As there is no combustion to increase the rate of work done.

During the phase between -154 and 0 degrees the compression stroke means that the engine is having to do work to compress the gas before combustion begins and the piston begins to move downwards.

Figure 10-Rate of change of pressure from data

Figure 10 more clearly shows the start of combustion, the change in pressure fluctuates noticeably between 348 and 354 degrees of crank angle. This is just before top dead center and the peak rate of change of pressure is 10 degrees past top dead center this is ideal as it means that the maximum pressure increase, and peak pressure are providing force to push the piston downwards rather than resisting the pistons rise.

During the intake and exhaust strokes this graph is spiky, this could be due to inconsistent flow into the cylinders caused by turbulence created in the intake and exhaust manifold or pressure waves and vortices created when the air moves around the valves. As I’m not sure that this can be simulated without using a CFD package I would expect that then simulation will be smooth around these areas.

# Simulation

Figure 11-Volume, both calculated and from data

Figure 11 is a plot of the way that the volume changes as the crank rotates, on first look it appears to be a simple sin curve, however because of the geometry between the con rod and the crank. This being that the con rod acts as a much longer hypotenuse for a triangle than the crank offset does.

Figure 12-Combustion pressure simulated against data

First time attempting to simulate pressure was done without considering that when the valves open the pressure will remain roughly ambient. That graph as expected showed a smooth curve which peaked as the volume was at its lowest.  The simulated numbers here peak slightly too high and have too steep of a gradient after peak pressure, this is likely an error in the polytropic index or the use of the wiebe function.

Figure 12-Change in pressure simulated

Compared to figure 10 this graph shows much less fluctuation around the intake and exhaust strokes as well as around the start of combustion. This is because in order to simplify the simulation process I have assumed that during valve open times the pressure will remain around ambient. This is not quite true as when air flows its pressure will change as its total energy will remain approximately the same. It also shows more clearly a rate of pressure change drop at the start of combustion. This could be due to the start of combustion where the spark plug fires adding energy into the system and the beginning of the reaction.

Figure 13-Work done against crank angle simulated

This shows a similar shape to the calculated values on figure 9, however the values are vastly different, this is probably a unit’s issue which I have missed. It does show that the majority of the work is done around combustion as it should the major difference being that the work done this graph shows is tiny in the hundreds of joules rather than the thousands. The negative work done before combustion starts and during the compression stroke here matches the shape of the real data however the values are different.

Figure 14-Simulated PV

Figure 14 has an odd shape because the compression and expansion values for polytropic index are the same. This is an error as their values will be different shown by figure 6. This difference is because the gas combination inside the cylinder is different after combustion and as such will compress and expand with different characteristics. The pressure shown during the intake and exhaust strokes is a straight line as these values are set to be atmospheric for the duration. The strange shape may also be due to the assumption I made about when the valves open and therefore when the pressures will begin to significantly change.

# Conclusion

The first part of this work analyzing the engine data showed that when the valves of an engine are close the characteristics are well modelled by first law thermodynamics equations and this is born out when compared to the simulated values. However, when the valves are open there are more external factors which complicate the process such as fluid flow characteristics and energy loss through the exhaust being inconsistent.