#include <main.h>

#define LCD_ENABLE_PIN PIN_D2

#define LCD_RS_PIN PIN_D0

#define LCD_RW_PIN PIN_D1

#define LCD_DATA4 PIN_D4

#define LCD_DATA5 PIN_D5

#define LCD_DATA6 PIN_D6

#define LCD_DATA7 PIN_D7

#include <lcd.c>

// Global variables

unsigned int pot_1 = 0;

unsigned int adc_value;

void analog(void);																					// define function/subroutine prototypes

unsigned int adc;																					// declare global variables

unsigned int knight_light[]={128,64,32,16,8,4,2,1,1,2,4,8,16,32,64,128,};

// function prototypes

void read_pots(void);  // reads pots values from ADC channels

void display_pots(void); // pot values on the display

void button_press(void);  // checks for button press

void spin_motor_counter_clockwise(void); // Spins Motor CCW

void spin_motor_clockwise(void); // spins motor CW

void stop_motor(void); // stops motor

void display_LED(void); // pot values in binary on LEDs

void pot_values_turn(void);

void main()

{

// Initialise the display

lcd_init();

// Configure ADC port for AN0

setup_adc_ports(sAN0);

setup_adc(ADC_CLOCK_INTERNAL | ADC_TAD_MUL_0);

// Main loop

while(TRUE)

{

switch(input_a() >> 2) // bit shift to move unwanted inputs from button/pot

{

case 1: // task 1, cases count up in powers of 2 as looking for 2^nth input along A starting from RA2

output_b(0);

read_pots();       // Acquire new pot value

display_pots();    // Display the pot value

pot_values_turn();	// motor functions in 1

break;

///////////////////////////////////////////

case 2: // task 2// LED pot value

printf("\f");

printf(lcd_putc, "\f");

read_pots();       // Acquire new pot value

display_pots();	 // Display the pot value

output_b(pot_1); // outputs binary val of pot 1 to LEDs

break;

///////////////////////////////////////////

case 4:    // task 3 // button

printf(lcd_putc, "\f");

printf("\f");

output_b(0);

if(input(PIN_A1))

{

spin_motor_clockwise();

lcd_gotoxy(27, 2); // position on lcd

printf(lcd_putc, "pressed      "); // could be neater but spaces do the job

printf("\f");

printf("pressed, CW \r\n"); // prints to terminal

delay_ms(200);

}

else

{

stop_motor();

lcd_gotoxy(27, 2);

printf(lcd_putc, "Not pressed");

printf("\f");

printf("not pressed, Stop \r\n");

delay_ms(200);

}

break;

///////////////////////////////////////////

case 8:   //task 4// Knight rider

printf(lcd_putc,"\f");

printf("\f"); 						//clear the LCD display

lcd_gotoxy(27,2);							//got to center of line 4 on the LCD display

printf(lcd_putc,"Knight time");				//print to LCD display

printf("Knight Time \r\n");		//if disp is 1 print to virtual termial

unsigned int q;

int i;

for (i=0;i<16;i++) // loop runs as long as i<16, 8 lights flashed twice= 16

{

output_b(knight_light[i]);

delay_ms(200);

}

break;             // ends case 8, task 4

}

} // ends the switch statement

}

// functions ///////////////////////////////////////////

void read_pots(void)

{

// Pot 1 on AN0

set_adc_channel(0);

delay_us(50); // lets value settle if it was changed

pot_1 = read_adc();

}

void display_pots(void)

{

lcd_gotoxy(7, 1);// roughly center line 1

printf(lcd_putc,"Pot1: %3u", pot_1); // prints value of Pot between 0 and 255

printf("\f");

printf("Pot1=  %3u\r\n" , pot_1);

delay_ms(300);

}

// functions for motor

void spin_motor_counter_clockwise(void) // Allows motor to spin CCW as pin a7 is high and a6 low so wants to spin left to right or CCW

{

output_low(PIN_C0);

output_high(PIN_C1);

lcd_gotoxy(7, 2);// roughly center line 1

printf(lcd_putc, " CCW"); // prints value of Pot between 0 and 75, CCW

}

void spin_motor_clockwise(void)     // allows motor to spin CW as a6= high and a7 = low so wants to go right to left or CW

{

output_low(PIN_C1);

output_high(PIN_C0);

lcd_gotoxy(7, 2);// roughly center line 1

printf(lcd_putc, "  CW");

}

void stop_motor(void)   // Pins Low so no spinning

{

output_low(PIN_C0);

output_low(PIN_C1);

lcd_gotoxy(7, 2);// roughly center line 1

printf(lcd_putc, "stop");

}

void pot_values_turn(void) //

{

if (pot_1 < 76)

{

spin_motor_counter_clockwise();

}

else if ((pot_1 >= 75) && (pot_1 <= 176))

{

stop_motor();

}

else if (pot_1 > 175)   //

{

spin_motor_clockwise();

}

}

// end //

Comments

2.. 20   3.. 30    4.. 40   5.. 10

****************************************

2..Your code has a number of structures and all work except you don't put the value of RV1 on the LEDS in section 1. Your statements indicate what most lines do 68%

3.. Your program flows nicely 75%

4.. Your program achieves the desired results. 75%

5.. Your program is nicely laid out and well indented but could have been remarked better. 70%