| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 4 | LEFEL 4 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| January 2024 | IONAWR 2024 |

MODULE CODE: ACDA4004

Micros Peripherals & Interfaces

TIME:  2 HOURS

answer all questions

Equipment or materials required. Please select from the following:

Adnoddau neu ddefnyddiau sydd eu hangen.

X

Cyfrifiannell/Calculator

Llyfr Agored/Open Book

Arall/Other				Nodwch/Please Note ……………………….

ANSWER ALL QUESTIONS

- The following comprise a number of mnemonics from the PIC16F84 microprocessor instruction set. Give the meaning of what each mnemonic stands for and explain what the instruction actually does.

  - MOVLW

[2 Marks]

  - DECFSZ

[2 Marks]

  - CLRF

[2 Marks]

  - SUBWF

[2 Marks]

  - DECF

[2 Marks]

  - ANDLW

[2 Marks]

- For the successive approximation ADC shown in Fig Q2, explain the general principle of operation for the circuit.

DAC

SAR

Clock

Reset

Output

Vin

Q0

Qn

Digital Output

Fig Q2.

[10 Marks]

- Draw the logic symbol for an XOR gate and provide the truth table for it.

[4 Marks]

- Write the Boolean equation and truth table for the circuit shown in Fig Q4

Fig Q4

[6 Marks]

- Compare and contrast the Harvard and Von Neumann processor architectures and state common applications for each.  You may wish to use diagrams to support your answer.

[10 Marks]

- Using De Morgan’s Theorem, show how you can implement a NAND function using only NOR logic.

[8 Marks]

Diwedd/End