ACDA4007 Physics Solutions (2022)

Q1: (14 marks)

- W = 7 *2*pi rads per min = 14*pi /60 rad s-1     = 0.73 rad s-1    	 (3 marks)

- I=0Lr2  dm  =  0Lr2  dr/L    = ML3/3L =  ML2/3 = 5*9/3 = 15  kg m2      (8 marks)

- Torque -= I *ang acceln   therefore I = 300/40   = 7.5 N m  to 2 DP (3 marks)

Q2: (12 marks)

- I’m looking for comments such as the assumption of a

- linear velocity profile of H.E. products between plates

- no fracture of plates

(4 marks)

- Selection and double application of  appropriate asymmetric Gurney formula to generate:

Vm   = 	1235.70 ms-1						(4 marks)

Vn  =  	134.58 ms-1						(4 marks)

Where m=   1 kg  and n =  10 kg plates.

Q3: (19 marks)

- A body of mass 5 kg is on an inclined plane inclined at an angle θ to horizontal. The frictional force between the body and the plane is represented by a coefficient of friction, µ = 0.6  .

Calculate:

  - The force on the body parallel   to the plane due to gravity.

5.0 kg

θ

Force parallel to  plane  = mgsinθ = 5x9.8sinθ = 49 sinθ				(3 marks)

  - The reaction force, R, that the plane exerts on the body.

R=  mgcos θ =  49cos θ							(3 marks)

  - The maximum angle of inclination for the body not to slide down the plane.

This occurs when frictional force jus balances force due to gravity parallel to plane

i.e.     µR =  mgsin θ

µmgcos θ = mgsin θ

i.e. 	tan θ= µ

θ =  tan-1(0.6) = 30.96 degrees to  2 d.p. 				(4 marks)

- A body of mass 500g is dropped from a vertical height of 2.5m. Calculate the speed at impact on ground.

Assume acceleration due to gravity g = 9.8 ms-2

Potential Energy  is totally converted into  Kinetic Energy

mgh=  0.5 mv2

where h= 2.5m,  m = 0.5kg  and v is v speed on impact at ground level.

v2= 9.8 x 2.7x 2 = 52.92

v= 7.27 ms-1 to 2 d.p.

(4 marks)

- A projectile of mass 30g penetrates a target to a depth of 0.6m. If the speed at initial impact is 700 ms-1 determine the resistive force that the body exerts on the projectile, assuming uniform resistance.

Kinetic Energy at impact  =  work done by resistive force in stopping body

0.5 x 0.030 x 7002 = F x 0.6

F = 12,250 N

(5 marks)

Q4: (20 marks)

- Using simple sketches, compare the stress- strain relationship for a ductile metal and a brittle material.	A simple bilinear approximation (or better) is expected for this assessment.

stress

stress

Ductile metal

failure

brittle

Yield stress

Yield stress

UTS

plastic

elastic

strain

strain

(6 marks)

- An applied force stretches a uniform wire. If the change in length is 5x 10-3 m and the strain was 0.03, calculate the original length of the wire.

Strain =extension/original length

strain= 5x10-3/L

L = 0.17  m 							(4 marks)

- A vertical uniform wire has Modulus of Elasticity 150 x 109 pascals, cross-sectional area 0.2 cm2 and natural length 2.5 m. Calculate the stress and strain values when a mass of 80 kg is suspended from one end.

Assume acceleration due to gravity g = 9.8 ms-2

Stress= force/cross-sectional area  = 80x9.8/0.2x10-4  = 3.92 x 107 Pa 		(5 marks)

Strain = Stress/E  = 3.92 x 107 /150 x 109   = 2.61 x 10-4

(5 marks)

Q5: (15 marks)

- How much energy is required to raise the temperature of a body with specific heat 400 J kg-1 K-1 and mass 2 kg through a temperature increase of 20 K?

From definition of specific heat Q= mCΔT   where C= 400, m= 2 kg and ΔT=20

Q= 16,000J

(5 marks)

- A metal bar of initial length 5 m at 30OC is raised in temperature to 120OC. If the coefficient of thermal expansion is 2x 10-4 K-1, what is the new length of the bar?

L(T) = L(0)[1+ αΔT]  where   α = thermal exp coeff, L(T) is length at new temp ad L(0) is length at original temp

L(120)  = 5 [(1 + 0.0002x90] = 5 x 1.018  = 5.09 m

(5 marks)

- A laser pulse of total energy 8 J is applied for 2.2 ms. If the pulse has a rectangular temporal profile, what is the power?

For a rectangular pulse  energy =  area under power –time curve .

i.e. 8= Power x 0.002

Power = 3636.36 W

(5 marks)