# LAB 3:

# Preparation of Methyl-3-nitrobenzoate

# and Nitroguanidine

L6 -Advanced Chemistry of OME (ACDA6020)

2223155

Table of Contents

Table of Equations	2

Table of figures	2

Aim	2

Hypothesis	2

Variables	2

Theory	3

Observations/Data	3

methyl-3-benzoate:	3

Nitroguanidine:	3

Results and Discussion	4

Conclusions	4

References (IEEE)	4

# Table of Equations

# Table of figures

# Aim

This experiment aims to prepare two nitrated compounds, Methyl-3-nitrobenzoate and Nitroguanidine. This is done through the process of nitration and will show how energetic compounds from propellants to high explosives are manufactured at a lab scale for the OME industry and the importance of using the correct process to ensure a safe product.

# Hypothesis

It is hypothesised that methyl benzoate will undergo nitration under the conditions of the experiment; the nitro group, forming methyl 3-nitrobenzoate. In the nitration of Guanidine, the nitric acid will react with the guanidine and attach to one of the nitrogen atoms in the molecule to become nitroguanidine. Both nitration reactions will be slow reactions, as they must be kept under specific temperatures to avoid runaway reactions. At a lab scale, the predicted yield is around 60-80%.

# Variables

Control – volume of reactants, reaction temperatures

Independent – starting reactants

Dependant – mass of nitrated compound

# Theory

## Methyl-3-nitrobenzoate

Methyl Benzoate undergoes an Electrophilic Aromatic Substitution to form methyl-3-nitrobenzoate. This happens with nitric acid and sulphuric acid are mixed, as the sulphuric acid is stronger, it donates a hydrogen to nitric acid, forming H₂NO₃⁺, then as this is an unstable molecule, it forms the nitrenium ion and water [1]. The overall reaction is shown below:

Equation 1 - Formation of the electrophile [2]

The products from this reaction are added slowly in small quantities to the methyl benzoate. The nitrenium ion is positively charged and is the electrophile in this reaction, as it wants to gain electrons from the methyl benzoate. The benzene ring contains a cloud of delocalised electrons, which is why the positive electrophile is attracted towards it. Two delocalised electrons form a bond to the electrophile, giving the benzene ring an overall positive charge.[3] In the second part of the reaction, the HSO4- from products shown in equation 1, this negatively charged ion is attracted to the positive benzene ring and takes the hydrogen from the benzene ring to reform sulphuric acid. The following shows the steps of the mechanisms, where the arrows represent the movement of electrons: [4]

Figure 1 - Mechanism for nitration of Methyl Benzoate to Methyl-3-Nitrobenzoate [5]

This reaction is highly exothermic and produces large amounts of heat. This, however, can speed up the reaction and cause runaway reactions that are difficult to control, as the heat can cause it to react faster. To stop this, the reaction mixture and reactants are kept in an ice bath to keep it below 10 degrees, and reactants cannot be added until it cools back to 5 degrees

Once the reaction is complete, the product is dissolved in ethanol at warm temperatures and left to cool. As it cools, crystals form; the ethanol helps promote this formation as well as dissolving some of the impurities in the solution. [6]

If the reaction was started using toluene instead of methyl benzoate, then di-nitrotolene would be created when this is nitated further, tri-nirotolene would be formed, which is also known as TNT.

## Nitroguanidine

Nitroguanidine is an energetic composition that is used in triple base propellants, which are used most in artillery systems to give the product propulsion [7].

This reaction only uses a high concentration of sulphuric acid as well as guanidine nitrate. The sulphuric acid breaks down the nitrate ion in guanidine nitrate to form NO3-, which is negatively charged. this is a negatively charged ion that will be used as a nucleophile for the reaction. The positive charge on the attracts the nucleophile charge at the end of the double bond. The nucleophile then offers electrons to the nitrogen ion in NH2. This makes an overall positive charge with the 2 hydrogens. The sulphuric acid ion negative charge then takes the two positively charged hydrogen back and reforms as sulphuric acid and water. [8]

This reaction is exothermic, so to avoid over-nitration forming dangerous, highly sensitive chemicals, the reaction is kept below 10 degrees in an ice bath where the reactants id added slowly in small amounts and only when the temperature reduces to 5 degrees.[9]

Equation 2 -  equation for the conversion of guanidine nitrate to nitroguanidine

CH6N4O3→CH4N4O2+H2O

# Observations/Data

## methyl-3-benzoate:

Before looking at the yield for the experiment, the theoretical yield can be calculated using the following equation. The results are shown below:

Equation 3 - Theoretical Yield

theoretical yield=mass of methyl benzoate × Mr (methyl 3-nitrobenzoate)Mr (methyl benzoate)

Mr of Methyl-3-nitobenxoate = 181.15

Mr of Methyl benzoate = 136.15

theoretical yield=2.699 × 181.15)136.15)

theoretical yield=2.699 × 1.3305

theoretical yield=3.5910

The masses collected are shown below in the table

Table 1 - methyl-3-nitrobenzoate results

| Mass of methyl benzoate | 2.699 |
| --- | --- |
| Mass of the labelled glass bottle | 8.639 |
| Mass of labelled glass bottle + dried sample | 10.153 |
| Mass of methyl-3-nitrobenzoate | 1.514 |

Using these, a % yield can be calculated by comparing the actual vs the theoretical yield shown below

Equation 4 – yield question

% yield= Actual yieldTheoretical yield×100

% yield= 1.5143.5910×100

% yield= 42.2%

## Nitroguanidine:

The results from the experiment are shown in the table below

Table 2 - results for nitroguanidine

| Mass of labelled bottle | 83.730 |
| --- | --- |
| Mass of labelled glass bottle + dried NQ | 85.399 |
| Mass of NQ | 1.669 |

The percentage yield is then calculated using the equation below

Equation 5 - yield calculation for nitroguanidine

% yield= Mass of NQ4×122104×100

% yield= 1.6694×122104×100

% yield= 0.417×1.173×100

% yield= 48.9%

# Results and Discussion

The synthesis of the Nitroguanidine and Methyl-3-Nitrobenzoate yields 48.9% and 42.2% respectively. These are very low yields for the materials. It was hypothesised that the experiment would have low yields; however, these are much lower than the expected amount, but the desired products were still produced.

The amount of the reactants used was lower in the initial reaction, which could have caused a higher likelihood of a low yield, as small amounts of product are lost while the solutions are transferred between vessels would have a greater impact on the results. The Buchner flask used could have drawn through some of the product; however, this is not expected to cause the yield to drop to 40% Therefore, there is another factor involved.

For methyl-3-nitrobenzoate, the reaction requires strong acids to form the electrophile if these are not strong enough. The product is also slightly soluble in water, so when it is washed in the Buchner flask, some of the product can be lost. [3]

Another factor could be the temperature of the reaction, as they are exothermic. As there is a higher likelihood of forming unwanted products if the temperature gets too high, if the reactants are added too quickly, then the temperature gets too high, there is a higher likelihood of runaway reactions and over-nitrating, which can be dangerous

The final factor to consider is the time it takes for all the products to form the solution is left to sit before the reaction takes place to form these crystals. If the reaction is not complete when it is extracted from the solution, then the yield will decrease, as not all the product has had a chance to form

Nitroguanidine when synthesised, forms brittle needle-like strands; these become incredibly sensitive if they are snapped. When the product is pressed into a pellet, it can form voids and is more likely to snap. These two factors make the product less stable, making it more dangerous to handle and may not perform as expected, which poses a safety risk. Because of this, a very controlled process of breaking these crystals on purpose is done, so when they press, they are more compacted and less likely to get voids. This also makes the product more sensitive; however, if all are gridded, then it is much more predictable, and performance is more consistent. Impurities in the product can also cause a more sensitive product; these can come from the original reactants or over-nitration during synthesis. The most likely impurity is unreacted guanidine nitrate.  [7]

Error in this experiment can be due to measurement error and loss of product during transfer from one vessel to another, as well as allowing the temperature to get too high.

# Conclusions

The product's synthesis of the Nitroguanidine and Methyl-3-Nitrobenzoate, but with a much lower yield than expected this is most likely due to products lost in solvents and incomplete reaction times. If the product was manufactured with too many impurities, then the output could be a lot more susceptible than expected making it much more dangerous to handle and a change in performance of the overall system it is used in.

# References (IEEE)

[1] “Electrophilic substitution reactions can be used to introduce halogens or nitro (NO2) groups onto a benzene ring.,” Senecalearning.com, 2021. https://senecalearning.com/en-GB/revision-notes/ib-diploma/chemistry/chemistry-ib-diploma-higher-level/6-5-1-electrophilic-substitution (accessed Dec. 12, 2025).

[2] “electrophilic substitution - the nitration of benzene,” Chemguide.co.uk, 2025. https://chemguide.co.uk/mechanisms/elsub/nitration.html

[3] Echemi, “Nitration of Methyl Benzoate | Process and Applications,” ECHEMI, Oct. 24, 2023. https://www.echemi.com/cms/1434927.html

[4]“An Electrophilic Aromatic Substitution: The nitration of methyl benzoate.” Available: https://www.westfield.ma.edu/PersonalPages/cmasi/organic_lab/labs/nitration_methylbenzoate.pdf

[5]Anasazi Instruments, “Electrophilic Aromatic Substitution Nitration of Methyl Benzoate Anasazi Experiment Series.” Available: https://www.aiinmr.com/wp-content/uploads/2020/01/Electrophilic-Aromatic-Substitution-Nitration-of-Methyl-Benzoate.pdf

[6]“Nitration of Methyl Benzoate Experiment,” UKEssays.com, May 18, 2020. https://www.ukessays.com/essays/chemistry/nitration-of-methyl-benzoate-experiment.php

[7]R. R. SANGHAVI, A. G. S. PILLAI, S. P. VELAPURE, and A. SINGH, “Studies on Different Types of Nitrocellulose in Triple Base Gun Propellant Formulations,” Journal of Energetic Materials, vol. 21, no. 2, pp. 87–95, Apr. 2003, doi: https://doi.org/10.1080/713845496.

[8]“TECHNICAL LIBRARY.” Accessed: Dec. 12, 2025. [Online]. Available: https://apps.dtic.mil/sti/tr/pdf/ADA016888.pdf

[9]K. Schofield, Aromatic Nitration. CUP Archive, 1980.

‌

‌