| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 5 | LEFEL 5 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| JAN 2026 | JAN 2026 |

MODULE CODE: ACDB5002

Electronics I

Marking Scheme

SENARIO

A smart energy-saving device is to be installed in rural homes where street lighting is minimal. The goal is to design a compact system that can automatically detect low light conditions and trigger an emergency light (LED), while also displaying status messages on an LCD.

The system will use a PIC16F877A microcontroller and should operate using standard electronic components. It must be developed and tested using Proteus simulation software and programmed using CCS C.

SYSTEM SPECIFACTIONS

- A 12V DC source must be stepped down and regulated to 5V using a voltage regulator.

- An LDR-based light sensor and an LM393 comparator must be used to detect ambient light:

  - Output LOW (0V) in darkness.

  - Output HIGH (5V) in brightness.

  - Output connected to RB0 of PIC16F877A.

- An LED connected to RD0 must:

  - Turn ON in dark conditions.

  - Turn OFF in bright conditions.

- A 16x2 LCD must display:

  - "DARK CONDITION" when it is dark.

  - "BRIGHT CONDITION" when it is bright.

Answer the Following

- Design the following circuits using correct component values and connectivity.

  - Power Supply Circuit.							[10 Marks]

SOLUTION

    - Correct use of 7805 voltage regulator (input, output, ground pins properly connected)								[2 Marks]

    - Input capacitor (typically 0.1µF or 10µF) connected between input and ground									[1 Mark]

    - Output capacitor (typically 0.1µF or 10µF) connected between output and ground								[1 Mark]

    - Input voltage source correctly set to 12V DC			[1 Mark]

    - Regulated 5V output successfully provided			[2 Marks]

    - Correct grounding for all components				[1 Mark]

    - Circuit layout clarity and completeness (no floating pins, labelled components)

[2 Marks]

  - LDR-based Light Detection Circuit using LM393 comparator 		[10 Marks]

SOLUTION

- LDR and fixed resistor correctly arranged as a voltage divider

[2 Marks]

- Pr Reference voltage (via potentiometer or fixed resistor) correctly connected.

[1 Mark]

- to IN−oper connection of voltage divider to comparator input (IN+)

[1 Mark]

- Comparator output connected with a pull-up resistor		[1 Mark]

- Correct Vcc and GND connections to the LM393			[1 Mark]

- Output logic:

  - LOW when dark,						[1 Mark]

  - HIGH when bright – correctly implemented		[1 Mark]

- Clarity and completeness of circuit diagram (labelled, no open pins, clean layout)								[2 Marks]

- Simulate and validate the above (i) said designed circuits:

  - Power Supply Circuit simulation.						[5 Marks]

SOLUTION

[5 Marks]

  - LDR-based Light Detection Circuit simulation.				[5 Marks]

SOLUTION

[2.5 Marks]

[2.5 Marks]

- Develop a single CCS C program to:

  - Read digital input from RB0.

  - Control LED output on RD0.

  - Display "DARK CONDITION" or "BRIGHT CONDITION" on the LCD based on input.											[20 Marks]

SOLUTIONS

Including Adequate comments							[2 Marks]

Variable Naming and Declaration: Considering the Ports from the question and declaring them.										[ 2 Marks]

Code Optimization								[2 Marks]

Code Structure and Readability: Marks for logical Flow and easy understandability

[2 Marks]

Correctness and Functionality.						 	[2 Marks]

- Simulate the complete system and demonstrate functionality by submitting clear screen shots showing the correct outputs for the responses on LED and LCD under dark and bright conditions.							[20 Marks]

- Correct simulation setup in Proteus (includes power supply, PIC, LDR, comparator, LED, LCD)									[4 Marks]

- Functional output for dark condition:
- LED turns ON								[2 Marks]
- LCD displays "DARK CONDITION"					[2 Marks]

- Functional output for bright condition:

- LED turns OFF								[2 Marks]

- LCD displays "BRIGHT CONDITION"					[2 Marks]

- Clear and labelled screenshots showing both states (dark & bright)	[4 Marks]

- Accuracy and presentation of the overall system (clear layout, correct logic, no floating connections, realistic behaviour)					[4 Marks]

- Answer the following theoretically

  - Explain the working of the Night Light Sensor circuit.			[10 Marks]

SOLUTION

- The purpose of Night light sensor is to make the LED ON when there is no light/ during darkness.

- Light sensors can be used to measure illuminance, respond to changes in the amount of light received, or convert light to electricity.				[2 Marks]

- The LM393 IC is a low-power, single-supply, low-offset voltage, double, differential comparators.

•	 It is used to calculate the voltages at two dissimilar terminals and contrasts the dissimilarity in voltage quantity.

•	If the voltage of the first terminal has a high voltage than the second terminal, the switch will activate.

•	If the first terminal has a low voltage than the second terminal, the switch will deactivate.									[4 Marks]

•	This circuit uses a photoresistor for controlling the circuit of a voltage divider.  When this circuit absorbs to bright light, then the output device will be turned OFF.

•	When the circuit absorbs the darkness then the output device will be turned ON.

•	If the inverting terminal (𝑉1  ) of the IC voltage is higher than the non-inverting terminal V2 then the output device activates. 𝑉1>𝑉2→LED ON.

•	Similarly, if the inverting terminal 𝑉1 of the IC voltage is lower than the non-inverting terminal 𝑉2 , then the output device deactivates. 𝑉1<𝑉2→LED OFF. 	[4 Marks]

  - Explain the working of the Voltage Regulator circuit with a neat diagram.												[10 Marks]

SOLUITION

- The voltage regulator has an unregulated dc input voltage Vi applied to one input terminal, a regulated output dc voltage Vo from a second terminal, and the third terminal connected to ground.						[2 Marks]

[3 Marks]

Fixed-Positive Voltage Regulator

- The series 78XX regulators are the three-terminal devices that provide a fixed positive output voltage.

- An unregulated input voltage Vi is filtered by a capacitor C1 and connected to the IC’s IN terminal.

- The IC’s OUT terminal provides a regulated +12 V, which is filtered by capacitor C2. The third IC terminal is connected to ground (GND)		[3 Marks]

Fixed-Negative Voltage Regulator

- The series 79XX regulators are the three-terminal IC regulators that provide a fixed negative output voltage.					[2 Marks]

- Explain the functions of RS, RW, E, D0–D7, VSS, VDD and VEE in a 16x2 LCD module.												[10 Marks]

SOLUTION

- RS (Register Select):

  - Selects between instruction register (RS = 0) and data register (RS = 1).

  - Controls whether the input sent is a command or display data.

[2 Marks]

- RW (Read/Write):

  - Controls the direction of data.

  - RW = 0 for write operations (from MCU to LCD),

  - RW = 1 for read operations (from LCD to MCU).

[2 Marks]

- E (Enable):

  - Acts as a latch.

  - On a high-to-low pulse, the LCD reads data from the data lines.

  - Must be toggled each time data/command is sent.

[2 Marks]

  - D0–D7 data pins – State their role in sending data or instructions; mention 4-bit/8-bit modes.							[2 Marks]

- VSS and VDD pins – Identify ground and positive supply terminals.

[1 Mark]

- VEE pin – Mention its use for adjusting display contrast.

[1 Mark]

Diwedd/End