# Preparation of a Bill of Quantities

Contents

Introduction	3

Assumptions Made	3

Cost	3

Conclusion	3

# Introduction

Within this report I will be informing the client to how ive prepared his bill of quantities highlighting all important aspects from the bill of quantities and assumptions I have made to come to the prices and quantities. I will ensure I use the Spons architectural 2020 book to ensure the clients prices are up to date. I will ensure that I have used the new rules of measurement 1 book to correctly set out my bill of quantities.

# Assumptions Made

To complete to carry out this project I had to provide a bill of quantities for the internal renovation at the innovation matrix. As preparing some areas were straight forward, on other occasions I had to make some assumptions which are estimates to provide a cost. I had to make an assumption on the ceiling height which I used 2.4m in order to provide an estimate for the wall areas that were to be painted. Due to having no finishing schedule I had to create assumptions for paint colours which I opted for Dulux vinyl matt as well as this I have measured the square meterage for the abov. The flooring I made an assumption from the 3mm self-level latex screen and had to provide a cost based off an assumption for the carpet, vinyl and entrance matting, which I opted for the 500x500 Forbo Flooring, 2mm Altro Aquarius and 18mm Gradius Topguard in two sizes 1200x750 and 2400x1200mm. The client had allowed for the doors. However, I had to meet the building standards so I made an assumption to allow for an 926x2040 FD30 white oak veneered door with hardwood lipping to all edges with premdor fire shield or similar approved. Includes 2no 150x700mm fire rated vision panels. As well as the fire doors I had to allow for 2no R100 Framed intellegant automatic 4 wing revolving door 2400Dx2600H.

# Cost

The total renovation of the innovation matrix came to £351,604.28 this has allowed for 10% waste on all materials. See below breakdown of the cost:

Floor Finishes- £158,845.50 which ive allowed for carpet, underlay, vinyl, wooden and coved skirting.

Wall Finishes - £10,990.34 which includes paint to walls and skirting. However, I have measured the wall cladding buy that is to be carried out by others.

Doors - £58,464.72 this allows for 46no 926x2040 FD30 white oak veneered door with hardwood lipping to all edges with premdor fire shield or similar approved. Includes 2no 150x700mm fire rated vision panels. It has also allowed for 2no R100 Framed intellegant automatic 4 wing revolving door 2400Dx2600H

Ceilings – £91,999.69. Within this section I have allowed for some ceilings to be painted and some to have a suspended ceiling system with perimeter trim.

# Conclusion