EXERCISE 1

You need to design a flexible road pavement for a motorway with 3 lanes in every direction. The information given is as follows:

Daily flow (count data according to COBA):

- Buses and coaches. 	32 cv/d

- 2 axle rigid. 		467 cv/d

- 3 axle rigid.		67 cv/d

- 3 and 4 axle articulated	274 cv/d

- 4 axle rigid.		49 cv/d

- 5 axle articulated.	938 cv/d

- 6 or more axle.		530 cv/d

Foundation class 4;

Construction of a SBM B1 base with gravel coarse aggregate.

Define the following:

- Commercial Vehicle Flow at opening and percentages corresponding to OGV2 and OGV1+PSV.

- General design period considered.

- Growth factor for OGV2 and OGV1+PSV.

- Wear factor for every type of commercial vehicle.

- Percentage of commercial vehicles in the heaviest loaded lane.

- Calculate the total future cumulative flow.

- Calculate the section of the road base and asphalt thickness.

|  | F | Y | G | W | P | msa |
| --- | --- | --- | --- | --- | --- | --- |
| Buses and coaches |  |  |  |  |  |  |
| 2 axle rigid |  |  |  |  |  |  |
| 3 axle rigid |  |  |  |  |  |  |
| 3 and 4 axle articulated |  |  |  |  |  |  |
| 4 axle rigid |  |  |  |  |  |  |
| 5 axle articulated |  |  |  |  |  |  |
| 6 or more axle |  |  |  |  |  |  |
| TOTAL |  |  |  |  |  |  |