| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 6 | LEFEL 6 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| Mock 2024 | MOCK 2024 |

MODULE CODE: ACDA6018

Advanced Digital Signal Processing System

TIME:  2 HOURS

ANSWER ALL THE QUESTIONS

Each Question Carries 25 Marks

Equipment or materials required – Scientific Calculator

Adnoddau neu ddefnyddiau sydd eu hangen -

QUESTION 1

- Find the Fourier Transform of f(t) = sin(ωct+θ)					[15 Marks]

Note: Your answer may be selected from one of the provided options. Choosing the correct answer will earn you 1 Mark. However, to receive full marks, you must demonstrate your reasoning for the selected option. Below are the options:

- Fjω= = 2Aπ[cosω0t-14cos3ω0t+19cos5ω0t-…]

- Fjω= 2. sincω+4. sinc(2ω)

- A certain function of the time f(t) has the Fourier Transform Fjω=  1ω2+1e-2ω2(ω2+1)

Using the properties of the Fourier Transform, write the Fourier transforms of

- f2t											[5 Marks]

- 4ddtft										[5 Marks]

QUESTION 2

- Compute the Discrete Fourier Transform (DFT) for the below sequence to identify its frequency components

xn= cosnπ4						[10 Marks]

- Design an ideal LPF with a frequency response of

[15 Marks]

QUESTION 3

- An SDR receiver processes the RF signal 𝑆(𝑡) with an amplitude of 4, a frequency of 2200𝐻𝑧, and a phase of  𝜋/4. The local oscillator generates a signal with an amplitude of 2 and a frequency of 2150𝐻𝑧. This LO signal is used to down convert the RF signal into baseband components. The receiver generates a complex signal 𝑍(𝑡). The system also encounters a narrowband interference at 2230𝐻𝑧, which must be removed using appropriate filtering techniques.

Based on the SDR Receiver, answer the following,

- Using the given values for s(t) and the LO, specify the expressions for I(t), Q(t) and Z(t).

[12 Marks]

- Draw the block diagram of the SDR receiver, labelling all components including the LO signal with its amplitude and frequency.					[4 Marks]

- Briefly explain how the system converts the RF input into its baseband representation.

[2 Marks]

- On Considering the impact of interference on the system, answer the following,

- An interference signal with a frequency of 2230Hz is mixed with a local oscillator operating at 2150Hz and derive the resulting frequencies from the mixing process. Explain which frequency should be retained in the baseband after applying a low-pass filter.									[2 Marks]

- Discuss how the addition of interference affects the output frequency compared to the output frequency without interference. 					[2 Marks]

- What measures are recommended to recover the desired frequency effectively?

[3 Marks]

QUESTION 4

- For the discrete signal yn={0, 1, 2, 3, 4, 5, 6, 7}, Compute the Discrete Fourier Transform Y(K) using the Decimation-in-Time Fast Fourier Transform (DIT FFT) algorithm. Show the intermediate steps including

- Dividing the sequence into 4 even and 4 odd terms. 			[2 Marks]

- Computing the Twiddle Factors						[2 Marks]

- Using the Decimation-in-Time Fast Fourier Transform (DIT FFT) with a butterfly diagram that clearly shows the stages of computation.					[21 Marks]

Diwedd/End