Formula Sheet

ACDA6019 – Advanced Radio Frequency systems

| Frequency f=Cλ | Transition Distance DTD=2D2λ | Fractional Bandwidth F.B %=fupper-flowerfcentre×100 |
| --- | --- | --- |
| Centre Frequency fcentre=fupper+flower2 | Absolute Bandwidth BWAbsolute=fUL-fLL | Aperture Efficiency ηap (%)=AEffAPhysical×100 |
| Effective radiating area AEff=λ2G4π | Physical Aperture APhysical=πD22 | Antenna Efficiency ηe(%)=PradPinput×100 |
| Velocity of Light c=1εμ | Directivity D=4πΩA Ddb=10log10(D) D=UU0=4πUPrad | Loss less network: Pincident =Ptransmitted+Preflected |
| Characteristic impedance: Z0=LC | Phase constanct: β=ωLC Where , ω=2πf β =2πλ | Return loss Return loss =-20log10S11 Return loss=-20log10Γ |
| Normalised load impedance ZL=ZLZ0 | Input impedance of the line Zin=ZinZ0 | Input admittance of the line Yin=YinY0 |
| Voltage reflection Coefficient Γ=s-1s+1 Γ=ZL-Z0ZL+Z0 Γ=Emax-EminEmax+Emin | Gain G=ηD Gdb=10log10(G) Gain =20log10S21 Gain=PoutPin Gain=S212 | Standing Wave Ratio SWR=EmaxEmin=ImaxImin S=R0RL; RL<R0 S=RLR0 ; RL>R0 S=1+Γ1-Γ |
| Length of Antnna L=λ2 L=λ4​ L=λ4 | Pitch angle α=tan-1(SπD) | Log Periodic element Length Ln=τ×Ln-1 N=log⁡(fmaxfmin)log(1τ) |
| Gain of an Array Garray=Gelement +10log10(N) | Length of an Array Larray=(N-1)×spacing | Natural Phase shift ϕ=kd |
| Wave Number k=2πλ | Insertion Loss Insertion loss =-20log10S21 | Isolation Isolation=-20log10S12 |
| Phase Constant β=ωLC β =2πλ | Lossless Notwork S112+S212+…+Sn12=1 S122+S222+…+Sn22=1 | Propagation Constant γ=α+jβ |
| Radiation Intensity U=r2Wrad | Quarter wavelength Transformer N1N22= ZinZL | Power Radiated Prad=SWrad. ds Prad=4πr2Wrad Prad=ΩU. dΩ Prad=02π0πU.Sinθ dθ dϕ Prad=ΩU0. dΩ Prad=U0ΩdΩ Prad=4πU0 |
| Width of Patch W=c2frεr+12 | Effective Length of Patch: Leff =c2frεreff |  |
| Effective Relative permittivity εreff=εr+12+εr-1 2 (11+12hw) |  |  |
| Length Extension of Patch ∆L=0.412h(εreff+0.3)(wh+0.264)(εreff-0.258)(wh+0.8) |  |  |
|  |  | Length of the Patch L=Leff-2∆L |

Trigonometric Formula

Sin3θ=3sinθ-4sin2θ

Integration Formula

dx=x+c

adx=ax+c

xndx=xn+1n+1+c

sinxdx=-cosx+c

cosxdx=sinx+c

exdx=ex+c

sin (n x)dx=-cos (nx)n+c

cos (n x)dx=sin⁡(nx)n+c

udv=uv-vdu

Differentiation formula

d xn=nxn-1dx

d(cosθ)dθ=-sinθ

d(Sinθ)dθ=cosθ