| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 7 | LEFEL 7 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| April 2016 | Ebrill 2016 |

MODULE CODE: M7X01375

TIME:  2 HOURS

answer Three quESTION out of four

Equipment or materials required – Calculators may be used and Moving Average SPC paper to be provided and Capability paper

Adnoddau neu ddefnyddiau sydd eu hangen - Dim

ANSWER three QUESTIONS

Question 1

- Evaluate why the use of Short Run SPC charts is a good idea in modern batch manufacturing.

(5 marks)

- Drakkes Manufacturing Ltd supply chiller units which are filled with an anti-freeze chemical.  The concentration of the antifreeze is checked daily and the following results have been obtained over the next 20 days.

| Day | Result (ppm) | Day | Result (ppm) |
| --- | --- | --- | --- |
| 1 | 36.8 | 11 | 33.0 |
| 2 | 38.0 | 12 | 37.2 |
| 3 | 31.7 | 13 | 38.4 |
| 4 | 32.0 | 14 | 34.0 |
| 5 | 35.3 | 15 | 32.7 |
| 6 | 31.4 | 16 | 30.1 |
| 7 | 33.5 | 17 | 38.3 |
| 8 | 31.1 | 18 | 36.1 |
| 9 | 37.8 | 19 | 30.2 |
| 10 | 31.9 | 20 | 39.2 |

Using a sample size of n = 3 construct a moving average moving range chart and comment on your results.

(15 marks)

- Explain where Cusum charts can be used and the reasons for their use.

(5 marks)

Question 2

FMEA is widely seen as a very valuable risk assessment technique that provides quantitative as well as qualitative analysis.

Explain in detail how you would undertake an FMEA exercise.

(25 marks)

Question 3

Quality function deployment (QFD) is a way to capture, organise and deploy the voice of the customer both internally and externally.

- Critically analyse the role that the Kano Model plays in improving quality.

(10 marks)

- Evaluate the relationship that QFD has with Total Quality Management (TQM)

(7 marks)

- The following data in Figure 1 has been obtained from a QFD study.  Calculate the how much values and explain the importance of these values in QFD analysis

(8 marks)

Figure 1	QFD Study Aircraft Seats

|  | Importance | Width of armrest | Armrest recess depth | Profile of back | Back thickness | Liquid resistant fabric | Stain resistant fabric | Washable fabric | Fabric wear resistant | Material FST performance | Mountings interchangeable |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Arm rest folds away | 5 |  |  |  |  |  |  |  |  |  |  |
| Arm rest wide enough | 5 |  |  |  |  |  |  |  |  |  |  |
| Enough leg room | 8 |  |  |  |  |  |  |  |  |  |  |
| Lumbar support | 8 |  |  |  |  |  |  |  |  |  |  |
| Doesn’t soak up split drink | 5 |  |  |  |  |  |  |  |  |  |  |
| Fabric stain resistant | 6 |  |  |  |  |  |  |  |  |  |  |
| Easy to clean | 6 |  |  |  |  |  |  |  |  |  |  |
| Durable | 6 |  |  |  |  |  |  |  |  |  |  |
| Lightweight | 9 |  |  |  |  |  |  |  |  |  |  |
| Fit more passengers in | 5 |  |  |  |  |  |  |  |  |  |  |

Question 4

Machine capability studies help to provide a picture of the performance of the process as a whole.

- Explain why machine capability studies use 8s when Cm and Cp values use 6s.

(5 marks)

- Using the data below perform a machine capability study using a tolerance of 24.69mm ±0.05mm and comment on your findings.

(15 marks)

| Sample | Reading | Sample | Reading | Sample | Reading | Sample | Reading | Sample | Reading |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 24.70 | 11 | 24.74 | 21 | 24.68 | 31 | 24.72 | 41 | 24.72 |
| 2 | 24.66 | 12 | 24.64 | 22 | 24.74 | 32 | 24.70 | 42 | 24.70 |
| 3 | 24.72 | 13 | 24.78 | 23 | 24.72 | 33 | 24.74 | 43 | 24.78 |
| 4 | 24.74 | 14 | 24.68 | 24 | 24.70 | 34 | 24.74 | 44 | 24.68 |
| 5 | 24.70 | 15 | 24.72 | 25 | 24.76 | 35 | 24.66 | 45 | 24.76 |
| 6 | 24.78 | 16 | 24.76 | 26 | 24.68 | 36 | 24.76 | 46 | 24.70 |
| 7 | 24.68 | 17 | 24.70 | 27 | 24.74 | 37 | 24.72 | 47 | 24.74 |
| 8 | 24.68 | 18 | 24.66 | 28 | 24.66 | 38 | 24.70 | 48 | 24.66 |
| 9 | 24.72 | 19 | 24.72 | 29 | 24.74 | 39 | 24.68 | 49 | 24.72 |
| 10 | 24.64 | 20 | 24.68 | 30 | 24.72 | 40 | 24.65 | 50 | 24.70 |

- Evaluate what steps should be carried out prior to performing a machine capability study.

(5 marks)