Figure 1 - Elements of a communication system [1]

# Simulation of Digital Modulation Techniques

OME Digital Signal Processing and Communications

### Abstract

This report investigates the performance of  various DSP modulation techniques under varying levels of Additive White Gaussian Noise.

Simulink models are included.

Recommendations for future study are included.

Alexander Dand | OME Professional Apprenticeship | Dec 2020

Contents

Abstract	0

Introduction	2

Task Description	2

Background Information	2

Conversion of an Analogue signal to Digital – Carrier Waves	4

Fourier Transfom	5

Sampling	5

Methods of DSP	7

Amplitude Shift Keying (ASK)	7

Frequency Shift Keying (FSK)	8

Phase Shift Keying (PSK)	9

Advanced methods – Quadrature Phase Shift Keying (QPSK)	9

Quadrature Amplitude Modulation (QAM)	10

Simulation Models	13

Results	17

Discussion	19

Conclusion	25

References	26

Table of Figures	28

Appendix	30

Tables	30

# Introduction

## Task Description

The author was tasked with critically comparing the performances of various methods of Digital Signal Processing (DSP) under a range of noise conditions, with an emphasis on amplitude shift keying (ASK) and phase shift keying (PSK) using additive white Gaussian noise (AWGN) in varying levels.

The systems are to be modelled in Simulink and analysed. It has also been required to investigate more advanced models such as Quadrature Phase Shift Keying (QPSK) and Quadrature Amplitude Modulation. Each system shall consist of a modulator and a demodulator (modem) and data shall be obtained via a scope connected at various points to the system.

The analysis criteria will be met by collating data from each simulation and calculating the bit error rate (BER).

# Background Information

Digital signal processing is the method of converting an analogue stream of data (represented by a waveform) into digital form (represented by a series of discrete values, High and Low, or 1 and 0) by the use of an analogue-to-digital converter (ADC), transmitting the digital data to a receiving source, and converting the digital data back into an analogue form using a digital-to-analogue converter (DAC) [2].

Figure 2 - Basic Elements of a Digital Communication System [3]

Historically, long distance communications were sent via wire, but these techniques are vulnerable to disruption and data loss through distortion caused by imperfections in the communication medium, interference with said medium, and sometimes the sheer distance travelled using a prohibitively large amount of power to make the signal be ‘heard’ at the other end.

To this end, and with increasing requirement for secure and accurate transmission of data, it was discovered that data can be compressed and encoded in a much more efficient way. This allows for the time taken to exchange such data to be dramatically decreased, in turn reducing the chance of loss, and increasing the accuracy of the received signal [4]. As the signal is broken down to only two possible states (0 or 1, off or on), the potential for misinterpretation is reduced as the data is recognised as ‘close enough’ to be interpreted as the correct state.

Figure 3 - Interpretation of digital signals. (a) Transmitted signal. (b) Received distorted signal (without noise). (c) Received distorted signal (with noise). (d) Regenerated signal (delayed). [5]

Other advantages of communicating digitally include lower cost, ease of design, reduced chance of decryption by an outside source (privacy), and reduced chance of interference (by the use of spread spectrum technique [6]).

### Conversion of an Analogue signal to Digital – Carrier Waves

In order to transmit data digitally, a transmission media is generated in the form of a carrier wave. This is created by an electronic oscillator and takes the form of a sinusoid wave (plotted voltage against time), generally of a much higher frequency than the original data. This is then modulated by the original wave to form an impression on the carrier and transmitted.

This wave can be expressed with the equation [7]:

xt=Acos2πfat+θ        -∞<t<+∞

xt=Acos⁡(ωa+θ)

Where:

xt=x with respect to time

A=amplitude

fa=frequency (Hertz)

t=time (seconds)

θ=phase (radians)

ωa=angular frequency (radians per second)

### Fourier Transfom

The still analogue carrier wave is expressed as a function of time (t) and is constantly changing. It can then be translated into a function of frequency (f) by a process know as the Fourier Transform. This is a mathematical process that converts a function into its constituent parts [8]. In this case, the process can be expressed as:

g(t)⇔G(f)

Gf=Fg(t)=-∞∞g(t)e-j2πftdt

So the resulting spectrum of the sinusoid can be seen as shown in Figure 4 where the amplitude of the original signal gives the height of the transformed peak, and the frequency translates to the offset from zero.

Figure 4 - Spectrum of two sine waves before and after the Fourier Transform [9]

This has the effect of changing a continuous wave into a series of one-dimensional spectra, with amplitude as it’s only variable.

### Sampling

As this new format of spectra does not move in the time domain, in order to encode the variable signal, it is necessary to take a series of ‘snapshots’ of the waveform, in a process known as sampling.

Figure 5 - The sampling process. (a) Analogue signal. (b) Instantaneously sampled version of the analogue signal. [1]

Samples are taken at a uniform time period (Ts) to recreate the original data signal. If Ts is smaller and thus of a higher frequency, the signal is more accurate but requires more processing power to process the higher number of samples. If Ts is larger (lower frequency) the waveform is replicated less accurately, and more errors are likely to occur at demodulation.

As an example:

Figure 6 - Sampling a low frequency signal (arrows indicate sample instants). [2]

Figure 7 - Sampling a high frequency signal (same sample instants as Figure 6). [2]

If the sample rate stays the same, the output signal will be shown as the dashed line, which is significantly different to the actual signal. As a result, a convention is used for efficient sampling known as the Nyquist-Shannon sampling theorem. This states that the sampling frequency must be at least twice that of the initial waveform [10].

Sampling is key to DSP and allows for the production of a binary code representing the voltage in the analogue domain. [11]

### Methods of DSP

As a wave has three characteristics (Amplitude, Frequency and Phase), the manipulation of an individual characteristic (or ‘modulation’) allows for translation into discrete states, or ‘bits’.

### Amplitude Shift Keying (ASK)

Modulation of the amplitude of a signal into discrete signals (0 or 1) means that the amplitude of the signal shall be represented as Figure 8, where a ‘1’ is represented by the carrier signal being transmitted as normal, and a ‘0’ is represented by a stop in the transmission. Due to this, this method is also known as ‘ON-OFF’ keying.

Figure 8 - ASK modulation waveforms [12]

ASK applications are limited, as this method of modulation does not allow for changes in amplitude.

### Frequency Shift Keying (FSK)

With FSK, two separate carrier signals are transmitted for modulation of the data signal. Each has its own individual frequency, and the discrete levels are transmitted by switching between the two frequencies. The waveform for this is shown as:

Figure 9 - FSK modulation waveforms [13]

This method is a much more versatile method of modulation than ASK, and as such is used more widely, however it requires more bandwidth and has a limited data rate, so it is really only used in low-speed modems [14].

### Phase Shift Keying (PSK)

The third characteristic that can be modulated is phase. Like FSK, two carrier waves are transmitted, but they are out of phase with each other, and the state switch is identified as the transition between phases. The waveform for this can be seen as:

Figure 10 - PSK modulation waveforms [15]

PSK allows for more data to be transmitted more efficiently compared with FSK, although it is not as bandwidth efficient as ASK. This method is also more secure and less vulnerable to interference than the previous two methods [16]. This is also known as Binary Phase Shift Keying (BPSK) as it is still a 1-bit transmission.

### Advanced methods – Quadrature Phase Shift Keying (QPSK)

PSK opens up the possibility of sending more than one additional carrier wave form out of phase, which allows for a higher number of discrete states. QPSK modulates two bits simultaneously, with 4 different phase offsets at 90̊ between each. This doubles the rate at which information can be transmitted on a single channel, and the waveform is shown as:

Figure 11 - QPSK modulated waveforms [17]

The advantages of this method are the increased data transmission rate, half the bandwidth of BPSK, and is less vulnerable to noise or errors. However, building the circuit itself is much more of a complex task than other methods. [17]

### Quadrature Amplitude Modulation (QAM)

QAM, similarly to BPSK/QPSK, allows for multiple transmissions to utilise the same channel by modulating data to different amplitudes.

Figure 12 - Input carrier waves for QAM [18]

Figure 13 - QAM output waveform [18]

With this method, a very high data rate can be achieved as multiple levels of QAM can be transmitted along the same channel, increasing the number of bits that can be transmitted simultaneously.

The bit positioning of QAM can be shown on a constellation diagram.

Figure 14 - 16QAM Constellation Diagram [1]

# Simulation Models

Figure 15 - ASK Modem

Figure 16 - PSK Modem

Figure 17 - QPSK Modem

Figure 18 - QAM Modem

# Results

Figure 19 - ASK model results (See Appendix for detailed results)

Figure 20 - BPSK model results (See Appendix for detailed results)

Figure 21 - Overlay of ASK and BPSK results

# Discussion

During the performance of these experiments, the author found considerable difficulties. The experiments for ASK and BPSK ran to an acceptable level, with results indicated in the section above, and generally running in line with established research:

Figure 22 - BER vs SNR of different modulation schemes [19]

Figure 23 - BER Performance of different modulation schemes with SNR [20]

Figure 24 - BER vs SNR curve [21]

Errors were discovered in each run as shown below:

Figure 25 - errors indicated in Port 7 scope view – ASK @ 0dB

However due to the sampling rate, very few of these were picked up at low noise, and as previously discussed, ASK has a greater chance of correctly calling the data point as here it is still quite easy to say ‘close enough’. For a more accurate view of error rate, the sample time should be increased, however as discussed earlier, this would increase the processing time required.

Figure 26 - Noise seen as SNR increasing - ASK @ -10dB)

As shown in Figure 26, noise is being seen quite rapidly increasing and even after the filter (Port 6), the signal is becoming unrecognisable. Should this experiment be run again, a smaller, more defined range of noise levels should be studied to more closely understand the increase in BER as the noise increases.

The author found that any increased AWGN above 100dB resulted in no real change, implying that the threshold of BER is around 50%. Each level of AWGN was run ten times for each model, and the time of transmission was increased from 10 seconds to 100 seconds for a larger sample size. The data input rate was 1 bit/sec, which is a glacially slow bit rate (minimum acceptable BER for telecommunications is 10-9 and to obtain a figure of this order would take a billion seconds, or nearly 32 years per run). If this experiment was to be run again, it would be advisable to increase the number of bits sampled for a more eclectic range of samples.

During the simulation involving the QPSK modem, no useable data was discovered, as there was a total spread of approximately 2% between the lowest and highest results. The following warning was displayed during each run:

Figure 27 - Error message obtained during QPSK simulation

This indicates an issue with the data entering the Error Rate calculator block, and is supported by the scope results:

Figure 28 - typical scope view - QPSK simulation

As can be seen, Port 2 is returning a constant 0V signal, hence the constant ~0.75 BER. As there is a 25% chance for the data to turn up a 0 read, this seems correct, given the results. Whilst the author has identified where the issue lies, unfamiliarity with the software prevents finding a fix. It was considered that the filter design my be at fault, but as a 29th order filter, this consideration was discarded.

Figure 29 - Filter Design – QPSK simulation

Should this experiment be run again, more training and orientation with MATLAB would be essential.

Similar issues were found with the QAM simulation, as all results were coming back at about a 0.9 BER. The author successfully managed adjust certain settings within the blocks to remove warnings (also the Random Integer generator was not generating random integers, but the same stream of data each time), but was still unable to obtain meaningful results as seen below:

Figure 30 - typical scope view - QAM simulation

There seems to be a discrepancy in the received signal (Port 2) as there doesn’t seem to be an even spread of received data (no data between levels 4 and 8).

# Conclusion

In conclusion, the author was disappointed in the lack of results, but several improvements have been identified for future experiments. The main issue would be the lack of knowledge of the software, and the inability to successfully troubleshoot the problem blocks. Also a more focussed approach to the SNR and an increased but more refined approach to sampling would improve the data output.

# References

| [1] | S. Haykin, Digital Communication Systems, New Jersey: Wiley, 2014. |
| --- | --- |
| [2] | M. Parker, Digital Signal Processing - Everything you need to know to get started, Massachusetts: Elsevier, 2010. |
| [3] | Tutorials Point, “Digital Communications - Quick Guide,” Tutorials Point, [Online]. Available: https://www.tutorialspoint.com/digital_communication/digital_communication_quick_guide.htm. [Accessed 15 December 2020]. |
| [4] | D. Rachesti, T. Purboyo and A. Prasanti, “Comparison of Text Data Compression Using Huffman, Shannon-Fano, Run,” International Journal of Applied Engineering Research, vol. 12, no. 23, pp. 13618-13622, 2017. |
| [5] | B. Lathi, Modern Digital and Analog Communication Systems - 3rd Edition, Sacramento: Oxford University Press, 1998. |
| [6] | R. Pickholtz, D. Schilling and L. Milstein, “Theory of Spread-Spectrum Communications-A Tutorial,” IEEE TRANSACTIONS ON COMMUNICATIONS, Vols. COM-30, no. 5, pp. 855-884, 1982. |
| [7] | S. Mosey, Communication Principles, Swansea, 2020. |
| [8] | D. Kammler, A First Course in Fourier Analysis, Illinois: Cambridge University Press, 2007. |
| [9] | B. Duggan, “Processing: the fourier transform,” 05 11 2015. [Online]. Available: http://sopnmr.blogspot.com/2015/11/processing-fourier-transform.html. [Accessed 18 12 2020]. |
| [10] | E. Davies, Computer & Machine Vision - Theory, Algorithms, Practicalities - 4th Edition, London: Elsevier, 2012. |
| [11] | Electronics Tutorials, “Analogue to Digital Converter,” Electronics Tutorials, [Online]. Available: https://www.electronics-tutorials.ws/combination/analogue-to-digital-converter.html. [Accessed 18 12 2020]. |
| [12] | ElProCus, “Amplitude Shift Keying (ASK) Working and Applications,” ElProCus, [Online]. Available: https://www.elprocus.com/amplitude-shift-keying-ask-working-and-applications/. [Accessed 18 12 2020]. |
| [13] | Tutorialspoint, “Frequency Shift Keying,” Tutorialspoint, [Online]. Available: https://www.tutorialspoint.com/digital_communication/digital_communication_frequency_shift_keying.htm. [Accessed 18 12 2020]. |
| [14] | ElProCus, “Frequency Shift Keying (FSK) Working & Applications,” ElProCus, [Online]. Available: https://www.elprocus.com/frequency-shift-keying-fsk-working-applications/. [Accessed 18 12 2020]. |
| [15] | ElProCus, “Phase Shift Keying (PSK) : Types and Its Applications,” ElProCus, [Online]. Available: https://www.elprocus.com/phase-shift-keying-psk-types-and-its-applications/. [Accessed 18 12 2020]. |
| [16] | ElProCus, “Phase Shift Keying (PSK) : Types and Its Applications,” ElProCus, [Online]. Available: https://www.elprocus.com/phase-shift-keying-psk-types-and-its-applications/. [Accessed 18 12 2020]. |
| [17] | ElProCus, “Quadrature Phase Shift Keying : Waveform and Its Advantages,” ElProCus, [Online]. Available: https://www.elprocus.com/quadrature-phase-shift-keying-definition-with-circuit-diagram/. [Accessed 18 12 2020]. |
| [18] | ElProCus, “Quadrature Amplitude Modulation : Working Principle and Its Applications,” ElProCus, [Online]. Available: https://www.elprocus.com/quadrature-amplitude-modulation/. [Accessed 18 12 2020]. |
| [19] | A. Koranga and K. Barman, “Performance Enhancement of WiMAX using Adaptive Modulation Scheme,” International Journal of Scientific & Engineering Research, vol. 4, no. 4, pp. 1794-1797, 2013. |
| [20] | D. Anguita, D. Brizzolara and G. Parodi, “VHDL Modules and Circuits for Underwater Optical Wireless Communication Systems,” WSEAS TRANSACTIONS on COMMUNICATIONS, vol. 9, no. 9, pp. 525-552, 09 2010. |
| [21] | S. Bharati, M. A. Rahman and P. Podder, “Implementation of ASK, FSK and PSK with BER vs. SNR comparison over AWGN channel,” Bangladesh, 2020. |

# Table of Figures

Figure 1 - Elements of a communication system [1]	0

Figure 2 - Basic Elements of a Digital Communication System [3]	3

Figure 3 - Interpretation of digital signals. (a) Transmitted signal. (b) Received distorted signal (without noise). (c) Received distorted signal (with noise). (d) Regenerated signal (delayed). [5]	4

Figure 4 - Spectrum of two sine waves before and after the Fourier Transform [9]	5

Figure 5 - The sampling process. (a) Analogue signal. (b) Instantaneously sampled version of the analogue signal. [1]	6

Figure 6 - Sampling a low frequency signal (arrows indicate sample instants). [2]	6

Figure 7 - Sampling a high frequency signal (same sample instants as Figure 6). [2]	7

Figure 8 - ASK modulation waveforms [12]	8

Figure 9 - FSK modulation waveforms [13]	8

Figure 10 - PSK modulation waveforms [15]	9

Figure 11 - QPSK modulated waveforms [17]	10

Figure 12 - Input carrier waves for QAM [18]	11

Figure 13 - QAM output waveform [18]	11

Figure 14 - 16QAM Constellation Diagram [1]	12

Figure 15 - ASK Modem	13

Figure 16 - PSK Modem	14

Figure 17 - QPSK Modem	15

Figure 18 - QAM Modem	16

Figure 19 - ASK model results (See Appendix for detailed results)	17

Figure 20 - BPSK model results (See Appendix for detailed results)	18

Figure 21 - Overlay of ASK and BPSK results	19

Figure 22 - BER vs SNR of different modulation schemes [19]	20

Figure 23 - BER Performance of different modulation schemes with SNR [20]	20

Figure 24 - BER vs SNR curve [21]	21

Figure 25 - errors indicated in Port 7 scope view – ASK @ 0dB	22

Figure 26 - Noise seen as SNR increasing - ASK @ -10dB)	22

Figure 27 - Error message obtained during QPSK simulation	23

Figure 28 - typical scope view - QPSK simulation	24

Figure 29 - Filter Design – QPSK simulation	24

Figure 30 - typical scope view - QAM simulation	25

Figure 31 - Detailed ASK results	30

Figure 32 - Detailed BPSK results	31

Figure 33 - QPSK results	32

Figure 34 - QAM results	33

# Appendix

### Tables

Figure 31 - Detailed ASK results

Figure 32 - Detailed BPSK results

Figure 33 - QPSK results

Figure 34 - QAM results