Marking Key

ANSWER ALL THE QUESTIONS

Question 1 [25 Marks]

- For the circuit shown in fig1, Calculate the following

- Total resistance in the circuit			[6 Marks]

- Total current in the circuit.			[2 Marks]

- Current across each resistor		[10 Marks]

- The voltage across R7 resistor			[2 Marks]

- Power delivered by the battery			[2 Marks]

Fig: 1

Answers:

- Total Resistance in the circuit

R1 and R2 are in parallel

Req1=11R1+1R2=116+13=10.5=2Ω						[2 Marks]

R4 and R5 are in parallel

Req2=11R4+1R5=115+110=10.3=3.33Ω						[1 Mark]

Req2 and R3 are in series

Req3=Req2+R3=4+3.33=7.33Ω 					[1 Mark]

R6+R7 in parallel with Req3

Req4=(R6+R7)×Req3R6+R7+Req3=2+4×7.332+4+7.33=3.3Ω					[1 Mark]

Req4 and Req1 are in series

Rt=Req4+Req1=3.3+2=5.3Ω					[1 Mark]

- Total current in the circuit

I=VRt=255.3									[1 Mark]

I=4.71A									[1 Mark]

- Current across each resistor

Current across R1

I1=IR2R1+R2= 4.7×36+3=1.57A							[1 Mark]

Current across R2

I2=IR1R1+R2= 4.7×66+3=3.14A							[1 Mark]

Current across R3

I3=I(R6+R7)R6+R7+Req3= 4.7×(2+4)2+4+7.33=2.1A						[2 Marks]

Current across R4

I4=I3R5R4+R5= 2.1×105+10=1.4A 							[2 Marks]

Current across R5

I5=I3R4R4+R5= 2.1×55+10=0.7A 							[2 Marks]

Current across R6 and R7

I6=IReq3R6+R7+Req3= 4.7×7.332+4+7.33=2.55A						[2 Marks]

- Calculate the Voltage across R7 Resistor

V7=I6R7=2.44×4=10.2V 						[2 Marks]

- Power delivered by the battery

P=VI=25×4.71=117.75W			[2 Marks]

- A solar yard light has a solar collector that can provide 1.0W of power for charging the 3.0V batteries. What is the maximum charging current the solar collector could supply to the fully discharged 3.0V batteries?							[3 Marks]

Answers:

I=PV=13 									[2 Marks]

I=0.33A 									[1 Mark]

Question 2 [25 Marks]

- Calculate the current flowing when a charge of 60C flows past a point over a period of 2 minutes.										[2 Marks]

Answers:

I=Qt 											[1 Mark]

I=60120=0.5A									[1 Mark]

- What length of German silver wire, diameter 0.050 cm, is needed to make a 28 resistor, if the resistivity of German silver is 2.2×10-7  m?				[5 Marks]

Answers:

A=π r2 										[1 Mark]

r=2.50×10-4m

A=π×( 2.50×10-4)2=1.96×10-7m2					[1 Mark]

L=RAρ											[1 Mark]

L=28×1.96×10-72.2×10-7=24.95m     							[2 Marks]

- Determine the current across each resistor in the circuit shown in fig.2, using mesh analysis (Kirchhoff’s Voltage Law)	[14 Marks]

Fig.2

Answers:

Apply KVL in ABEFA

-5I1-3I1-I2-5-2I1+20=0 						[2 Marks]

-10I1+3I2=-15 …………(1)						[2 Marks]

Apply KVL in BCDEB

-6I2-7I2-2I2+5+3I1-I2=0 						[2 Marks]

-3I1-18I2=-5 …………(2)							[2 Marks]

Simplifying Equation (1) and (2)

I1=1.67A									[2 Marks]

Substitute, I1 in equation (1)

-10 1.67+3I2=-15

-16.7+3I2=-15

3I2=-15+16.7

I2=1.73=0.567A									[2 Marks]

Current across branch BE

I1-I2=1.67-0.567=1.1A							[2 Marks]

Current Across R1and R3=I1=1.67A

Current Across R2=I1-I2=1.1A

Current Across R4 R5 and R6=I2=0.567A

- A silicon diode in a circuit has a forward voltage drop of 0.7 V and is in series with a 1 kΩ resistor and a 5 V supply. Calculate the current through the diode.		[4 Marks]

Answers:

VR=VS-VD 									[1 Mark]

VR=5-0.7=4.3V 								[1 Mark]

I=VRR=4.31000=4.3mA 								[2 Marks]

Question 3 [25 Marks]

- Differentiate Step- up Transformer and Step-Down Transformer (minimum of two differences)									[4 Marks]

Answers:

| Sl. No | Step-up Transformer | Step-down Transformer |
| --- | --- | --- |
| 1 | Input voltage is lower than the output voltage | Input voltage is higher than the output voltage |
| 2 | Number of Secondary winding is higher than the primary winding | Number of Secondary winding is less than the primary winding |
| 3 |  |  |

[Each difference two marks, Maximum of 4 marks]

- Explain the following terms:

- Resistance									[2 Marks]

- Reactance									[2 Marks]

- Impedance									[2 Marks]

- Peak inverse voltage								[2 Marks]

Answers:

- Resistance

Opposition to flow of current. Unit is Ohm - DC circuit property		[2 Marks]

- Reactance

Opposition to change of flow of current. Unit is Ohm - AC circuit property	[2 Marks]

- Impedance

Opposition to change of flow of current. Unit is Ohm- AC circuit property 	[2 Marks]

- Peak inverse voltage

The maximum reverse voltage a diode can withstand without breaking down. Exceeding the PIV can cause permanent damage to the diode.					[2 Marks]

- The alternating current is given by i=40 sin314t A. Determine the following

- Peak Value of current			[1 Mark]

- RMS Value of Current			[2 Marks]

- Frequency			[2 Marks]

- Time period			[1 Mark]

Answers:

- Peak Value of current

From the given equation, Im=40A						[1 Mark]

- RMS Value of Current

IRMS=Im2=402									[1 Mark]

IRMS=28.28A									[1 Mark]

- Frequency

From the equation, ω=314

We know that, ω=2πf

314=2πf 									[1 Mark]

f=3142π=50Hz									[1 Mark]

- Time

T=1f=150=0.02sec								[1 Mark]

- An oscilloscope display shows the below voltage waveform. The oscilloscope settings are shown in the box next to the waveform.

Oscilloscope settings:

2V/div

10ms/div

Fig:3

- Estimate the period of the waveform.					[1 mark]

- From the estimated period, determine its frequency.			[2 marks]

- Determine its peak-peak amplitude					[2 marks]

- Determine the r.m.s. value of the waveform.				[2 marks]

Answers:.

- Estimate the period of the waveform.

Period=4 div ×10 ms=40 ms				[1 mark]

- From the estimated period, determine its frequency.

f=1t

f= 140 ms=25 Hz						[2 marks]

- Determine its peak-peak amplitude

Vpk-pk=4 div ×2 V=8 V				[2 marks]

- Determine the r.m.s. value of the waveform.

VRMS=Vpk2=42=2.828 V					[2 marks]

Question 4 [25 Marks]

- An RLC circuit is shown in figure 4 below. Calculate the following

- The total circuit impedance 						[4 Marks]

- Total current flowing in the circuit.					[2 Marks]

- Determine the voltage drop across each component in the circuits.	[6 Marks]

Fig: 4

Answers:

- Total impedance

XL=2πfL=2×π×50×1 mH=0.3142 Ω					[1 Mark] XC=12πfC=12×π×50×470 μF=6.773 Ω  						[1 Mark]

Z=R2+XL-XC2=1002+0.3142-6.7732=100.208 Ω		[2 Marks]

- Circuit Current

I=VZ=120100.208=1.198 A								[2 Marks]

- Determine the voltage drop across each component in the circuits.

VR=I×R=1.198×100=119.8 V					[2 Marks]

VL=I×XL=1.198×0.3142=0.376 V				[2 Marks]

VC=I×XC=1.198×6.773=8.114 V				[2 Marks]

- Describe the construction and working of PN Junction diode with the aid of necessary diagrams.									[10 Marks]

Answers:

A junction formed between a sample of P type semiconductor and N type semiconductor is known as PN junction (or) Junction diode.						[1 mark]

It has two electrodes

- P region named as Anode

- N region Named as Cathode							[1 mark]

[1 mark]

When no supply is given to the  PN junction diode.

[1 mark]

- At the junction, electrons in N-type semiconductor moves towards holes in P-type region and Holes in P-type region moves towards the electrons in the N- type region.

- Electrons from N region diffuse to P region, recombine with holes and form negative charge immobile ions, near the junction of P region.

- Similarly, Holes from P region diffuse to N region, recombine with electrons and from positive charge immobile ions, near the junction of N side.

- These immobile ions forms a barrier at the junction, which is known as Depletion region or Space charge region.

- After certain period of time, the negative charge ions in the P-region and the Positive charge ions in the N-region prevents further charges moving from one region to another.			[4 marks]

Voltage can be applied in PN junction diode in two ways to control the width of the depletion region. They are

- Forward biasing

- Reverse Biasing

When an external voltage is applied to the PN junction in such a way that its cancels the potential barrier and permits the current flow, it is called Biasing		[2 marks]

- Determine the voltage across each resistor in the circuit shown in fig.5. 	[3 Marks]

Fig.5

Answers:

As all the resistors are in parallel, the Voltage across the resistor is the same.

V1=V2=V3=V4=V5=10V							[3 Marks]